3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417723

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	7.15.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC and give our views.

2 Discussion

Regarding the applicability of CSSF optimization, it was discussed in last RAN4 meeting with no consensus reached. But in RAN#105 meeting, the issue was agreed and the WID was updated as below [3]:

- The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

Following the agreements above, when we discuss the CSSF optimization, the requirements are based on the assumption that UE is not operating with Multi-Rx based L3 measurement, i.e., we do not consider Multi-Rx and CSSF optimization together. But after defining the requirements, the CSSF optimization can apply to CA/DC scenario independently of the UE support of Multi-Rx capabilities.

Proposal 1: Follow RAN plenary conclusions about the applicability of CSSF optimization.

Regarding the scenarios/measurement procedure to apply CSSF outside gap optimization, the following agreement was achieved in RAN4#112 meeting:

Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

In our understanding, all the three aspects should be considered to apply CSSF optimization and no need to further discuss priority. Because we understand all the intra-frequency, inter-frequency and inter-RAT measurement requirements refer to the same section and use the same CSSF definition. Once the solutions are decided, the enhanced CSSF can be applied to all the three types of measurement. One concern in last meeting is that some of the solutions can only be used for certain type of measurement (e.g., intra-frequency), but we think it can be reflected in the corresponding measurement requirements rather than in the CSSF definition. For that part, we can discuss in detailed solutions. And once it impacts the CSSF definition or calculation, it will also apply to all the measurements.

Proposal 2: The enhanced CSSF apply to all the following aspects and do not define the priority.

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap.

Regarding the searcher assumption, it was agreed in last meeting to discuss the applicable scenarios and solutions based on 3 searchers and decide whether to define the requirements later.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

With the agreements above, we can focus on the solution discussion. In previous meeting, different solutions were raised to enhance CSSF outside gap, including adjusting searcher allocation, reducing unnecessary measurement and allowing additional searcher assumption and others. After the discussion in last meeting, the solutions are summarized as the following:

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

We are open to discuss all the solutions above and multiple solutions can be defined, but some further clarifications are needed and the details need to be decided for each option.

For option 1, companies are on the same page that UE can measure one serving CC per band if multiple serving CCs are in the same band. But the principle to choose the measured CC and the interpretation of not measured CC are needed to be clarified based on option 1A and option 1B.

In the existing intra-frequency measurement requirements, the similar solution is used for neighbour cell as below. PCC/PSCC shall be measured when it is configured in the band. When only SCCs are configured on the band, one of the SCCs shall be measured and selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - - 6 identified cells, and - - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - - PCC when UE is configured with SA NR operation mode with PCC in the band; or - - PSCC when UE is configured with EN-DC with PSCC in the band; or - - PSCC when UE is configured with NR-DC with PSCC in the band; or - - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

In our understanding, the same principle can be reused for serving cell. In last meeting, there are proposals to let network indicate the measured SCC and make the network indication mandatory, but we think there is no need to introduce the dedicate signalling to indicate the measured SCC. Because from network side, we may not know the UE status and have no full acknowledge which SCC should be measured. Meanwhile, there are many UEs in the deployment with different requests, and it is hard for network to indicate the target measured CC for all the UEs. Even if some companies have strong demand to introduce this indication, it should not be mandatory and the flexibility should be left to network. If no indication is provided from network, the selected SCC should be left to UE implementation.

Proposal 3: Reuse the legacy principle for neighbour cell measurement for solution 1 (UE can measure one serving CC per band if multiple serving CCs are in the same band).

For option 2 and 2A, the difference is whether to adjust the searcher occupation ratio for PCC/PSCC. We are initially fine to use PCC/PSCC searcher to speed up SCC measurements. But meanwhile we also realize that such solution would extend the PCC/PSCC measurement and may impact the communication in some of scenarios which was also raised by other companies in last meeting. We think there are two approaches to resolve the issue.

One approach is to keep option 2A but to add some conditions (e.g., the serving cell measurement quality is larger than a threshold) for this solution as we suggested initially. This approach will limit the applicable scenarios and the reasonable ratios are needed. If we are going to use this approach, it is better to define one or multiple fixed ratios in the specification rather than to dynamically indicate the ratio. And if multiple ratios are defined, the applied ratio can be indicated by network.

And the second approach is to update the solution from option 2A to option 2 which keep the searcher for PCC/PSCC but adjust the searcher occupation of different SCCs. This approach will guarantee the PCC/PSCC measurement and the communication quality but the delay reduction gain may be limited. If we are going to use this approach, we do not need to define the indication dynamically either. We think network can indicate the priority of the SCCs if needed and UE measure the SCCs based on the order of the priority so that the certain SCC can be prioritized. If the priority is not provided by network, the measurement priority is up to UE implementation which is same as the legacy. From our side, we prefer the second approach.

Proposal 4: UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g., NW can indicate the priority of the SCCs. If the priority is not provided, the measurement order is up to UE implementation which is same as the legacy.

For option 3 which is based on 3 searchers assumption, it depends on existing UE capability, and no additional UE implementation complexity is required. But according to the discussion so far, it may not apply to all the cases and some conditions (e.g., both FR1 and FR2 measurement objects are configured) are needed.

Proposal 5: For the solution based on 3 searchers assumption, define the applicable scenarios (e.g., all the measurements are outside gap and both FR1 and FR2 measurement objects are configured).

For the feature capability, based on the solution discussions above, we think it depends on the specified solutions, e.g., new capability is not needed for option 3 but is needed for other solutions. So we think it can be discussed at the end of core part when the solutions are decided.

Proposal 6: The feature capability depends on the solutions and can be discussed at the end of core part:

- For option 1 and option 2, a new UE capability is needed to indicate the UE support of new solution.

- For option 3, the existing per-FR gap capability can be reused.

3 Summary

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: Follow RAN plenary conclusions about the applicability of CSSF optimization.

Proposal 2: The enhanced CSSF apply to all the following aspects and do not define the priority.

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap.

Proposal 3: Reuse the legacy principle for neighbour cell measurement for solution 1 (UE can measure one serving CC per band if multiple serving CCs are in the same band).

Proposal 4: UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g., NW can indicate the priority of the SCCs. If the priority is not provided, the measurement order is up to UE implementation which is same as the legacy.

Proposal 5: For the solution based on 3 searchers assumption, define the applicable scenarios (e.g., all the measurements are outside gap and both FR1 and FR2 measurement objects are configured).

Proposal 6: The feature capability depends on the solutions and can be discussed at the end of core part:

- For option 1 and option 2, a new UE capability is needed to indicate the UE support of new solution.

- For option 3, the existing per-FR gap capability can be reused.

4 References

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#112bis

- R4-2415656, Topic summary for [112bis][209] NR_RRM_Ph5_Part1, Apple, RAN4#112bis

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105