3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417724

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Source:	CATT

Agenda Item:	7.15.3

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the fast SCell activation for UE supporting Rel-18 EMR was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on the fast SCell activation with EMR and give our views.

2 Discussion

2.1 General

Time line for fast SCell activation

Based on the WID [3] and the agreement in last meeting, RAN4 should aim to close the core part of this objective by November meeting.

Latest WID RP-242380 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024 Agreement in RAN4#112bis meeting: RAN4 aims to close core part of this objective by Nov meeting as per the RAN plenary guidance.

In last meeting, some companies argued that the completion date is the aim not a mandatory deadline, but we think this is not the case. Every work item has the target completion date which should be followed and respected and this is the way of working in 3GPP. If all the work items hold the mentality that the date is just a target and no need to strictly follow, the work in 3GPP would be extremely inefficient. So we think we should follow the RAN plenary guidance to complete this objective in November meeting and the potential extension cannot be the reason to extend the endless discussion on the controversial issues.

Considering this is the last meeting for fast SCell activation for UE supporting Rel-18 eEMR, RAN4 should not further extend the scope and should focus on the scenarios/cases with group consensus. For the additional scenarios which cannot reach consensus, we should not include them in the scope and can close the core part of this objective with main issues resolved and common scenarios defined, and not to extend the completion date for the controversial cases with no consensus.

Proposal 1: RAN4 to close the core part of this objective with the requirements for common scenarios defined, and not to extend the completion date due to the controversial cases with no consensus.

Applicability of fast SCell activation

In last meeting, the applicability of fast SCell activation delay requirements was discussed but no consensus was reached.

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

The issue is whether to define requirements for the case when UE supports Rel-18 eEMR measurements but the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. According to the definition in Rel-18, if the duration is not configured, UE is not required to perform the validity check and UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. In such case, it is not clear whether the UE will report the measurement results and even if it reports, it is not clear when the measurements are performed and whether the results can be used for SCell activation, i.e., the results cannot be guaranteed as valid.

But based on the objective description in the WID [4], the target is to reduce the SCell activation delay with valid EMR reporting. If the validity of measurement results cannot be guaranteed, the reduced delay requirements cannot apply. So we understand we do not need to consider the case when the durations for validity check are not configured.

But in last meeting, during the discussion on known condition updates, there were the proposals to define different requirements for different duration configurations, e.g., when the configured duration is smaller than a threshold, the legacy known SCell activation delay requirements can be reused, and when the configured duration is larger than the threshold, additional SSB samples are needed to perform fine time tracking and AGC. If we are going to follow this logic, maybe the latter requirements can also be used for the case when duration is not configured but UE has eEMR reports. So we suggest to firstly discuss the known condition updates based on the case when validity check duration is configured in issue 1-2-1a, and to check if the longer requirements can also be applied to the case when the duration is not configured. But if no consensus on the applicability of the requirements regarding the case when duration is not configured, it will not delay the completion of this objective.

Proposal 2: Define the requirements for the case when validity check duration is configured as baseline, and check whether the requirements can be applied to the case when the duration is not configured afterwards.

Proposal 3: The completion of this objective will not be delayed due to the applicability of the requirements regarding the case when validity check duration is not configured.

Target scenarios of fast SCell activation

In last meeting, there were some proposals about the target scenarios of fast SCell activation but were not discussed due to lack of time.

#### Issue 1-1-3: Target scenarios of fast SCell activation Proposals - Proposal 1: (CMCC, CTC) - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1. - Proposal 2: (Nokia, Ericsson) - RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR. - Proposal 3: (Qualcomm, Samsung) - RAN4 to not consider the following SCells for the eEMR based fast SCell activation requirements: - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA) - SCell being in the same band as PCell in FR2 (i.e. intra-band CA) - Proposal 4: (Nokia) - RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell + FR2 Pscell/SCell. - Proposal 5: (Nokia) - RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay). - Proposal 6: (Samsung) - RAN4 to consider the following scenarios as the starting point: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

The discussion for this objective is to define the requirements for a new case when valid EMR report is provided. What we care about is how to define a smaller delay for this new case, not to reduce the delay of existing known or unknown case. So for proposal 1 and 2, after the known conditions are updated based on the discussion in 1-2-1a, the requirements for known and unknown cases will be clear and no need to have the dedicated conclusion for these two proposals.

For proposal 3, 4 and 6 regarding whether to define requirements for different CA scenarios, as cited below, the conditions are already included in the existing requirements, and we think no updates are needed.

For FR1: - - However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - - the SCell is contiguous to an active serving cell in the same band, and - - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - - its SMTC offset is same as the one of contiguous FR1 active serving cell - - its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; For FR2： If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2: If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is: - - 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is: - - max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time. If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - ... If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - - 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - ... If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - …

For proposal 5, the intention seems to limit the RRC setup/resume delay, but we understand the SCell activation delay requirements we discussed are not only related to the RRC setup/resume delay but also depend on when UE performs the measurement and when network sends the SCell activation command. The RRC setup/resume delay is only a small part of the delay comparing to other components, and we do not see the necessity to limit the scenarios regarding RRC setup/resume.

Proposal 4: The requirements applicability regarding CA configurations in the existing SCell activation delay can be reused and no updates are needed.

Proposal 5: Do not limit the scenarios regarding RRC setup/resume delay.

2.2 RRM requirements

Regarding how to define the fast SCell activation delay requirements with valid EMR reporting, the following agreements were made in last meeting based on direct SCell activation.

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

The remaining part is the requirements applicability regarding the validity check duration. For FR1, all the companies are on the same page that we can use existing SSB based known SCell activation delay requirement with [2] SSB samples, i.e., the requirements when measurement period of the SCell is larger than 2400ms when the validity check duration is configured. But for FR2, there was no consensus whether to define the side condition regarding the duration configuration. To move forward and complete the objective on time, we suggest using the compromise solution, i.e., option 3 or 4. The difference is option 3 only define the minimum requirements with a longer delay while option 4 defines two sets of requirements based on the configured duration values. To simplify the requirements, we prefer to use the option 3 to define the minimum requirements. If any technical issues are identified, we can further consider option 4.

Proposal 6: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

  - For FR2:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

    - delay components: 2 SSB samples * beam sweeping +1 SSB sample

Besides the updates above, there are also the following proposals for SCell activation delay requirements with Rel-18 eEMR report:

- Proposal 7: - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity]. - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true.

We understand the intention is to emphasize that UE should at least receive one SSB sample every measurement period or SMTC periodicity and if UE received multiple SSB, the measurement report should be beam level report. But we think the first bullet is natural since we have already agreed the SSB detectability during the whole SCell activation procedure and the SSB measurement delay is already included in the requirements, so we understand no clarification is needed for the SSB receiving. For the second bullet, the existing requirements in FR2 are already based on SSB measurement with SSB index and no additional clarification is needed either. Meanwhile, we think this is not the eEMR specific issue which has already been included in the legacy requirements and no specific updates are needed.

Proposal 7: No additional clarifications on SSB sample receiving or beam level report are needed.

Regarding the issue about measurement period condition as below, in the existing requirement, two sets of requirements are defined for FR1 known SCell activation based on different measurement periods, in which one additional SMTC periodicity is required to perform AGC when the SCell measurement period is larger than 2400ms.

#### Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation Proposals - Option 1: (CATT) - No need to update the measurement period condition for FR1 known SCell activation. - Option 2a: (Apple) - For fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases. - Option 2b: (Qualcomm, Nokia) - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity]. - Option 3: (Samsung) - If the SCell is known based on the eEMR based known condition and belongs to FR1, 2400 ms can be modified to Y ms - The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

But based on the analysis for proposal 6 above, the requirements for fast SCell activation with Rel-18 eEMR are always based on two SSB samples i.e., the AGC is always needed. So the legacy measurement period condition is not needed anymore.

Proposal 8: The legacy measurement period condition for FR1 known SCell activation is not needed anymore since the requirements are always based on the longer one.

The last issue is the indication to network for fast SCell activation as below:

#### Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: (CATT, Apple, Samsung) - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation. - Option 2: (Ericsson, Nokia) - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

Companies mentioned that network may be not aware of this fast SCell activation enhancement and would not use the EMR reporting to activate the SCell, so a UE indication to network would be helpful. But we understand the valid EMR report according to Rel-18 validity check can already be the indication to network, and what we are trying to do in this work item is to define requirements for the case when UE has this indication. It means the requirements are based on Rel-18 indication and we do not need to introduce a new indication. After getting this report, it is up to network how to use it to configure or activate SCell.

The proponents of option 2 also mentioned that the indication can be per UE per cell and network can get the information from UE whether the fast SCell activation can be applied to a certain cell, but we understand this cannot work since UE doesn’t know whether the fast SCell activation requirements can be applied before it receives the activation command from network. Meanwhile, from network side, we do not need to know which requirements UE should apply. And even UE report this indication, network may not follow the information to configure or activate the SCell since there are so many UEs in the deployment and network needs to consider the system performance rather than a certain UE. Based on the analysis, we don’t think such indication is needed.

Proposal 9: No need to introduce additional indication to the network for fast or slow SCell activation.

The above discussions are based on direct SCell activation, and in last meeting, it was agreed that MAC CE based SCell activation including PUCCH SCell should also be considered.

#### Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported. - Note: MAC CE based PUCCH SCell activation is included.

Based on the existing requirements, we understand the known conditions for direct SCell activation and MAC CE based SCell activation are almost same. The only difference is that the proximity between measurement report and SCell activation command for FR1 is max(5*measCycleSCell,  5*DRX cycles) for MAC CE based SCell activation while it is 5s for direct SCell activation. But since for eEMR based requirements, the target is to use the measurement in IDLE/INACTIVE state and the SCell has not been configured, there is no measCycleSCell configuration. We understand the fixed value can also be used for MAC CE based SCell activation and we suggest using the same value as direct SCell activation, i.e., 5s.

The other issue regarding MAC CE based SCell activation is that there was one proposal in last meeting to introduce a new dedicated eEMR based fast SCell activation indication.

- Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

From NW perspective, we understand we only need to send the SCell activation command and the corresponding configurations, and whether to apply fast SCell activation is based on the comparison of the conditions and configurations from UE side. We do not see the necessity to introduce additional indication from NW side. Based on the discussions above, we think the conclusions of direct SCell activation can also be applied to MAC CE based SCell activation delay requirements.

Proposal 10: Do not introduce new dedicated eEMR based fast SCell activation indication for MAC CE based SCell activation.

Proposal 11: For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition.

According to all the discussion above, a CR was provided in [4] to reflect the proposed updates.

Proposal 12: RAN4 to define the requirements for fast SCell activation with Rel-18 eEMR report as in CR [4].

2.3 UE capability

After defining the requirements, we are considering to clarify the UE capability for this feature. Since this objective is to define the requirements for UE supporting Rel-18 eEMR report, the corresponding capability has already been defined in Rel-18. It means that for a Rel-18 UE supporting eEMR, it still follows the legacy SCell activation delay requirements, but for a Rel-19 UE supporting eEMR, it can follow the enhanced fast SCell activation delay requirements. Since network can differentiate the release of accessing UE according to the following signaling, we think no additional UE capability is needed for fast SCell activation with Rel-18 eEMR report.

-- ASN1START -- TAG-ACCESSSTRATUMRELEASE-START AccessStratumRelease ::= ENUMERATED { rel15, rel16, rel17, rel18, spare4, spare3, spare2, spare1, ... } -- TAG-ACCESSSTRATUMRELEASE-STOP -- ASN1STOP

But considering we still have the other objective and the whole WI completion date is next year, the UE capability issue can be discussed together at the end of the core part.

Proposal 13: No additional UE capability is needed for fast SCell activation with Rel-18 eEMR report. But the UE capability discussion can also be left to the end of the core part.

3 Summary

In this paper, we have some further discussions on fast SCell activation with Rel-18 EMR, and the following proposals are given:

Proposal 1: RAN4 to close the core part of this objective with the requirements for common scenarios defined, and not to extend the completion date due to the controversial cases with no consensus.

Proposal 2: Define the requirements for the case when validity check duration is configured as baseline, and check whether the requirements can be applied to the case when the duration is not configured afterwards.

Proposal 3: The completion of this objective will not be delayed due to the applicability of the requirements regarding the case when validity check duration is not configured.

Proposal 4: The requirements applicability regarding CA configurations in the existing SCell activation delay can be reused and no updates are needed.

Proposal 5: Do not limit the scenarios regarding RRC setup/resume delay.

Proposal 6: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

  - For FR2:

    - use existing SSB based known SCell activation delay requirement with 2 SSB samples.

    - delay components: 2 SSB samples * beam sweeping +1 SSB sample

Proposal 7: No additional clarifications on SSB sample receiving or beam level report are needed.

Proposal 8: The legacy measurement period condition for FR1 known SCell activation is not needed anymore since the requirements are always based on the longer one.

Proposal 9: No need to introduce additional indication to the network for fast or slow SCell activation.

Proposal 10: Do not introduce new dedicated eEMR based fast SCell activation indication for MAC CE based SCell activation.

Proposal 11: For MAC CE based SCell activation delay requirements with eEMR report, reuse the same conclusions as direct SCell activation at SCell addition.

Proposal 12: RAN4 to define the requirements for fast SCell activation with Rel-18 eEMR report as in CR [4].

Proposal 13: No additional UE capability is needed for fast SCell activation with Rel-18 eEMR report. But the UE capability discussion can also be left to the end of the core part.

4 References

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN4#112bis

- R4-2415657, Topic summary for [112bis][210] NR_RRM_Ph5_Part2, CATT, RAN4#112bis

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105

- R4-2417725, CR on Fast SCell activation for UE supporting Rel-18 eEMR, CATT, Apple, RAN4#113