3GPP RAN WG4 Meeting #113																	R4-2417744

Orlando, USA, 18th – 22nd November 2024

Agenda item:	6.11.2

Source:	Apple

Title:	On simultaneous Rx/Tx

WI/SI:	LTE_NR_R19_Simult_RxTx

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

A new work item [1] on simultaneous Rx/Tx has been agreed in RAN#104. One of its objectives is to clarify and improve existing mandatory simultaneous Rx/Tx requirements. This discussion paper discusses three main topics. The first topic is about the Rel-17 agreement on FR1+FR1 FDD-TDD combinations and the missing requirement in TS 38.101-1/3. The second topic is about the correct handling and request of fallbacks. The third topic is about CA_n40A-n41A and the requirements when supporting simultaneous Rx/Tx.

# 2  Discussion

#### 2.1 Background on issue of CA_n40A-n41A

A UE is required to feature a minimum of four Rx antenna ports for band n41. According to TS38.101-1 clause 7.2  this requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antennas and switches to reduce implementation complexity and number of required antennas.

In case of non-simultaneous Rx-Tx it is possible to share antennas between n40 and n41 as uplink and downlink transmission never occurs at the same time for both bands. However, in case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. Part of this issue comes from the required protection of the close-by 2.4GHz ISM band which used for WIFI. On antenna side it is challenging to optimize and achieve the required performance for all the required antennas over the entire frequency of n40 and n41. If an implementation is done for MIMO support with simultaneous Rx/Tx then separate antennas are needed for each Tx and Rx path to optimize the overall performance for the entire frequency range. Integration of six or more antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Dependent on the RF front-end implementation a UE might not be able to support simultaneous Rx/Tx for CA_n40A-n41A together with MIMO operation. In this case the UE might either indicate sole support of MIMO or indicate sole support of simultaneous Rx/Tx for CA_n40A-n41A but not both capabilities. For such devices it might be beneficial to introduce relaxation to enable simultaneous support of MIMO and simultaneous Rx/Tx.

#### 2.2 New test point

In RAN4#112bis [3] it was proposed to introduce a new test point as the Rel-18 test point features high MSD. It has been proposed to use a wider DL channel and to reduced UL channel so that the main interference source is >ACLR2. This approach shall reduce the MSD value which is specified in TS 38.101-1. However, the problem of CA_n40A-n41A and simultaneous Rx/Tx is the insufficient isolation between the two bands. A new test point only provides a different point of view without changing the fundamental issue that this band combination needs to deal with. If no improvements to the way of operation are considered, then creating a new test point with more favourable channel setup would be a simple re-packaging of the same issue. We would like to encourage to further discuss performance improvements which could be achieved by the 2Rx relaxation. More details on this proposal and on the question why it could improve DL MIMO reception (even under similar MSD level) can be found in section 2.3.

Observation 1: The problem of CA_n40A-n41A and simultaneous Rx/Tx is the insufficient isolation between the two bands. A new test point only provides a different point of view without changing the fundamental issue that this band combination needs to deal with. If no improvements to the architecture are considered, then creating a new test point with more favourable channel setup would be a simple re-packaging.

Proposal 1: Consider a new test point and the proposed 2Rx relaxation as a package with Proposal 3 to achieve meaningful performance gains.

Proposal 2: In case a new test point for CA_40A-n41A is introduced consider removing the legacy Rel-18 test point.

#### 2.3 Relaxation for CA_n40A-n41A with simultaneous Rx/Tx

Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx, the two Rx chains sharing antennas with Tx observe high de-sense. The diversity antennas perform better due to the antenna isolation. In last meeting it has been indicated that the MSD value might only change slightly when removing antenna sharing through the proposed 2Rx approach. This is no surprise as the interference level of Rx sharing the antenna with Tx are larger compared to the diversity antennas. With assuming maximum ratio combining, the diversity antennas dominate the MSD result due to their lower noise level. However, if we consider MIMO reception the noise from the shared antennas is affecting all the layers. This aspect shall be shortly explored in the following paragraph.

As a simplified example it is assumed that there is a device receiving two MIMO layers with a two-antenna setup. One of the antennas shares Rx with Tx while the other antenna represents the diversity antenna. The simplification is used to reduce the complexity of the equations so that the fundamental issue can be explained more efficiently. The basic result is translatable to a 4-layer MIMO setup. The linear equations for the received data on the two antennas y1 and y2 are provided below with h being the channel, x being the transmitted data for each layer and n being the noise/interference.

y1=h11*x1+h12* x2+ n1

y2=h21*x1+h22* x2+ n2

Solving the equations for each layer to obtain the estimated data x̂1 and x̂2 reveals that each MIMO layer is subject to the interference of both antennas. The received data from the antenna sharing Rx and Tx (i.e. n1 features very high noise/interference) creates considerable noise contribution on both layers:

x̂1=h12n2- h22n1- h12y2+ h22y1h11 h22- h12 h21

x̂2=h21n1-h11n2+ h11y2- h21y1h11 h22- h12 h21

The result shows that the high interference from Rx sharing antenna with Tx has a considerable effect on all MIMO layers. The result from the simplified example is also true for 4-layer reception and means that all four layers will be highly distorted by the two antennas sharing Rx and Tx. Removing antenna sharing through the proposed 2Rx relaxation will neglect the interference from the shared antennas and might allow to improve the overall throughput. Hence, the above analysis shows that there is considerable noise improvement inside the DL MIMO layer even if MSD would not significantly change 2Rx.

Observation 2: The noise/interference from the antennas sharing Rx and Tx is affecting all DL MIMO layers. Removing antenna sharing through the proposed 2Rx relaxation will neglect the high interference and reduce the noise level for the DL MIMO layers. Hence, the MIMO layer noise analysis shows that there is considerable noise improvement inside the DL MIMO layer even if MSD would not significantly change for 2Rx.

The core proposal for the 2-layer MIMO relaxation is that a handheld which signals support of 4-layer MIMO and simultaneous Rx/Tx shall only be expected to handle a maximum of 2-layer MIMO if both capabilities are configured concurrently. The focus on 2Rx antenna ports is required to avoid SRS antenna switching on the highly de-sensitised antenna ports which would share Tx and Rx. The advantage of this approach is that impact on downlink throughput is minimised as 2-layer MIMO might still be possible on band n41. This approach allows the UE to support 4-layer MIMO for non-simultaneous Rx/Tx while it is possible to indicate 2-layer MIMO support for the case of simultaneous Rx/Tx being scheduled. This is a considerable advantage over option 3 where band n41 would be reduced to single layer downlink operation in case of simultaneous Rx/Tx.

Table 1: MIMO operation for handhelds indicating 4-layer MIMO support on n41 for

CA_n40A-n41A with concurrent simultaneous Rx/Tx support

| TDD pattern with simultaneous Rx/Tx | MIMO operation |  |
| --- | --- | --- |
|  | n40 | n41 |
| No | 2-layer / 2Rx | 4-layer / 4Rx |
| Yes | 2-layer / 2Rx | 2-layer / 2Rx |

Observation 3: Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe considerable de-sense. The combined sensitivity degradation on n41 can be up to 28.1dB. Such a high degradation has the potential to disrupt reception of the MIMO layers. While the degradation of the antennas which are sharing Rx/Tx is high the degradation of the diversity antennas is expected to be lower as the impact from the Tx is reduced by the antenna isolation.

A handheld which signals support of 4-layer MIMO per band (n40 and n41) and simultaneous Rx/Tx for CA_n40A-n41A shall only be expected to handle a maximum of 2-layer MIMO per CC if both features are configured concurrently. The reduction to 2Rx antenna ports per CC is required to avoid SRS antenna switching on the highly de-sensitised antenna ports sharing Tx and Rx. This approach allows the handheld to support 4-layer MIMO per CC for network layouts where non-simultaneous Rx/Tx is a possible configuration and 2-layer MIMO per CC whenever the network layout necessitates TDD patterns with simultaneous Rx/Tx.

Proposal 3: For handheld UEs which signal support of 4-layer MIMO per band (n40 and n41) and simultaneous Rx/Tx for CA_n40A-n41A, requirements for concurrent operation of MIMO and simultaneous Rx/Tx shall be specified assuming an RF architecture with 2Rx antenna ports per CC.

For FWA devices RAN4 should discuss whether this restriction is valid for the associated RF architecture and whether it is possible to assume unrestricted simultaneous Rx/Tx operation or not.

Proposal 4: For FWA devices RAN4 should discuss whether Rx port restrictions with concurrent MIMO and simultaneous Rx/Tx operation are needed or not.

With the 6Rx requirements making progress in Rel-19, RAN4 should also discuss concurrent MIMO and simultaneous Rx/Tx operation for 6Rx devices in future meetings.

Proposal 5: RAN4 should discuss concurrent MIMO and simultaneous Rx/Tx operation for 6Rx devices in future meetings.

# 4	Conclusions

This contribution provides considerations on concurrent MIMO and simultaneous Rx/Tx operation. The following observations and proposals are made:

Observation 1: The problem of CA_n40A-n41A and simultaneous Rx/Tx is the insufficient isolation between the two bands. A new test point only provides a different point of view without changing the fundamental issue that this band combination needs to deal with. If no improvements to the architecture are considered, then creating a new test point with more favourable channel setup would be a simple re-packaging.

Observation 2: The noise/interference from the antennas sharing Rx and Tx is affecting all DL MIMO layers. Removing antenna sharing through the proposed 2Rx relaxation will neglect the high interference and reduce the noise level for the DL MIMO layers. Hence, the MIMO layer noise analysis shows that there is considerable noise improvement inside the DL MIMO layer even if MSD would not significantly change for 2Rx.

Proposal 1: Consider a new test point and the proposed 2Rx relaxation as a package with Proposal 3 to achieve meaningful performance gains.

Proposal 2: In case a new test point for CA_40A-n41A is introduced consider removing the legacy Rel-18 test point.

Observation 3: Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe considerable de-sense. The combined sensitivity degradation on n41 can be up to 28.1dB. Such a high degradation has the potential to disrupt reception of the MIMO layers. While the degradation of the antennas which are sharing Rx/Tx is high the degradation of the diversity antennas is expected to be lower as the impact from the Tx is reduced by the antenna isolation.

Proposal 3: For handheld UEs which signal support of 4-layer MIMO per band (n40 and n41) and simultaneous Rx/Tx for CA_n40A-n41A, requirements for concurrent operation of MIMO and simultaneous Rx/Tx shall be specified assuming an RF architecture with 2Rx antenna ports per CC.

Proposal 4: For FWA devices RAN4 should discuss whether Rx port restrictions with concurrent MIMO and simultaneous Rx/Tx operation are needed or not.

Proposal 5: RAN4 should discuss concurrent MIMO and simultaneous Rx/Tx operation for 6Rx devices in future meetings.

# 4	References

- RP-242088, “Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, 3GPP TSG RAN Meeting #105

- R4-2417058, “WF on simultaneous Rx-Tx for CA_n40A-n41A”, Hefei, China, October 14 – 18, 2024

- R4-2415359, Further discussion on simultaneous Rx-Tx of CA_n40-n41, Murata

- R4-2411157, “On simultaneous Rx/Tx”, Apple, Maastricht, Netherlands, 19th – 23rd August, 2024