3GPP RAN WG4 Meeting #113																	R4-2417745

Orlando, USA, 18th – 22nd November 2024

Agenda item:	6.20.2

Source:	Apple

Title:	n28: A-MPR and MSD for PC2 including wider CBW

WI/SI:	NR_n28_PC2_40MHz_CBW-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The objectives of the Rel-19 WI [2] are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28. It is encouraged to evolve the existing dual duplexer architecture assumption to meet the current industry demand. This contribution proposes PC2 requirements for NS_17 considering a full-band duplexer.

# 2  Discussion

##### 2.1 Out-of-Band rejection for UE-to-UE coexistence

The situation for UE-to-UE coexistence for a 40MHz channel with lower edge at 703MHz is depicted below. The out-of-band emission limit is applicable up to frequency 694MHz and not limited by FOOB since Note 15 is used. Note 15 causes the emission requirement to be applicable inside the FOOB range. For our evaluation we also considered the -50dBm/MHz requirement from n71. The entire setup is visualised below in Figure 1.

Figure 1: UE-to-UE coexistence used to evaluate the emission impact of a 40MHz channel

Simulations have been conducted for PC3 and a 40MHz channel placed right at the lower band edge. Three scenarios have been checked. Those scenarios are 1RB edge case at the lower channel edge, fully allocated channel and inner full RB allocation. For each scenarios the corresponding MPR was applied. Table 1 captures the emission level exceeding the coexistence limits. It can be observed that the largest exceedance is ca. 11dB for DFT-s-OFDM and Outer Full allocation.

Table 1: Emissions exceeding UE-to-UE coexistence limits for PC3

|  | 1 RB Edge (Lower) | Outer Full Allocation | Inner Full Allocation |
| --- | --- | --- | --- |
| DFT-s-OFDM | 5.9dB (1dB MPR) | 10.9dB (1dB MPR) | 1.1dB (0dB MPR) |
| CP-OFDM | 3.1dB (3dB MPR) | 10.2dB (3dB MPR) | 0dB (1.5dB MPR) |

Observation: According to simulations for PC3, a 40MHz channel would require at least 11dB filter suppression to meet the UE-to-UE coexistence requirements with applying MPR.

# 4	Conclusions

This contribution provides considerations on n28 full-band duplexer architecture. The following observation is made:

Observation: According to simulations for PC3, a 40MHz channel would require at least 11dB filter suppression to meet the UE-to-UE coexistence requirements with applying MPR.

# 4	References

- RP-241205, “Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28“, CMCC, CBN, 3GPP TSG-RAN Meeting #104

- R4-2400170, “On n28 with full band duplexer”, Apple, 3GPP RAN WG4 Meeting #110

- R4-2414274, “WF on introduction of PC2 and 40MHz CBW in NR band n28”, CMCC, CBN, China Broadnet, 3GPP TSG-RAN WG4 Meeting # 112

- R4-2309060, “On PC2 A-MPR Proposal for NS_18”, Apple, 3GPP TSG-RAN4 Meeting #107

- R4-2304519, “PC2 A-MPR for band n28 NS_18“, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #106bis-e