3GPP RAN WG4 Meeting #113																	R4-2417747

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.8.2.2.1

Source:	Apple

Title:	Further considerations on the high-power UE Tx requirements

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN.

During the previous meetings RAN WG4 discussed which power classes and UE architectures should be considered further for the normative work. Thus, in this discussion paper we present our further views on NTN power classes and associated power domain enhancements.

# 2	High-power Tx requirements

## 2.1	Power classes and UE architectures

During the RAN4#112bis meeting companies discussed which power classes should be considered further for the NTN core specifications and under which UE RF architecture assumptions. It is worth noting that the existing NTN power class 3 is based on 1TX architecture that is common for both terrestrial and satellite bands FDD bands. Similarly, the existing power class 2, which is also based on 1TX architecture, can be easily extended from the existing terrestrial FDD bands to the satellite FDD bands. The PC1 power class is also based on 1TX architecture and is defined for the terrestrial FDD bands, from which it can be extended to the satellite operation.

Observation 1a:	Both PC2 and PC1 are based on the 1TX architecture and are defined for the existing terrestrial FDD bands, which makes them ideal candidates for efficient operation in the satellite FDD bands.

There was also a discussion on whether and how to enable the PC1.5 power class, for which several major concerns were raised. Firstly, this power class is defined only for the terrestrial TDD bands and at the moment is not applicable to the FDD bands. Secondly, since this power class assumes 2TX architecture, the resulting performance will be quite sub-optimal comparing to the 29dBm transmission with 1TX if it is enabled for the FDD satellite links.

Observation 1b:	 The existing PC1.5 power class is defined only for the TDD terrestrial bands and is based on the 2TX architecture, making it sub-optimal solution for the FDD satellite links.

Nevertheless, since there is an interest from the industry to enable 29dBm transmission power for the NTN devices, we suggest studying it further in the follow-up work once RAN WG4 completes the work on PC2 and PC1 classes.

Proposal 1a:	In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices and 31dBm 1TX (PC1) for non-handheld devices.

Proposal 1b:	29dBm for the FDD bands and handheld devices can be tackled in the follow-up normative work.

## 2.2	SEM and ACLR

During the previous RAN4#112bis meeting several companies expressed the view that some of the existing parameters, such as ACLR or SEM, can be relaxed to increase the maximum output power. Generally speaking, it is indeed true that unless even more stringent regulatory requirements are used, the ACLR and SEM are the major limiting factors. And since the ACLR is the requirement defined by 3GPP, it looks quite appealing to boost the UE power for NTN by relaxing the corresponding ACLR values. However, it is worth noting that ACLR becomes the limiting factor only for large UL allocations. As can be seen from the Figure 2.2-1 below, the ACLR usually limits the maximum output power only when a UE is scheduled with a large UL allocation.  On the contrary to it, small UL allocations are limited by SEM, not by the ACLR. And since a UE will be allocated small UL grants to maintain link budget of the satellite link, potential relaxations of ACLR will not provide any gains in the maximum output power. Of course, 3GPP can contemplate further relaxations in the spectral emission mask. On the other hand, SEM is defined by regulations, not by 3GPP, and thus it remains somewhat questionable on whether regulations will allow this type of relaxation.

Figure 2.2-1: Illustration of limiting factors with ACLR 30dBc.

Observation 2a:	ACLR becomes the limiting factor only for large allocations, whereas small UL allocations are limited by SEM.

Observation 2b:	Relaxing ACLR will not boost the maximum output power for the satellite links because a UE will be assigned small UL allocations.

Observation 2c:	It is not clear whether relaxations of the existing SEM is possible because it is defined by regulations, not by 3GPP.

As summary, it is important to ensure that an NTN UE uses the highest possible maximum output power. And to achieve that 3GPP can consider further which relaxations of the existing requirements can indeed benefit the satellite links without violating the regulatory requirements.  Meanwhile, existing 3GPP ACLR and SEM requirements can be adopted for the high-power NTN UEs as a baseline assumption.

Proposal 2a:	As a baseline approach, existing SEM and ACLR requirements are re-used for the high-power NTN UE.

Proposal 2b:	RAN WG4 can study which requirements, SEM or ACLR, can be relaxed for the best power gains for NTN.

# 3	Conclusions

In this discussion paper we have presented our further views on several open issues discussed during the previous RAN WG4 meetings. More specifically, we suggest focusing on the PC2 and PC1 power classes for NTN because these power classes are based on the 1TX architecture and are already defined for the FDD bands making them ideal candidates for the further NTN enhancements. Even higher power classes for the handheld devices, e.g. 29dBm, can be tackled in the follow-up normative work.  As for SEM and ACLR, our preference is to take existing requirements as a baseline, at the same time allowing RAN WG4 to study further which relaxations can result in the best gains for the maximum output power.

Observation 1a:	Both PC2 and PC1 are based on the 1TX architecture and are defined for the existing terrestrial FDD bands, which makes them ideal candidates for efficient operation in the satellite FDD bands

Observation 1b:	 The existing PC1.5 power class is defined only for the TDD terrestrial bands and is based on the 2TX architecture, making it sub-optimal solution for the FDD satellite links.

Observation 2a:	ACLR becomes the limiting factor only for large allocations, whereas small UL allocations are limited by SEM.

Observation 2b:	Relaxing ACLR will not boost the maximum output power for the satellite links because a UE will be assigned small UL allocations.

Observation 2c:	It is not clear whether relaxations of the existing SEM is possible because it is defined by regulations, not by 3GPP.

Proposal 1a:	In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices and 31dBm 1TX (PC1) for non-handheld devices.

Proposal 1b:	29dBm for the FDD bands and handheld devices can be tackled in the follow-up normative work.

Proposal 2a:	As a baseline approach, existing SEM and ACLR requirements are re-used for the high-power NTN UE.

Proposal 2b:	RAN WG4 can study which requirements, SEM or ACLR, can be relaxed for the best power gains for NTN.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"