3GPP RAN WG4 Meeting #113																	R4-2417750

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.8.3.2

Source:	Apple, Globalstar

Title:	Power back-off values for the 3MHz channel in band n254

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. After the RAN#104 meeting a new WI was agreed [3], one of the aims of which is to enable the 3MHz channel for the NTN bands. Thus, in this discussion paper we present our preliminary A-MPR simulation results for the 3MHz channel in the NTN band n254 frequency range.

# 2	Power back-off simulations for NTN L-/S-band

## 2.1	Simulation assumptions and scenarios

Satellite L-/S-band (n254) was standardised during Rel-18, whereupon the smaller considered channel bandwidth was 5MHz with the corresponding results gathered and summarised in TR 38.791. Here we present power back-off results for the PC3 power class and the 3MHz channel with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI (NS_04 and NS_05) and FCC (NS_03)

While specifying the A-MPR values for the 5MHz channel, several A-MPR regions were identified as shown in Figure 2.1-1 below:

-	the left-most region has the highest power back-off (1612.5 <= Fc <= 1613.9)

-	the next region requires power back-off only for the large allocations (1613.9 <= Fc <= 1615.7)

-	the right-most region also requires some power back-off (1622.4 <= Fc <= 1624).

Since the largest power back-off is typically needed only for the edge channels, the initial 3MHz power back-off simulations will be conducted for the corresponding channels at the left- and right-edges the band. Furthermore, we will consider ETSI requirements because they are more stringent than FCC.

Figure 2.1-1: A-MPR regions for the 5MHz channel for NS_04 and NS_05.

## 2.2	Power back-off simulations

### 2.2.1	1610-1618MHz ETSI sub-range (NS_04N)

Figure 2.2.1-1 present power back-off results for a case when the 3MHz channel is located right at the lower edge of the band.  So, for both CP-OFDM and DFT-s-OFDM most allocations do not need any specific power back-off, except for the large allocations spanning all (or almost all) 15RBs within the 3MHz channel. For those cases an extra power back-off, exceeding MPR, could be needed.

Figure 2.2.1-1: Power back-off results for NS_04N (Fc=1611.5MHz).

And once we shift the 3MHz channel farther away from the lower edge of the band, all allocations become power back-off free or stay within the MPR limits. Figure 2.2.1-2 below presents the power back-off values when the 3MHz channel is shifted by 1.4MHz (i.e. same offset which was considered in the 5MHz studies).

Figure 2.2.1-2: Power back-off results for NS_04N (Fc=1612.9MHz).

### 2.2.2	1618-1626.5MHz ETSI sub-range (NS_05N)

Next, Figure 2.2.2-1 presents simulations results for 3MHz channel placed at the right-most edge of the band under the NS_05N flag. As can be seen from the figures, most of the allocations do not need any power back-off and the ones, which need it, are still within the existing MPR limits. The only exception is the right-most allocations within the 3MHz channel, which might need extra power back-off not covered by MPR.

Figure 2.2.2-1: Power back-off results for NS_05N (Fc=1625MHz).

And shifting the 3MHz channel away from the upper edge of the band reduces a lot required power back-off as can be seen from Figure 2.2.2-2 below (the offset of 1.6MHz from the upper edge is the same as was considered for the 5MHz studies).

Figure 2.2.2-1: Power back-off results for NS_05N (Fc=1623.4MHz).

### 2.2.3	Summary

Based on the presented power back-off values for the 3MHz channel one can see that we can in principle adopt the same frequency partition as for the 5MHz channel. At the same time, we can simplify it a bit because there will be no need to differentiate between high and low A-MPR parts, as it was the case with the 5MHz channel. As can be seen from the Table 2.2.3-1 below, we need only two A-MPR partitions, at the lower and the upper edge of the band, to meet the ETSI regulatory requirements defined by NS_04N and NS_05N. Same information is also presented in Figure 2.2.3-1. As for the FCC NS_03N, only one A-MPR partition is needed at the lower edge of the band.

Table 2.2.3-1: Summary of the frequency regions where A-MPR is needed

| Frequency region (MHz) | NS_03N (FCC) | NS_04N (ETSI) | NS_05N (ETSI) |
| --- | --- | --- | --- |
| 1611.5 <= Fc <= 1612.9 | X | X | X |
| 1623.4 <= Fc <= 1625 |  | X | X |

Figure 2.2.3-1: A-MPR regions for the 3MHz channel for NS_04 and NS_05

# 3	Conclusions

In this discussion paper we have presented our initial results for the power back-off values for the 3MHz channel operating in the NTN band n254 frequency range. As can be seen from the results, a smaller 3MHz channel needs smaller power back-off when compared to the 5MHz channel. In fact, in most cases no power back-off is required or it is within the MPR margins. There are only two regions – at the lower and the upper edge of the band – where extra A-MPR values in the order of 0.5..1dB might be needed.

Proposal:	The 3MHz channel needs A-MPR for the lower (NS_03N and NS_04N) and upper (NS_05N) edges of the NTN band n254

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"