3GPP RAN4 Meeting #113		R4-2417756

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.12.2.1

Source:	Apple

Title:	Discussion on XR devices TRP and TRS test time reduction

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN4 #112bis meeting, the following agreements have been achieved in WF [1].

The goal of this contribution is to discuss and propose the test time reduction methods for TRP and TRS measurements.

# 2	Discussion

As discussed during the RAN4 #112bis meeting, there are many proposals discussing how to reduce test time for XR testing. This contribution makes the following observations and proposals.

## 2.1 Test Time Reduction Method of TRP Measurements

According to TS 38.161, the measurement is performed with a constant sampling step, e.g., 15 degrees or 30 degrees, in both theta (θ) and phi (ϕ) axes for TRP measurement.

The Spiral scan method can significantly reduce the test time for TRP measurements compared to the standard constant step size approach. This procedure depends on continuous rotation of theta and phi positioner during the collection of data points. Both DUT and measurement antenna rotate continuously, and power measurement can be taken on the measurement antenna for both the theta and phi polarizations. The measurement value and the measurement location (theta, phi) shall be recorded at every measurement point.

The Spiral scan method does not require to have measurement point spacing exactly 15 degrees. However, equal spacing of the test points shall be achieved using following criteria:

- For each polarization, the maximum theta rotation between successive spiral turns at the same phi angle shall be 15 degrees.

- For each polarization, the maximum distance in phi between adjacent points shall be 15 degrees.

- The minimum number of data points shall be 350, oversampled relative to standard 266 measurement points.

Observation 1: Based on 15 degrees constant sampling step, there are 266 measurement points, our observation is test time for one channel takes around 10-15 minutes in anechoic chamber.

Observation 2: Based on 300 measurement points for Spiral scan, our observation is test time for one channel takes around 5-8 minutes in anechoic chamber.

Proposal 1: Use the spiral scan procedure for test time reduction for TRP measurements.

## 2.2 TRS Test Procedure for Test Time Reduction

As per agreement in RAN4#112bis meeting [1] , this test procedure is to allow the radiated Rx sensitivity three-dimensional scan as defined in TS 38.161 to be performed at low Tx power levels. This will extend the DUT battery life during testing, enabling more testing to be done without interrupting the DUT and test measurement setup.[2]

The measurement is performed with a constant sampling step of 30 degrees in both theta (θ) and phi (φ) axes for TRS measurement.

UE Transmit Power configuration for TRS test time reduction method

DUT configuration for TRS test time reduction method is similar as specified in TS 38.161 Clause A.2.3. DUT shall be configured with p-NR-FR1= 10dBm for low power TRS measurements. For single point offset full power TRS measurement, DUT shall be configured for PC3, p-NR-FR1= 23dBm and for PC2, p-NR-FR1= 26dBm.

Test Procedure for TRS test time reduction method

- Perform the TRS measurement process following the procedure specified in TS 38.161 Clause A.3.3.3. The TRS value is calculated using the equation and integration approaches outlined in TS 38.1.6.1 Clause A.3.5.2. This TRS is called TRS Tested with low power

- Determine the peak position and polarization for the maximum radiated sensitivity measurement from Step 1, the EIS obtained at the peak position is called EIS low Tx power.

- Configure the UE to transmit at full power to measure the EIS maximum Tx power at the same peak position determine from Step 2.

- Calculate the TRS for the DUT using the following equation:                                         	                                       TRS = TRS Tested with low power + EIS maximum Tx power - EIS low Tx power

These steps shall be repeated for each specified channels and bands.

Observation 2: TRS testing at low transmit power, enables more testing can be done without charging the DUT.

Proposal 2:  To approve the test time reduction for TRS test procedure.

# 3	Conclusions

This contribution provides our views on the topic of headworn XR TRP and TRS test time reduction methods. This contribution makes the following proposals.

Proposal 1:	Use the spiral scan procedure for TRP measurements.

Proposal 2:	To approve the test time reduction for TRS test procedure.

# 4	References

R4- 2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3” vivo, 3GPP RAN4 #112bis, 14th – 18th Oct 2024

R4-2415321, “Discussion on XR test method and Configurations”, Apple, 3GPP RAN4 #112bis, 14th – 18th Oct 2024