3GPP RAN4 Meeting #113		R4-2417760

Orlando, USA, November 18-22, 2024

Agenda item:	7.28.2

Source:	Apple

Title:	On IoT NTN TDD mode frequency error

Release:	Rel-19

Work Item:	IoT_NTN_TDD-Core

Document for:	Discussion

# 1	Introduction

During the RAN #105 meeting a new work item was approved to specify a TDD mode to accommodate IoT NTN deployments in unpaired spectrum (notably, the Iridium 1616-1626.5MHz band) [1].  The objectives of the WID are as follows:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

During the RAN1 #118bis meeting this work item was first discussed, with the overall work plan for all working groups approved in [2].  According to the arrangements agreed during RAN #105 the RAN4 discussions starts in RAN4 #113 with the following objectives listed in the work plan:

The discussion during RAN1 #118bis meeting is summarized in [3], with the following agreements captured in the RAN1 Chair notes:

Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): N = 9: D = 8, D = 20, D = 30 N = 8: D = 20 Companies to report which subframe indices are included in the set of consecutive downlink subframes. Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): N = 9: U = 8, U = 20, U = 30 N = 8: U = 20 Companies to report which subframe indices are included in the set of consecutive uplink subframes. Other combinations are not precluded to be reported by companies For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

This contribution focuses on the impact of the downlink/uplink pattern under discussion in RAN1 on the frequency error RF requirement.

# 2	Discussion

3GPP introduced the NB-IoT technology into the specifications in Rel-13.  Support for NB-IoT was envisioned to include half-duplex FDD devices, and one issue that was discussed in both RAN4 and RAN1 was the impact of long uplink repetition periods on the frequency error compensation by half-duplex FDD devices.  The problem statement and the related analysis is well summarized in [4] (it should be noted that other companies also contributed their analysis in both working groups at that time).  The resolution of the NB-IoT discussion on uplink compensation gaps is captured in the RAN4 LS [5] and the following RAN1 agreement [6]:

RAN4 specified the frequency error requirement for NB-IoT devices as follows [7]:

The uplink compensation gap procedure was captured in Clause 10.2.3.4 of TS36.211 [8]:

As 3GPP considers options related to the UL/DL pattern for the new TDD mode for IoT NTN, a similar set of problem statements emerges:

-	During the DL portion of the pattern, are there enough reference signal resources available for the UE to correct frequency error which accumulated during the remainder of the pattern?

-	During the UL and DL-UL gap portions of the pattern, what is the maximal accumulation of frequency error that needs to be corrected during the DL portion?

A summary of the key results and conclusions from [4] are relevant to these problem statements and is provided below:

- Analysis considered two initial frequency error scenarios with 900 Hz (0.45 ppm at 2 GHz) and 500 Hz (0.25 ppm at 2 GHz) and tested the frequency offset estimator performance at {-10, -12, -15} dB SNR - In the minimum valid set of NB-RS, which is available for all NB-IoT UEs, every 20 ms there are 4 subframes with NB-RS available for frequency offset estimation. With each subframe consisting of 8 NB-RS REs, we have 32 REs available for frequency offset estimation every 20 ms. - With SNR = -12 dB and the minimum valid set the initial frequency error of 0.25 ppm can be corrected to an accuracy of 0.2 ppm within 160 ms; correction to an accuracy of 0.1 ppm within is not possible within 600 ms - With SNR = -12 dB and the maximum valid set the initial frequency error of 0.25 ppm can be corrected to an accuracy of 0.2 ppm within 40 ms and to an accuracy of 0.1 ppm within 240 ms - Frequency offset correction for lower SNR levels (such as -15 dB) is possible only with greater valid subframe allocations; with the maximum valid set the initial frequency error of 0.25 ppm can be corrected to an accuracy of 0.2 ppm within 120 ms and to an accuracy of 0.1 ppm within 600 ms

RAN1 has started the discussion of specifying the DL/UL pattern for the IoT NTN TDD mode with several options of the pattern parameters, which will be used in subsequent evaluations.  In the context of the potential impact of these parameters on the RF frequency error requirement, a summary of the possible interpretation of the parameters and their relationship is shown in Table 1 below.

Table 1: Summary of UL/DL pattern options and worst-case impact on frequency error

|  | Units | Case 90/8/8 | Case 90/8/20 | Case 90/8/30 | Case 90/20/8 | Case 90/20/20 | Case 90/20/30 | Case 90/30/8 | Case 90/30/20 | Case 90/30/30 | Case 80/20/20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Period | subframes | 90 | 90 | 90 | 90 | 90 | 90 | 90 | 90 | 90 | 80 |
| Consecutive DL | subframes | 8 | 8 | 8 | 20 | 20 | 20 | 30 | 30 | 30 | 20 |
| Consecutive UL | subframes | 8 | 20 | 30 | 8 | 20 | 30 | 8 | 20 | 30 | 20 |
| Tx-Rx gap (Last DL to first UL) | subframes | 74 | 62 | 52 | 62 | 50 | 40 | 52 | 40 | 30 | 40 |
| Tx-Rx gap + UL (Last DL to last UL) | subframes | 82 | 82 | 82 | 70 | 70 | 70 | 60 | 60 | 60 | 60 |
| Worst-case freq error before correction | ppm | 0.182 | 0.182 | 0.182 | 0.17 | 0.17 | 0.17 | 0.16 | 0.16 | 0.16 | 0.16 |
| NOTE 1: Assuming f > 1 GHz and the baseline NB-IoT frequency error requirement of 0.1 ppm NOTE 2: Assuming low-cost oscillator with 1 ppm per second drift without correction NOTE 3: Tx-Rx gap calculated assuming consecutive DL and consecutive UL are placed farthest apart within the period N; this depends on how RAN1 structures the consecutive DL/UL subframes within the period N (currently FFS) NOTE 4: Assuming 15 kHz SCS |  |  |  |  |  |  |  |  |  |  |  |

Observation 1:	Based on the assumption of the frequency drift of a low-cost oscillator of 1 ppm per second and f > 1 GHz, the UE may need to correct frequency errors in the range of 0.16 to 0.182 ppm within the DL period ranging from 8 to 20 subframes, depending on the UL/DL pattern selected.

Referring to the NB-RS density analyzed in [4], Table 2 below illustrates a comparison of available estimation resources across the pattern options considered by RAN1 for the IoT NTN TDD mode against the NB-IoT UL compensation gap baseline of 40 ms.

Table 2: Comparison of available estimation resources (assuming minimum set of NB-RS)

| Num DL subframes | 8 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| Num NB-RS subframes (minimum set) | 1 | 4 | 6 | 8 |
| Num NB-RS REs | 8 | 32 | 48 | 64 |
| Processing loss relative to NB-IoT baseline (dB) | -9 | -3 | -1 | 0 |

Observation 2:	Based on the TDD mode pattern parameters under consideration in RAN1, there is an overall processing loss of up to 9 dB relative to the NB-IoT baseline assumptions of NB-RS available for frequency error correction.

In summary, as we compare the parameters under consideration for the TDD mode pattern to the assumptions based on which the NB-IoT UL compensation gaps are specified, all options have fewer DL subframes than agreed for NB-IoT, and the impact on frequency offset estimation accuracy should be investigated to assist RAN1 in their effort to define the TDD mode pattern parameters.

Proposal 1:	RAN4 should perform the related investigations and inform RAN1 about the impact of the TDD mode frame structure parameters on the IoT NTN frequency error requirement.

# 3	Conclusions

This contribution focuses on the impact of the downlink/uplink pattern under discussion in RAN1 on the frequency error RF requirement with the following observations and proposals:

Observation 1:	Based on the assumption of the frequency drift of a low-cost oscillator of 1 ppm per second and f > 1 GHz, the UE may need to correct frequency errors in the range of 0.16 to 0.182 ppm within the DL period ranging from 8 to 20 subframes, depending on the UL/DL pattern selected.

Observation 2:	Based on the TDD mode pattern parameters under consideration in RAN1, there is an overall processing loss of up to 9 dB relative to the NB-IoT baseline assumptions of NB-RS available for frequency error correction.

Proposal 1:	RAN4 should perform the related investigations and inform RAN1 about the impact of the TDD mode frame structure parameters on the IoT NTN frequency error requirement.

# 4	References

RP-242415, “New WID on introduction of IoT-NTN TDD mode,” 3GPP RAN #105, September 2024

R1-2408273, “Work plan for New WID on introduction of IoT-NTN TDD mode,” Iridium, 3GPP RAN1 #118bis, October 2024

R1-2409039, “Feature lead summary #2 on IoT-NTN TDD mode,” 3GPP RAN1 #118bis, October 2024

R4-163255, “Simulation results of UCG parameters for NB-IoT,” Intel Corporation, 3GPP RAN4 #79, May 2016

R4-164459, “Response to LS on uplink transmission gap in NB-IoT,” 3GPP RAN4 #79, May 2016

R1-166023, “WF on UL gaps for NB-IoT,” Huawei, HiSilicon, CATT, Ericsson, Intel, MediaTek, Nokia, Samsung, Sony, u-blox, Vodafone, ZTE, Nubia Technologies, 3GPP RAN1 #84bis, May 2016

TS36.101, “E-UTRA: User Equipment (UE) radio transmission and reception”

TS36.211, “E-UTRA: Physical channels and modulation”