3GPP RAN WG4 Meeting #113		R4-2417762

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.1.1

Source:	Apple

Title:	On Rel-19 HPUE CA/DC MSD consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE RF requirements” [2]. For HPUE inter-band UL CA/EN-DC, there was the consideration of not specifying PC2/PC1.5 MSD requirements anymore but only relying on PC3 test configurations for MSD verifications or using some equations/look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD to hopefully ease the new HPUE CA/DC combination introducing process and simplify the RAN4 specifications. Along the line of such consideration, there were four options on the HPUE MSD rules proposed as captured in [2] which will be further discussed Rel-19.

In this contribution, we share our views on the possible handling for Rel-19 HPUE CA/DC MSD requirements and our preference on the MSD rules for Rel-19 HPUE CA/DC.

# Discussion

The MSD requirements have been introduced since the unveiling of LTE inter-band CA combination in Rel-11. The similar framework was also adopted for NR inter-band CA and EN-DC. In LTE, since only power class 3 (PC3) is supported for inter-band CA combinations including DL only CA combinations, MSD requirements are always specified at PC3. However, in NR, there have been PC3 (default power class), power class 2 (PC2), and power class 1.5 (PC1.5) specified for inter-band combinations. Owing to the concern that MSD is expected to be higher at PC2 and PC1.5 than at PC3 for the same band combination, RAN4 has elected to define MSD requirements for all supported power classes of a band combination. And that resulted in numerous MSD test configurations and requirements as summarized in Table 2-1.

Observation 1: In LTE, MSD requirements are only specified at PC3 as only PC3 is supported for inter-band CA combinations.

Observation 2: In NR, RAN4 has elected to define MSD requirements for all supported power classes of a band combination.

| MSD Mechanism | PC | Aggressor UL Power (dBm) |
| --- | --- | --- |
| UL harmonic | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| Harmonic mixing | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| Cross-band interference | PC3 | 23 |
|  | PC2 | 26 |
|  | PC1.5 | 29 |
| 2UL IMD | PC3 | 20 + 20 |
|  | PC2 (2Tx or 3Tx) | 23 + 23 |
|  | PC1.5 (3Tx) | 23 + 27.8 |

Table 2-1 NR MSD test configurations

Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5 where the PC2 and PC1.5 MSD requirements would not be able to leverage the analysis already performed in LTE for the same or similar band combination. Furthermore, if we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19. For example, if PC1.5 inter-band UL combination consisting of (PC2 FDD + PC2 TDD) with 2Tx or 3Tx would be introduced, then two more MSD analysis would be added.

Observation 3: Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5.

Observation 4: If we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19.

Considering that the current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process, the necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process. This concern has also triggered a recent RAN task to RAN4 to investigate a solution to enable HPUE maximum transmit power in downlink CA with single configured UL [3], though the similar issue also occurs for dual configured UL.

Observation 5: The necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process.

Owing to the above concern, RAN4 has come up with the following potential solutions on handling HPUE MSD in Rel-19 to hopefully ease the new HPUE CA/DC band combination introducing process and simplify the RAN4 specifications.

- Option 1:

- RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables.

- Option 2:

- Current working procedure of HPUE CA/DC introduction are kept unchanged.

- HPUE MSD tables are kept.

- Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed.

- Option 3:

- PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission.

- Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS.

- Option 4:

- PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission.

In our view, the MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration. There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations. On the other hand, if there is the preference from companies to recognize how much MSD would be increased with PC2 and PC1.5 configurations as compared to PC3, such information may be captured in the Annex of the technical specifications in the form of equations, LUT, or simplified guidelines.

Observation 6: MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration.

Observation 7: There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations.

Based on the above assessment, we propose RAN4 to adopt Option 3 as the way forward on handling the HPUE MSD requirements in Rel-19 and going forward.

Proposal: RAN4 to adopt Option 3 as the way forward on handling the HPUE MSD requirements in Rel-19 and going forward.

- Option 3:

- PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission.

- Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS.

# 3	Conclusion

In this contribution, we share our views on the possible handling for Rel-19 HPUE CA/DC MSD requirements and our preference on the MSD rules for Rel-19 HPUE CA/DC.

Observation 1: In LTE, MSD requirements are only specified at PC3 as only PC3 is supported for inter-band CA combinations.

Observation 2: In NR, RAN4 has elected to define MSD requirements for all supported power classes of a band combination.

Observation 3: Compared to LTE, the introduction of HPUE CA/DC combinations in NR has at least tripled the MSD efforts for a combination supporting up to PC1.5.

Observation 4: If we would follow the existing MSD framework for HPUE CA/DC combinations, more MSD analysis could be needed with new PC1.5 UL configurations in Rel-19.

Observation 5: The necessity of HPUE CA/DC MSD requirements has not only increased the RAN4 workload substantially, but also been hindering the HPUE support for numerous CA/DC combinations due to the slow MSD analysis process.

Observation 6: MSD requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance which can already be verified by the PC3 MSD requirements in conjunction with the Tx requirements under the HPUE PCMAX configuration.

Observation 7: There is essentially no need to further define separate MSD requirements other than PC3 MSD test configurations.

Proposal: RAN4 to adopt Option 3 as the way forward on handling the HPUE MSD requirements in Rel-19 and going forward.

- Option 3:

- PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission.

- Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2417098 “WF on HPUE RF requirements”, Samsung, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024

RP-241687 “Moderator's summary for offline discussion on CA Power Class Support”, Moderator (AT&T), 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024