3GPP TSG-RAN WG4 Meeting #113	R4-2417771

Orlando, United States, Nov. 2024

Source	:	Qualcomm

Title	:	24 GHz EESS band phase 2 compliance

Agenda item	:	6.21.2

WI code	: NR_mmWave_protect

Document for	:	Approval

# 1.	Introduction

In WRC-19, resolution 750, progressively stronger emissions limits were recommended for UEs and gNBs to protect passive EESS (Earth Exploration Satellite Services) in the 23.6 – 24.0 GHz band. The next significant event pertaining to this protected band is the adoption of ‘phase 2’ limits, currently recommended for September 2027, but brought ahead by several regional regulators. In RAN#105, a new WID [1] was approved for this purpose. In our submission [2], we broke down the problem into its two branches: signaling and AMPR determination. For the latter, expected back-off requirements for UEs for phase 2 limits are derived based on prior work on phase 1 limits.

# 2. 	Discussion

## 2.1	Need for new signaling

#### 2.1.1	Categories of regions, by ‘additional emissions requirements’

For convenience, the -5 dBm/200 MHz emissions requirement in the EESS frequency band of 23.6 to 24.0 GHz is referred to as EESS phase 2 in this document. In the previous WF [3], RAN4 decided to treat under the current WI, regions where NS_202 is applicable. The WI scope now applies to regions that fall into 2 categories.

The first category is for regions where the UE must comply with just the EESS phase 2 requirement and no other ‘additional emissions requirement’. This requirement is closely related to the existing NS_203 requirements (‘EESS phase 1’) and is understood to be a tightening of those requirements. This case has been analyzed, and the need for a new NS [NS_205] has been identified [2].

The second category is for regions where two ‘additional emissions requirements’ simultaneously apply: the -10 dBm/100 MHz requirement outside the SEM, and the EESS phase 2 requirement. This requirement is understood to be a tightening of the NS_202 requirements, hence the need for a new NS [NS_206] which is derived based off NS_202 and [NS_205].

#### 2.1.2	First category, regions where just the EESS phase 2 requirements apply

(This section pertains to replacement/update of NS_203) Reproduced from an earlier contribution [2] for convenience, NS_203 was instituted for UE adherence to the pre-2027 limit (+1 dBm/200 MHz), and UEs in the field before the 2027 date are expected to be ‘grandfathered in’. Changing the definition of NS_203 is therefore not feasible, and the phase 2 limits are best handled with new signaling [NS_205].

Observation 1: For regions where the EESS phase 2 emissions requirements apply exclusively (no other additional emissions requirements) a new NS solution is required, independent of NS_203.

Proposal 1:  RAN4 to create a new pair of NS ([NS_205] and [CA_NS_205]) for bands in regions where EESS phase 2 requirements are the sole ‘additional emissions requirements’.

As previously identified [2], any time there is a new combination of region and applicability date of the phase 2 regulations, a new NS must be created by RAN4 to protect legacy devices.

#### 2.1.3	Second category, regions where NS_202 applied prior to Jan 2024

When NS_202 is signalled, a UE already has significant A-MPR allowance. NS_202 A-MPR from 38.101-2 can be summarized as follows:

| UE type | Single CC | CA |
| --- | --- | --- |
| PC1 (35 dBm TRP limit) | 11.0 dB | 11.0 dB |
| PC2….7 (23 dBm TRP limit) | 1.0 dB | 2.0 dB |

In our estimation [2], there are conditions where more back off is required for EESS phase 2 compliance compared to existing A-MPR allowance for NS_202. i.e. NS_202 is no longer sufficient for all cases, due to applicability of EESS phase 2 requirement. As before, existing functionality of legacy devices brought into use before Jan 2024 must be protected. Together, this situation drives creation of a new NS [NS_206] to supersede NS_202 as defined in section 2.3

Observation 2: Regions where NS_202 applies require a new NS solution due to EESS phase 2 requirements.

Proposal 2:  RAN4 to create a new pair of NS ([NS_206] and [CA_NS_206]) to support UE compliance of EESS phase 2 requirements for bands where NS_202 and CA_NS_202 applied prior to Jan 2024.

## 2.2	A-MPR for [NS_205]

In the previous contribution [2] we shared the detailed procedure for deriving backoff values for [NS_205] based on consistency with treatment previously adopted for NS_203. The outcome of our analysis is reproduced here as our proposal for A-MPR for [NS_205].

Proposal 3: [NS_205] A-MPR is adopted as shown below (table references are placeholders)

#### 6.2.3.6 A_MPR for [NS_205] ##### 6.2.3.6.1 PC1 For a single contiguous allocation, the A-MPR is 8.0 dB if offset frequency < BWChannel, 6.0 dB otherwise. For non-contiguous allocations, the A-MPR is 13.0 dB if offset frequency < BWChannel, 6.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1] ##### 6.2.3.6.3 PC3 For a single contiguous allocation, the A-MPR is 2.0 dB if offset frequency < BWChannel, 0 dB otherwise. For non-contiguous allocations, the A-MPR is 8.0 dB if offset frequency < BWChannel, 0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1]

Proposal 4: [CA_NS_205] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2A.3.6 A_MPR for [CA_NS_205] ##### 6.2A.3.6.1 PC1 For contiguous ULCA with a single contiguous allocation, the A-MPR is 9.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 13.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA ##### 6.2A.3.6.3 PC3 For contiguous ULCA with a single contiguous allocation, the A-MPR is 5.0 dB if offset frequency < BWintraCA, 0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 8.0 dB if offset frequency < BWintraCA, 0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA

## 2.3	A-MPR for [NS_206]

As noted earlier [NS_206] represents simultaneous application of 2 independent emissions requirements:

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f  24.0 GHz | -5 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

Fortunately, the back off requirement for each emissions limit (each row in table) is separately known, by the A-MPR for NS_202 and [NS_205], respectively. Consistency dictates that the A-MPR for [NS_206] is determined by the greater of the A-MPRs for the 2 cases. More elaborately:

Observation 3: Consistency with previous A-MPR work dictates that A_MPR for [NS_206] is determined as max(A-MPR  for NS_202, A-MPR for [NS_205]).

Observation 4: Consistency with previous A-MPR work dictates that A_MPR for [CA_NS_206] is determined as  max(A-MPR for CA_NS_202, A-MPR for [CA_NS_205]).

The A-MPR for [NS_206] can be derived based on the principles in obs. 3, specifically by updating the [NS_205] proposal with values from NS_202 (see table in section 2.1.3). The change marks in the proposal highlight the changes needed for [NS_206] from the A-MPR proposal for [NS_205].

Proposal 5: [NS_206] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2.3.7 A_MPR for [NS_206] ##### 6.2.3.7.1 PC1 For a single contiguous allocation, the A-MPR is 11.0 dB. For non-contiguous allocations, the A-MPR is 13.0 dB if offset frequency < BWChannel, 11.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.7-1] ##### 6.2.3.7.3 PC3 For a single contiguous allocation, the A-MPR is 2.0 dB if offset frequency < BWChannel, 1.0 dB otherwise. For non-contiguous allocations, the A-MPR is 8.0 dB if offset frequency < BWChannel, 1.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.7-1]

Similarly, the A-MPR for [CA_NS_206] can be constructed as follows:

Proposal 6: [CA_NS_206] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2A.3.7 A_MPR for [CA_NS_206] ##### 6.2A.3.7.1 PC1 For contiguous ULCA with a single contiguous allocation, the A-MPR is 11.0. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 13.0 dB if offset frequency < BWintraCA, 11.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.7-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA ##### 6.2A.3.7.3 PC3 For contiguous ULCA with a single contiguous allocation, the A-MPR is 5.0 dB if offset frequency < BWintraCA, 2.0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 8.0 dB if offset frequency < BWintraCA, 2.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.7-1] BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA

## 2.4	Applicable frequency bands

The back-off requirement is often gated by the need to sufficiently suppress first order spurious products [2]. These products only recede 1 dB for every dB of signal power back off, and generally comprise products generated prior to the PA. Furthermore, location for these first order products cannot be generalized, for example ‘within x GHz of band’ or similar. All that can be said is that these first order products meet the general requirements for FR2 (-13 dBm/MHz), albeit with the help of MPR.

It is also well understood that FR2 UEs tend to have minimal front-end filtering due to relaxed emissions requirements, compared to say FR1 TDD bands.

Together, this means that when the back-off requirement is gated by first order spurs, it is not feasible to establish a ‘safe enough’ frequency distance from the band across all implementations, outside which first order spurs also start to naturally decline in level.

For this reason, it is not feasible to include EESS phase 2 requirements in the general coex requirements of n257 without first imposing significant new MPR for all bands.

Proposal 7: The new NSs for EESS phase 2 requirements apply to all bands approved for use in regions where the emissions requirements apply.

# 3.	Conclusion

Observation 1: For regions where the EESS phase 2 emissions requirements apply exclusively (no other additional emissions requirements) a new NS solution is required, independent of NS_203.

Proposal 1:  RAN4 to create a new pair of NS ([NS_205] and [CA_NS_205]) for bands in regions where EESS phase 2 requirements are the sole ‘additional emissions requirements’.

Observation 2: Regions where NS_202 applies require a new NS solution due to EESS phase 2 requirements.

Proposal 2:  RAN4 to create a new pair of NS ([NS_206] and [CA_NS_206]) to support UE compliance of EESS phase 2 requirements for bands where NS_202 applied prior to Jan 2024.

Proposal 3: [NS_205] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2.3.6 A_MPR for [NS_205] ##### 6.2.3.6.1 PC1 For a single contiguous allocation, the A-MPR is 8.0 dB if offset frequency < BWChannel, 6.0 dB otherwise. For non-contiguous allocations, the A-MPR is 13.0 dB if offset frequency < BWChannel, 6.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1] ##### 6.2.3.6.3 PC3 For a single contiguous allocation, the A-MPR is 2.0 dB if offset frequency < BWChannel, 0 dB otherwise. For non-contiguous allocations, the A-MPR is 8.0 dB if offset frequency < BWChannel, 0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1]

Proposal 4: [CA_NS_205] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2A.3.6 A_MPR for [CA_NS_205] ##### 6.2A.3.6.1 PC1 For contiguous ULCA with a single contiguous allocation, the A-MPR is 9.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 13.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA ##### 6.2A.3.6.3 PC3 For contiguous ULCA with a single contiguous allocation, the A-MPR is 5.0 dB if offset frequency < BWintraCA, 0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 8.0 dB if offset frequency < BWintraCA, 0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA

Observation 3: Consistency with previous A-MPR work dictates that A_MPR for [NS_206] is determined as

max(A-MPR  for NS_202, A-MPR for [NS_205]).

Observation 4: Consistency with previous A-MPR work dictates that A_MPR for [CA_NS_206] is determined as

max(A-MPR  for CA_NS_202, A-MPR for [CA_NS_205]).

Proposal 5: [NS_206] A-MPR is adopted as shown below (table references are placeholders):

#### 6.2.3.7 A_MPR for [NS_206] ##### 6.2.3.7.1 PC1 For a single contiguous allocation, the A-MPR is 11.0 dB. For non-contiguous allocations, the A-MPR is 13.0 dB if offset frequency < BWChannel, 11.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.7-1] ##### 6.2.3.7.3 PC3 For a single contiguous allocation, the A-MPR is 2.0 dB if offset frequency < BWChannel, 1.0 dB otherwise. For non-contiguous allocations, the A-MPR is 8.0 dB if offset frequency < BWChannel, 1.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.7-1]

Proposal 6: [CA_NS_206] A-MPR is adopted as shown below (table references are placeholders):yy

#### 6.2A.3.7 A_MPR for [CA_NS_206] ##### 6.2A.3.7.1 PC1 For contiguous ULCA with a single contiguous allocation, the A-MPR is 11.0. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 13.0 dB if offset frequency < BWintraCA, 11.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.7-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA ##### 6.2A.3.7.3 PC3 For contiguous ULCA with a single contiguous allocation, the A-MPR is 5.0 dB if offset frequency < BWintraCA, 2.0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 8.0 dB if offset frequency < BWintraCA, 2.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.7-1] BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA

Proposal 7: The new NSs for EESS phase 2 requirements apply to all bands approved for use in regions where the emissions requirements apply.

# 4.	References

[1] 	RP-242371, ‘New WID on mmWave UE spurious emission’, China Unicom et al, RAN#105, Melbourne, AUS, Sept. 2024.

[2]      R4-2415726, ‘24 GHz EESS band phase 2 requirements and UE compliance’, Qualcomm, RAN4#112bis, Hefei, China, Oct 2024.

[3]     R4-2417083, ‘WF on UE RF requirements for EESS protection’, Huawei et al, RAN4#112bis, Hefei, China, Oct 2024.