3GPP TSG RAN WG4 Meeting #113 	R4-2417772

Orlando, USA, November 18 - 22, 2024

Title:	RRM and demodulation considerations and simulation assumptions for IoT_NTN_TDD

Source: 	Iridium

Type:	Discussion

Document for:	Discussion

Agenda Item:	7.28.2

# Introduction

## The WID summary

In RAN#105 new WID on introduction of IoT NTN TDD mode operating in L-band (MSS band 1616 -1626.5 MHz) was approved in [1]. This contribution shares our view on the new IoT-NTN TDD RF band numbering, EARFCN mapping and UE RF characteristics.

The Objectives of the WID, as per [1]:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements).

## RAN1 agreements to date

As RAN1 is the lead WG for this WI, the discussions started at October meetings. For the full list of agreements on IoT_NTN_TDD, please see RAN1 Feature Lead summary from October meetings [2].
Here we are mentioning only the RAN1 agreements that are of RRM and demodulation interest, to be considered:

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes):

- N = 9:

  - D = 8

  - D = 20

  - D = 30

- N = 8:

  - D = 20

- Companies to report which subframe indices are included in the set of consecutive downlink subframes.

- Other combinations are not precluded to be reported by companies

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes):

- N = 9:

  - U = 8

  - U = 20

  - U = 30

- N = 8:

  - U = 20

- Companies to report which subframe indices are included in the set of consecutive uplink subframes.

- Other combinations are not precluded to be reported by companies

- For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances.

  - FFS: What is the TDD pattern

# Discussion

## Proposed Definition of the periodic TDD-DL-UL-Pattern

The Figure 1 below (from the contribution [3]) describes the proposed TDD-DL-UL-Pattern:

Figure 1 Definition of the periodic TDD-DL-UL-Pattern

The parameters in the Figure 1 could be described as:

NTN-TDD-structure-r19 ::= SEQUENCE {

nrOfDownlinkSubframes 			INTEGER

nrOfUplinkSubframes 				INTEGER

DownlinkToUplinkGuardPeriod 	INTEGER

hdfdd-offset-subframes    		INTEGER}

FFS: Their maximum values and bit allocations

Proposal 1: For the IoT-NTN TDD, the definition of the periodic TDD-DL-UL-Pattern depicted in Figure 1 to be adopted for RRM and demodulation considerations.

## Down-scoped list of parameters for RRM and Demodulation consideration

As per the WID [1]: “The periodic pattern should consist… which is periodic every N radio frames, with N=9 as baseline”, we propose taking N=9 for demodulation evaluation.

Proposal 2: The value of DL-UL-TransmissionPeriodicity used for demodulation NB-IoT NTN TDD evaluation is N=9 as baseline.

In order to simplify the evaluation phase, we propose focusing on a subset of RAN1 parameters form the agreements listed above, for RRM and demodulation considerations.

Proposal 3: The values of nrOfDownlinkSubframes and nrOfUplinkSubframes equal to 8 subframes is taken as a baseline.

Proposal 4: With respect to SIB1-NB repetition settings, we propose number of repetitions be set to 16, for the purpose of evaluation.

## Proposing RRM topics to be addressed

The periodic, long SAN DTX periods (every Nth radio frame) will have impact on RRM. The Paging and Sleep Cycles, Discontinuous Reception (eDRX) and Power Saving Mode (PSM) cycles are among proposed to be evaluated.

Proposal 5: The Paging and Sleep Cycles, Discontinuous Reception (eDRX) and Power Saving Mode (PSM) cycles are to be evaluated in the presence of DL-UL-TransmissionPeriodicity of 1 in N radio frames (N=9 as baseline).

Proposal 6: RAN4 is kindly asked to investigate and check if other RRM requirements are impacted by the new IoT NTN TDD pattern, for the normative phase.

# Conclusion

In this contribution, for background information, we summarized RAN1 work on this WI so far and have suggested a subset of initial parameters to evaluate RRM and demodulation impacts from IoT-NTN TDD mode. Our observations and proposals are listed below:

Proposal 1: For the IoT-NTN TDD, the definition of the periodic TDD-DL-UL-Pattern depicted in Figure 1 to be adopted for RRM considerations.

Figure 1 Definition of the periodic TDD-DL-UL-Pattern

The parameters in the Figure 1 could be described as:
NTN-TDD-structure-r19 ::= SEQUENCE {

nrOfDownlinkSubframes 			INTEGER

nrOfUplinkSubframes 				INTEGER

DownlinkToUplinkGuardPeriod 	INTEGER

hdfdd-offset-subframes    		INTEGER}

FFS: Their maximum values and bit allocations

Proposal 2: The value of DL-UL-TransmissionPeriodicity used for demodulation NB-IoT NTN TDD evaluation is N=9 as baseline.

Proposal 3: The values of nrOfDownlinkSubframes and nrOfUplinkSubframes equal to 8 subframes is taken as a baseline.

Proposal 4: With respect to SIB1-NB repetition settings, we propose number of repetitions be set to 16, for the purpose of evaluation.

Proposal 5: The Paging and Sleep Cycles, Discontinuous Reception (eDRX) and Power Saving Mode (PSM) cycles are to be evaluated in the presence of DL-UL-TransmissionPeriodicity of 1 in N radio frames (N=9 as baseline).

Proposal 6: RAN4 is kindly asked to investigate and check if other RRM requirements are impacted by the new IoT NTN TDD pattern, for the normative phase.

# References

[1]	RP-242415, “New WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

[2] 	R1-2409039, “Feature lead summary #2 on IoT-NTN TDD mode”, 3GPP TSG RAN WG1 Meeting #118bis, Hefei, China, October 14-18, 2024

[3]   R1-2407772, Discussion on IoT-NTN TDD mode, Hefei, CN, October 14th – 18th, 2024