3GPP TSG-RAN4 Meeting #113	R4-2417774

# Orlando, US, 18th – 22nd November, 2024

Title:	(NR_n30-Core) n30 NS_21 AMPR correction

Source:	Qualcomm Incorporated

Agenda item:	4.2.2

Release:	Rel-17

WI: 	NR_n30-Core

Lead WG: 	RAN4

Document for:	Discussion

# 1.	Introduction

N30 NS_21 AMPR was found to be not sufficient to accommodate the requirements and test fails were observed. After diligence, it was noticed that the simulation data and proposal for AMPR were not aligned very well and no analysis was provided for other than BPSK and QPSK in original contribution [1].  This paper discusses this issue in more detail and proposed change is companion CR [3].

# 2. 	Discussion

The simulation results kindly provided to address the tighter requirements for NS_21 in [2] are as below.

Figure 1. Original simulation results from [1] and resulting specification. Black thin line is covered AMPR region

From figure, comparing areas, it can be seen that where simulations showed 3.5 dB, the specification became 4 dB for CP-OFDM and where simulation showed > 2 dB, the specification became 2.5 dB.

In the paper, DTF-s 64QAM was not provided and the AMPR was defined to be the same for 64QAM as it is for QPSK. Below in Figure 2 we provide QPSK and 64QAM simulations and show the overall failed RB allocations for CP-OFDM and DFT-s-OFDM captured across different modulation schemes.

Figure 2. QPSK and 64QAM data with failed RB allocations identified

The data indicates multiple outer/inner RB allocations that fail and need for modification in the current AMPR specification. To enable proper AMPR and use of same RF TX than for non-NS cases for n30, we are proposing the change of AMPR for 5 MHz as follows in companion CR [3]. The new AMPR in graphical format is in Figure 3. Region ‘A4’ is also added to account for AMPR for outer/inner allocations in that region.

Figure 3. Graphical representation of the proposed AMPR table in [3].

And below is the modified table for convenience.

Table 6.2.3.14-2: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.72 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.72 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB >0.72 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.72 MHz/12/SCS | Region A4 RBstart>0.36MHz/12/SCS RBend<3.96MHz/12/SCS 0.72 MHz/12/SCS < LCRB ≤2.52MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  | Outer/Inner |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | ≤2.0 | ≤4.0 | ≤ 3.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 3.5 | ≤ 3.0 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 | ≤ 4.5 | ≤ 3.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 | ≤ 4.0 | ≤ 4.5 | ≤ 3.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  |  | 16 QAM | 5.0 | ≤5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

We did not see the need to change the 10 MHz AMPR.

# Conclusion

We discussed the background and motivation for changing the AMPR for NS_21 and presented new simulation results. We also submitted a CR [3] for agreement based on this discussion.

# Reference

[1]	] R4-2117960, “Follow up on additional requirements and A-MPR for NS_21”, Apple, RAN4#101-e

[2]	RP-221868, “CR to 38.101-1 on corrections of NS_21 requirements”, Apple, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6 - 11, 2022

[3]	R4-2417775, “(NR_n30-Core) n30 NS_21 AMPR correction CR to 38.101-1”, 3GPP TSG-RAN4 Meeting #113, Orlando, US, 18th – 22nd November, 2024