3GPP TSG-RAN WG4 Meeting #113	R4-2417780

Orlando, US, Nov 18 – Nov 22, 2024

Source:	Tejas Networks

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	7.17.4.1

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#112bis in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1] and the ad-hoc minutes [2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion, in this paper, we provide our views on issues related to requirements for beam prediction.

Discussion

Requirements for beam prediction

During the RAN4 #112bis meeting, RAN4 agreed to study the following aspects related to beam prediction requirements.

Measurement period for beam prediction

Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc)

The measurement period for beam prediction is essential for balancing accuracy and performance in AI/ML-enabled beam management. Legacy RAN4 RRM specifications set durations for measuring reference signals, and these should be reused while factoring in sharing factor and DRX configurations. Extending this period excessively could negate the efficiency of AI/ML models designed for rapid and effective beam prediction.

Observation 1: The legacy measurement period should be considered for reuse, with adaptations for factors like sharing factor and DRX, to maintain prediction efficiency.

The number of SSB samples needed for accurate beam predictions may depend on the complexity of the UE implementation. However, to prevent non-uniformity, even though the complexity of UE implementation influences sample requirements, the number of samples should be standardized. Additionally, AI/ML models, whether deployed on the UE side or the network side (e.g., gNB), may require different performance monitoring approaches. Since the control of AI/ML models might reside with an entity different from where the models are deployed, performance monitoring procedures must account for this distinction. Scenarios addressing these aspects are outlined in TR 38.843 [3]. Currently, for BM-Case1 and BM-Case2 with a network-side AI/ML model, no specific AI/ML-related impacts have been identified. Performance monitoring delays must be minimized to ensure that AI/ML models can effectively predict a wider set of beams from a smaller set of samples.

Proposal 1: The number of required SSB samples should be standardized to prevent non-uniformity, even though UE implementation complexity influences sample requirements.

Proposal 2: Performance monitoring delay must be minimized to maintain the effectiveness of AI/ML in beam management and ensure timely and accurate beam predictions

KPIs for beam prediction

RAN4 has studied various KPI metrics to evaluate the performance of AI/ML-enabled beam management. The following is a snippet from 3GPP TR 38.843 [3].

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

Accurate L1-RSRP prediction is essential because it directly impacts the reliability of beam selection. High L1-RSRP prediction accuracy ensures that the model not only identifies the strongest beams but also reflects their true signal strength, improving the network's ability to maintain robust connectivity. By setting a defined margin for acceptable RSRP deviation, this metric verifies that the predicted signal strength is close enough to the actual measured value, thus reducing the risk of selecting suboptimal beams and enhancing overall network performance under real-world conditions.

For beam prediction evaluation, Top-K/1(%) accuracy is a critical KPI alongside RSRP accuracy. This metric measures the likelihood that the actual strongest beam is correctly identified within the Top-K predictions, ensuring that the model’s beam selection aligns closely with the strongest available signal. Combined with RSRP accuracy, which validates the predicted signal strength accuracy, Top-K/1 accuracy confirms that the model prioritizes optimal beams, helping to maintain effective connectivity even in rapidly changing network conditions. Together, these KPIs offer a balanced assessment of the model’s ability to predict both beam selection and strength.

Observation 2: Accurate L1-RSRP and Top-K/1 metrics ensure reliable identification of the strongest beams.

We propose a new option, Option 5 to evaluate the consistency and accuracy of predicted beams based on beam rank, which refers to the order of beams based on their signal strength from the strongest to the weakest. Ensuring rank consistency is essential, as it confirms that the predicted beams not only include the strongest options but are also correctly ordered by strength. This accuracy is vital for reliable connectivity, particularly in dynamic environments, as it supports optimal signal quality and efficient resource utilization.

To evaluate the consistency and accuracy of the predicted beam rankings compared to the actual strongest beams, the following two-step procedure can be implemented:

Option 5:

### Step 1: Evaluate the correlation based on the inclusion of predicted beams within the Top-K strongest actual beams

- Determine how many of the predicted beams are present in the list of Top-K strongest beams.

- Compute the correlation score as the percentage of predicted beams that are in the Top-K strongest beams:

Correlation score=Number of predicted beams in Top-KK*100%

For example, if K = 6, and 4 out of 6 predicted beams are in the Top-K strongest beams, the correlation score would be 66.67%.

### Step 2: Assess rank order consistency and quantify deviation

- Rank predicted beams with respect to ground truth for corresponding beams:

- Re-rank the Top-K predicted beams based on their ground truth RSRP values as measured. This provides a ranking of the predicted beams according to actual signal strength.

- Compare Ranks and Quantify Consistency:

  - For each predicted beam, compare its position in the predicted Top-K ranking to its corresponding position in the ground truth Top-K ranking.

  - If the rank of a predicted beam matches its rank in the ground truth beams list, mark it as a correct prediction.

  - If the rank does not match, calculate the Rank Deviation as the absolute difference between the predicted rank and the actual rank for that beam:

Rank Deviation= Predicted Rank-Actual Rank

- Compute Rank Consistency Score:

  - Use the Rank Deviation values to calculate a Rank Consistency Score:

    - Count the number of correct predictions (where Rank Deviation = 0).

    - For beams with Rank Deviations, you may consider:

      - Weighted Rank Consistency Score: Penalize the Rank Consistency Score based on the magnitude of deviation, such as by subtracting points for each level of deviation (e.g., -1 for each rank deviation).

      - Threshold Score: Set a threshold for acceptable rank deviations, such as allowing ±x rank deviation as consistent, to account for minor prediction variance. The value of x can be further discussed.

  - Example formula for Rank Consistency Score:

Rank Consistency Score=Correct Predictions within ThresholdK*100%

Final Evaluation:

The final consistency metric would combine the Beam Inclusion Correlation (from Step 1) and Rank Consistency Score (from Step 2) to assess both the presence and rank order accuracy of predicted beams.

This procedure provides a robust measure of beam prediction performance, ensuring that the model’s predictions are accurate not only in identifying relevant beams but also in correctly ranking them relative to actual signal strengths.

Observation 3: Consistency and rank accuracy in beam predictions ensure reliable connectivity by accurately prioritizing the strongest beams.

Observation 4: Option 5’s two-step method verifies that predicted beams are both present in the Top-K and correctly ranked, supporting optimal network performance.

Proposal 3: Option 5 should be considered alongside other metrics, such as Option 1 and Option 2, to provide a comprehensive evaluation of beam prediction accuracy and rank consistency for beam management requirements.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: The legacy measurement period should be considered for reuse, with adaptations for factors like sharing elements and DRX, to maintain prediction efficiency.

Observation 2: Accurate L1-RSRP and Top-K/1 metrics ensure reliable identification of the strongest beams.

Observation 3: Consistency and rank accuracy in beam predictions ensure reliable connectivity by accurately prioritizing the strongest beams.

Observation 4: Option 5’s two-step method verifies that predicted beams are both present in the Top-K and correctly ranked, supporting optimal network performance.

Proposal 1: The number of required SSB samples should be standardized to prevent non-uniformity, even though UE implementation complexity influences sample requirements.

Proposal 2: Performance monitoring delay must be minimized to maintain the effectiveness of AI/ML in beam management and ensure timely and accurate beam predictions

Proposal 3: Option 5 should be considered alongside other metrics, such as Option 1 and Option 2, to provide a comprehensive evaluation of beam prediction accuracy and rank consistency for beam management requirements.

- References

- R4- R4-2417210, WF on requirements for AI/ML air interface, Qualcomm Incorporated, Hefei, Anhui, China, 14th – 18th October, 2024.

- R4-2417188, AI/ML ad-hoc meeting minutes, Qualcomm Incorporated, Hefei, Anhui, China, 14th – 18th October, 2024.

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.

- R4-2415231, Topic summary for [112-Bis][127] NR_AIML_air, Qualcomm.