3GPP TSG-RAN WG4 Meeting #113	R4-2417781

Orlando, US, Nov 18 – Nov 22, 2024

Source:	Tejas Networks

Title:	On Testability and Interoperability aspects for AI/ML Based Beam Management

Agenda item:	7.17.4.2

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#112bis in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1] and the ad-hoc minutes [2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide our views on issues related to testability and interoperability aspects.

Discussion

Test setup requirements

The testability aspects of AI/ML enabled beam management and related testing setup was further discussed in the recent RAN4#112bis meeting, leading to the following agreements.

Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity, number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

AI/ML-enabled beam management testing can be summarized in three main steps: First, the Device Under Test (DUT), which is the UE, measures the smaller set B beams transmitted by the test equipment and predicts the larger set A, sending the prediction to the test equipment (TE). Second, the TE transmits the set A beams, and the DUT measures these beams and sends the data back to the TE to establish a ground truth. Finally, the TE compares the DUT's prediction results with the reported ground truth to evaluate performance. The reliability of results from one test cycle depends on precise measurements and consistency; therefore, running the test under varying conditions helps ensure accuracy and a thorough assessment of the AI/ML model's capability.

The existing FR2 NR MIMO OTA test setup has the ability to simulate CDL channel models (CDL-A InO and CDL-C UMi) and the resulting AoA/ZoA after BS filtering. However, this setup is insufficient for AI/ML-based beam management testing, which requires emulating a higher number of Tx beams between 8 and 16 for set B and up to 64 to 128 for set A. The current system's constraints make it incapable of simulating the required number of different BS beams and implementing complex multi-path fading CDL model conditions simultaneously.

A multi-probe anechoic chamber test setup is critical for accurately verifying the feature due to the need for emulating a wide range of beam patterns and angles of arrival (AoA) and departure (AoD). In typical test setups, limitations on the number of probes may restrict the ability to simultaneously emulate all Set A/B beams, potentially leading to incomplete testing. Additionally, the ability to reproduce the diverse spatial conditions found in real-world environments, such as varying AoAs, is essential for accurate performance evaluation. To address these challenges, AI/ML techniques can leverage randomness in AoA and AoD sampling, enabling the prediction of a larger set of beam responses from a smaller subset of beams. By introducing sufficient randomness and capturing spatial correlations across different angles, the AI model can accurately map the subset of beams to the larger beam set. Furthermore, a multi-probe setup allows for the simultaneous emulation of more beam patterns, ensuring that the spatial diversity required for effective training and testing is captured, thereby enhancing the accuracy and reliability of the feature verification.

Observation 1: Current FR2 MIMO OTA test system is not capable to simulate the large number beams (8-16 for set B and 64-128 for set A) for AIML enabled beam management testing.

Observation 2: To achieve an accurate L1-RSRP prediction for Set A, sufficient randomness and variation in the time and spatial domains of Set B beams is required.

Proposal 1: A multi-probe test setup with diverse beam patterns and AoAs, along with sufficient randomness and spatial variation in Set B beams, is required to enable reliable prediction of Set A beams.

Allowing UE rotation within the multi-probe anechoic chamber can reduce the requirement for a large number of probes by enabling the reuse of probes to simulate different beam directions. By rotating the UE between BS beam transmissions, fewer probes may be needed to cover multiple beam scenarios. However, the practical feasibility, including potential implementation complexity and testing delays, would need to be addressed by test equipment vendors to ensure that the solution remains efficient and reliable.

Observation 3: Enabling UE rotation could help reduce the number of probes required for AI/ML-enabled beam management testing

For Rel-19 AI-BM, the focus should remain on testing scenarios involving a single TRP, as this forms the baseline for AI-BM prediction relevance and testability.

Proposal 2: RAN4 should limit the discussion to single TRP cases for Rel-19 AI-BM to establish a foundational assumption for testability studies.

Channel Model

Regarding the open issue of which channel model to be used during the conformance testing, companies in RAN4#112 had provided their views where some companies were advocating for CDL channel model whereas the others were in the opinion of using simplified channel models like TDL. There are pros and cons of each of the CDL and TDL channel models which are well known and need to be considered before deciding the channel model for the conformance testing of AI/ML-enabled BM use case.

- The CDL model is more suited for realistic AI/ML BM testing as it emulates complex, real-world propagation environments with multiple AoAs and spatially dispersed multipath clusters. These features are crucial for accurately assessing AI/ML algorithms that rely on detailed spatial information to predict and manage signal beams effectively. Unlike TDL, which cannot simulate 3D beam patterns or accurately represent the distinct angles and delays seen in practical deployments, CDL allows for beam-specific RSRP (Reference Signal Received Power) evaluations that are essential for dynamic beam management. CDL’s realistic spatial differentiation, achieved by supporting a wide range of AoAs, makes it indispensable for evaluating how UEs perform in high-density, multipath scenarios typical of FR2.

- Observation 4: CDL models provide the multi-directional AoA and spatial characteristics needed for accurate conformance testing, which are absent in TDL models, limiting TDL’s applicability in real-world assessments.

- Observation 5: The CDL model more closely aligns with the multi-beam gNodeB deployment scenarios expected in FR2, providing a relevant testing environment for AI/ML-based beam management algorithms.

- While the CDL model offers critical advantages in accuracy, it requires a complex test setup with numerous test probes to emulate each multipath cluster. To address this, several companies at RAN4#112bis proposed a Simplified CDL model, which reduces the number of clusters while retaining the key spatial characteristics necessary for AI/ML BM testing. By focusing on the most influential clusters, the Simplified CDL model allows for a cost-effective, feasible testing environment that still captures realistic signal behaviour. This model can simulate sparse FR2 channels effectively and provides a practical balance between testing complexity and realism.

- Observation 6: The standard CDL model requires a complex setup with multiple probes, increasing test system cost and complexity.

- Proposal 3: To ensure both testing feasibility and conformance accuracy, it is recommended to adopt the Simplified CDL model as the baseline for AI/ML BM testing in FR2, as it retains essential spatial features and requires fewer resources than the standard CDL model.

How to avoid UE cheating

During the RAN4 #112bis meeting, the issue of potential UE manipulation in AI-based beam management (BM) tests was discussed. It was observed that since both the predicted beam and the ground truth are reported by the UE to the test equipment, there is a risk of the UE aligning these values to pass the test.

- Proposal 4: Safeguards should be implemented to prevent UEs from manipulating prediction results to align with ground truth reports, ensuring reliable test outcomes.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: Current FR2 MIMO OTA test system is not capable to simulate the large number beams (8-16 for set B and 64-128 for set A) for AIML enabled beam management testing

Observation 2: To achieve an accurate L1-RSRP prediction for Set A, sufficient randomness and variation in the time and spatial domains of Set B beams is required.

Observation 3: Enabling UE rotation could help reduce the number of probes required for AI/ML-enabled beam management testing

- Observation 4: CDL models provide the multi-directional AoA and spatial characteristics needed for accurate conformance testing, which are absent in TDL models, limiting TDL’s applicability in real-world assessments.

- Observation 5: The CDL model more closely aligns with the multi-beam gNodeB deployment scenarios expected in FR2, providing a relevant testing environment for AI/ML-based beam management algorithms.

- Observation 6: The standard CDL model requires a complex setup with multiple probes, increasing test system cost and complexity.

Proposal 1: A multi-probe test setup with diverse beam patterns and AoAs, along with sufficient randomness and spatial variation in Set B beams, is required to enable reliable prediction of Set A beams

Proposal 2: RAN4 should limit the discussion to single TRP cases for Rel-19 AI-BM to establish a foundational assumption for testability studies.

- Proposal 3: To ensure both testing feasibility and conformance accuracy, it is recommended to adopt the Simplified CDL model as the baseline for AI/ML BM testing in FR2, as it retains essential spatial features and requires fewer resources than the full CDL model.

- Proposal 4: Safeguards should be implemented to prevent UEs from manipulating prediction results to align with ground truth reports, ensuring reliable test outcomes.

- References

- R4- R4-2417210, WF on requirements for AI/ML air interface, Qualcomm Incorporated, Hefei, Anhui, China, 14th – 18th October, 2024

- R4-2417188, AI/ML ad-hoc meeting minutes, Qualcomm Incorporated, Hefei, Anhui, China, 14th – 18th October, 2024

- R4-2415231, Topic summary for [112-Bis][127] NR_AIML_air, Qualcomm.