3GPP TSG-RAN WG4 Meeting #113	R4-2417785

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.17.1

Source:	CAICT, Qualcomm, Ericsson

Title:	Proposed update for TR 38.843 with RAN4 part

Document for:	Discussion

1	Introduction

In this contribution, we provide update for TR 38.843 with RAN4 part with the agreements till RAN4#112bis.

2	Text Proposal

------------------------------------------- Change --------------------------------------------------------

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

…….

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.
NOTE: the term is not applicable for performance/conformance testing.

……….

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.
NOTE: the term is not applicable for performance/conformance testing.

Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

……..

Reference decoder: A decoder used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE.

Reference encoder: An encoder used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification.

……

7.4	Interoperability and testability aspects

7.4.1	Introduction

In this section, the study of requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements, if applicable, are documented.

The need and implications for AI/ML processing capabilities definition is considered.

7.4.2	Common framework

7.4.2.1 General

The general requirements and testing frameworks for AI/ML based performance enhancements mainly focus on

- how to define requirements and tests for inference

- evaluate feasibility and necessity of requirements/tests for LCM

- requirements for data collection (in particular for training) could/need be defined

Requirements/tests for training will not be studied unless training procedures are defined. The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field.

The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

- FFS on whether and how many different test configurations/parameters are used for tests

- LCM would be tested

7.4.2.2 Principles on the definition of requirements

For the definition of AI/ML requirements, the following cases related to legacy performance should be considered

- For the cases with the existing legacy performance

  - Take the legacy performance as baseline for existing use cases/procedures/functionalities /measurements that are to be enhanced by AI/ML based methods

    - Further study may be needed on what is baseline performance in conditions different to the requirement condition but within the expected range of operation.

  - New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods

- For the cases without the existing legacy performance

  - New performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are carried out or are to be enhanced by AI/ML based methods

The following procedure can be considered for defining core requirements

- Performance monitoring procedure, including performance evaluation and decision-making procedure for AI/ML functionalities/models

- Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update

- Latency/interruption requirement for above procedures

Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency.

The following LCM related requirements can be considered:

- Model/Functionality select/switch/activate/deactivate/fallback

- Model/Functionality monitoring

- On whether requirements for data collection (in particular for training) could/need be defined:

  - Data collection requirements would only be defined if data collection procedure is defined in 3GPP specifications.

- On requirements for model transfer/update:

  - Requirements would only be defined if model transfer/update would be defined in 3GPP specifications.

UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode. The legacy framework for RRC/MAC-CE/DCI based core requirements (e.g., define delay requirements based on multiple delay components) can be used as the baseline for LCM procedures if the LCM related requirements are agreed to be introduced. If new procedures which legacy framework is not applicable to are introduced, additional core requirement framework can be discussed.

LCM related tests should consider how the framework can address the possibility of updates/activation/deactivation /switching to the functionalities/models after the deployment of the devices in the field.

Both static and non-static scenarios/configurations could be needed for AI testing.

- Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic

- Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

How to use them, including whether to use static scenarios/configurations as baseline need to be discussed and decided case by case.

7.4.2.3 Reference block diagrams for testing

Reference block diagrams provide test modules/functionalities of TE/DUT and testing framework for different use cases. Both reference block diagrams for 1-sided model and 2-sided model are studied.

7.4.2.3.1 Reference block diagram for 1-sided model

Figure 7.4.2.3.1-1 provides the reference block diagram for 1-sided model. LCM in the figure includes functionality and/or model ID based LCM. The link between TE and DUT are physical and not logical. The logical link will depend on the functionality being tested. The scope of the figure includes both performance and potentially LCM tests. Offline training is assumed and some blocks may not be used in some of the tests. LCM may not be tested depending on the purpose of the test.

LCM

Verification

AI/ML functions

Test configuration/controller

Signal generator

inference

LCM

DUT

TE

Figure 7.2.3.1-1 Reference block diagram for 1-sided model

7.4.2.3.2 Reference block diagram for 2-sided model

Figure 7.4.2.3.1-2 provides the reference block diagram for 2-sided model. LCM in the figure includes functionality and/or model ID based LCM. The link between TE and DUT are physical and not logical. The logical link will depend on the functionality being tested. The scope of the figure includes both performance and potentially LCM tests. Offline training is assumed and some blocks may not be used in some of the tests. LCM may not be tested depending on the purpose of the test.

LCM

Verification

AI/ML functions

Test configuration/controller

Signal generator

inference

LCM

DUT

TE

inference

Figure 7.2.3.1-2 Reference block diagram for 2-sided model

7.4.2.4 Test encoder/decoder for 2-sided model

(NOTE: At the current stage the framework in this session applies to CSI compression case.)

In order to determine the test encoder/decoder, the following issues are considered:

- Common assumptions for proposals of the test decoder / encoder (and the paired encoder/ decoder) for tester

- The need for and potential definition and derivation procedure of intermediate KPI for decoder evaluation and selection

- Data collection/generation for decoder evaluation, and the common assumptions/environment needed for data collection/generation

- How to minimize the impact of possible variations/differences in the test decoder/ test encoder design/implementation on UE/ gNB performance verification

- The impact of test decoder/ encoder for testing complexity to UE/gNB performance verification, and the advantage/disadvantage analysis of high/low complexity decoders.

The test decoder/encoder design should take into account complexity limitations based on e.g., feasibility of TE implementation and complexity levels considered feasible by network vendors/UE vendors for decoder/encoder deployment.

The choice of test decoder/encoder should aim as much as possible to avoid limiting the implementation choices, including e.g. complexity, back-bone model etc, of UE/gNB encoders/decoders operating in the field (this principle may not be fully achievable in practice).

Specification on the test may include some high-level parameters for the test decoder/encoder (e.g. parameters related to processing complexity, model structure, etc).

Following the above principles, the considered options of test decoder are listed below

- Option 1: DUT provides the decoder

- Option 2: Infra vendor provides the decoder

- Option 3: Full decoder specification in standard

- Option 4: TE vendor provides the decoder

Option 3 target is that a single decoder defined in the specifications for at least a single test for any DUTs.

For option 4, the following aspects should be considered

- TE vendor should be able to develop the decoder based on the specifications

- Test repeatability should be ensured (variation among TE vendor implementations should be bound)

- Other vendors should also be able to develop such a decoder and which can deliver similar performance

- Interoperability should be ensured based on the parameters that need to be specified

  - Parameters that need to be specified are FFS

- Candidate parameters/conditions that may be considered for defining test decoder include

  - Training data set for TE decoder training

  - Model structure (Activation function is included in the model structure)

  - Performance parameters for the TE decoder (e.g. cosine similarity, loss function, etc)

  - Maximum FLOPs allowed for the test decoder

  - Maximum number/size of model parameters

  - Compression ratio of decoder (output size/input size)

  - Quantization level

  - Other parameters are not precluded and to be further discussed.

  - Note: Feasibility of definition of parameters needs further investigated.

Option 4 target is that a single decoder implemented by each TE vendor will be enough for at least a single test for any DUTs. TE vendor should be able to implement the test decoder for Option 4 without any involvement from another party. If this is found infeasible, another option in which TE vendors need to collaborate with DUT/infra vendors to implement the decoder could be considered.

Further clarifications and analysis of the four options of test decoder are included in table 7.4.2.3-1. It is assumed that for Option 4 the TE vendors can implement the decoder just based on the specifications (no other party involved). The table would need to be revised if collaboration between TE vendor and DUT/infra vendor is needed.

Table 7.4.2.4-1 Comparison of the four options of test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | FFS Could be specified depending on how Option 4 will be defined |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type between DUT and decoder provider (source of training data should be consistent with the collaboration type) |  |  |  |  |
| Test decoder performance verification procedure at TE | Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE | - Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE - Need to ensure that decoder performance is good enough to enable a DUT that meets the minimum requirements to pass the test | Not needed as long as the standardized model implementation can be similar enough between TE vendors | Not needed as long as the model implementation can be similar enough between TE vendors |
| Feasibility of test decoder verification procedure | FFS | FFS | FFS | FFS |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (likelihood that test decoder would be used in actual field deployments ) |  |  |  |  |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE Lower than Option 3/4 in terms of that no training at TE is required | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE Lower than Option 3/4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Lower than Option 4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Higher than Option 3 in terms of that training at TE is required Note: How to ensure compatibility/interoperability between TE and DUT needs further study. |
| Specification Effort (defining test decoder and requirements) | Low | Low | Highest RAN4 needs to standardize the entire decoder | High RAN4 needs study and decide on what to standardize |
| Confidentiality/ IP issues in the testing procedure(after specs are published) |  |  | No | No |
| Applicability to different scenarios/conditions/ configurations |  |  |  |  |
| Complexity of testing for the ecosystem | Testing the encoder at DUT Higher than Option 3/4 Need for interaction between TE vendor | Testing the encoder at DUT Higher than Option 3/4 Testing complexity higher also than option 1. | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties |
| Complexity of verifying/testing the test decoder | Higher than option 3/4 FFS compared to option 2 | Higher than Option 3/4 FFS compared to Option 1 | Low | Low |
| Complexity of deploying for the ecosystem |  |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented |  |  |  |  |
| Relationship with reference decoder/encoder(used by RAN4 to define the performance requirements) for defining requirement |  |  |  |  |
| Whether model transfer/delivery is needed during the test procedure |  |  |  |  |

The feasibility of Option 3 and 4 are studied. Table 7.4.2.4-2 provides the agreed parameters for verifying the feasibility of aligning model among companies and additional parameters of the encoder/decoder and feedback are given in Table 7.4.2.4-3.

Table 7.4.2.4-2 System-level simulation parameters for verifying the feasibility of aligning model

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Table 7.4.2.4-3 additional parameters of the encoder/decoder and feedback

| Assumptions |  | Value |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | MLP, CNN, Transformer |
|  | Pre-processing | SVD to get channel eigenvectors |
| CSI reconstruction part | AI/ML model backbone | [MLP, CNN, Transformer] |
| Common description | Input type | Eigenvectors of channel matrix |
|  | Output type | Eigenvectors of channel matrix |
|  | Quantization /dequantization method | Scalar quantization |
| Dataset description |
|  | Train/Test | 10% of the training dataset as baseline |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Floating point (float 32) |
| Other parameters | Laten/reporting size | [X: below 80bits, Y: 100 – 140 bits] |

The basic step for the feasibility study of Option 3 is as follows:

- Step 1: Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Step 2: Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Step 3: Select one or more decoder for further analysis

  - selection criteria to be discussed

- Step 4: Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Step 5: Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

It should be noted that parameters agreed are just for the feasibility study of testing options and If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

Two options are listed for test decoder derivation procedure:

- Option 1: companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

- Option 2: companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

Option 1 is agreed to work on. MLP and CNN are chosen for further evaluation.

7.4.2.5 Data collection/generation for testing

Different generating methods of test dataset can be used for different tests. The following candidate methods are to be considered:

- Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

  - “Legacy approach” refers legacy test in which a channel model is used

- Field dataset (data collected directly from field measurements)

- TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data)

- Other methods are not precluded

Synthetic channels are used as baseline, and check whether it can be used for the individual use case. An analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data may be conducted and the field data can be considered for the analysis.

7.4.2.6 Data collection/generation for training

Some conditions and/or accuracy requirements for the training dataset or training data generation could only be introduced if the training procedure is defined in 3GPP specifications.

7.4.2.7 Generalization/scalability aspects

The necessity and feasibility of defining requirements or test to verify the generalization of AI/ML is studied.

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing:

- details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality

- what the minimum level performance for each identified scenario and/or configuration is

- what the significant degradation for other scenarios and/or configurations is

It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition.

Generalization related requirements/tests for each AI feature should be discussed and decided in the case-by-case manner. As for the handling of generalization tests, the following option is considered as baseline:

Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test)

- Specified UE configuration includes functionality and/or model ID if defined

7.4.2.8 AI/ML processing capability

The practical processing capability and implementation complexity for device under test should be assumed when specifying RAN4 requirements.

- The UE capability may be needed to handle different complexity for one side and two-side models.

- The complexity of UE should also be studied when making assumption on gNB side model, and vice versa.

7.4.2.9 Post deployment handling

When operating in the field, two aspects of UE operation may impact performance:

- Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

- Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive:

- Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs

  - Validation takes into account the UE hardware in which the model is to be deployed/activated.

  - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

  - FFS whether validation can be done at the device along with inference for another active functionality(ies)

  - FFS on the feasibility

  - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

  - Other aspects not precluded.

- Option 2: Using performance monitoring and LCM procedures

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

7.4.3	CSI feedback enhancement

Both time domain CSI prediction and spatial-frequency domain CSI compression are considered.

PMI reporting framework (follow PMI vs. random PMI test, use of γ as criteria, etc.) is taken as starting point for CSI related tests. Other metrics/framework is not precluded.

For metrics for CSI requirements/tests, the following test metrics are identified:

- Option 1: Throughput/relative throughput

- Option 2: SGCS, NMSE

- Option 3: CSI prediction accuracy

Option 1 should be used as baseline. For option 3, further discuss is needed on the feasibility to define the CSI prediction accuracy in WI. For metrics for CSI monitoring, further discussion is needed in WI.

For CSI prediction, relative throughput is used as requirement metric. AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

- FFS which legacy requirement/feature are relevant

- FFS whether AI/ML based performance should be better than the legacy

7.4.4	Beam management

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

- Option 1: RSRP accuracy

- Option 2: Beam prediction accuracy

  - Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

  - Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

  - Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

  - Related measurement accuracy can be considered to determine x

- Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition. In addition, measurements period, TCI states and other requirements if needed should be defined

- Measurement period for prediction (how long will set B be measured before making a prediction)

  - FFS on whether the legacy measurement period can be reused or not.

- TCI state known/unknown

The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam. The ground truth in RAN4 is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam and the following aspects should be considered

- SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP

- impact of multiple-AoA test setup

- the channel condition

- whether the ground truth will be changing or not

If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Vendors may take into account the test environment/conditions defined by RAN4 when training the UE. Sufficient test environment/conditions should be defined to enable vendors to create the data needed for training. It is strived to make the test conditions similar to field deployment conditions. Augment training data to avoid overfitting of UE models to the test environment could be considered.

For the test setup, issues listed below are investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

  - number of beams transmitted simultaneously

  - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

The maximum number of set B Tx beams that test system should be able to emulate: [8-16]. The maximum number of set A Tx beams that test system should be able to emulate: [64-128]. UE rotation/repositioning between different tests should be supported.

For Channel model, the following options considering TE feasibility is selected for down-select

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For CDL or simplified CDL channel model, the relationship between AoD range [and granularity], number of beams and number of probes is studied

- Set A and B should be generated in the chamber for ground-truth extraction.

[TBD]

For TDL channel model, the relationship between [dynamic power range], number of beams and number of probes is studied.

- Set A and B should be generated in the chamber for ground-truth extraction.

  - For single AoA, FSS how to derive Set A

[TBD]

RAN4 study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy. LLS is performed to evaluate the RSRP prediction accuracy and other KPIs with UE Model trained using ideal measurements.

[TBD]

7.4.5	Positioning accuracy enhancements

Both direct AI/ML positioning and AI/ML assisted positioning are considered.

For metrics for positioning requirements/tests, the candidate options include

- Option 1: positioning accuracy: Ground truth vs. reported

  - only option available for direct positioning

- Option 2: CIR/PDP, channel estimation accuracy

- Option 3: ToA, RSTD and RSRP, and RSRPP

- Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

The feasibility and testability of different options should be further justified in WI.

For case 1, [TBD]

For case 2a, no positioning accuracy requirements will be defined because positioning is LMF based. Existing requirements for existing metrics will be taken as baseline. How to define requirement for any new reported metrics needs further study.

For case 3a/3b, no positioning accuracy requirements will be defined. Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.