# 3GPP TSG-RAN WG4 Meeting #113	                                R4-2417786

Orlando, US, 18th – 22nd November, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on SBFD General aspects

Agenda Item:       7.20.1

Document for:     Discussion

# Introduction

This contribution presents our analysis of general issues related to SBFD, including DL/UL subband configurations and transmission bandwidth configuration for SBFD.

# Discussion

## DL/UL subband configurations support for SBFD

In previous RAN4 discussion, specifically, in RAN4#112bis, the following agreements are achieved [1]:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

In RAN4, it was decided that only DUD and DU patterns are permitted for sub-band configurations. The DU configuration has a single guard band between the DL and UL subbands, while the DUD configuration includes two guard bands. The total bandwidth occupied by the guard bands reduces the available transmission bandwidth, and this reduction percentage should be considered when determining the minimum channel bandwidth for SBFD operating bands.

In our view the {D, U, D} case, where the UL subband should be placed in the middle of channel bandwidth with {40MHz,20MHz,40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI. Since for the SBFD symbol represented by {D, U, D}, Adjacent channel gNB-gNB CLI is seen in the UL sub-band of the SBFD symbol due to DL slots of legacy TDD from neighbouring gNBs overlapping with SBFD symbols. Assuming a channel BW of 100 MHz with [D, U, D] configuration the bandwidth will be 98.28 MHz with 273 RBs and assuming a guard band of 3.6 MHz (1.8*2) there will be five RBs will be required. There will be approximately 3.7% Transmission bandwidth reduction due to SBFD inter-subband guard band will be there. Hence an optimum value ~ 3 to 4% can be considered. Moreover, when defining the RF requirements for UL/DL subband configurations, it is feasible to adopt a configuration-agnostic approach. In this approach, the RF requirements are specified independently of the subband configuration. The supported configurations can be declared without limiting SBFD deployment to a specific set of configurations.

Proposal 1: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested, and no restrictions should apply to other FR1 BS classes or FR2 BS.

Proposal 2: The UL subband size relative to the channel bandwidth can range from 20% to 40%.

## Transmission bandwidth configuration for SBFD

In previous RAN4 discussion, specifically, in RAN4#112bis, the following agreements are achieved [1]:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

As shown in [2], the transmission bandwidth configuration for the SBFD DUD pattern in a single NR channel is outlined. We will now share our interpretation based on the illustrative figures provided by Ericsson. Since the 30 KHz SCS is used in FR1 to balance coverage and throughput, and SBFD is supported in TDD operation, the 30 KHz SCS can be employed based on UE requirements. Additionally, using 15 KHz SCS in the lower band to achieve wider channel bandwidth would be challenging and unlikely to provide significant benefits. Instead, it would introduce unnecessary complexity without substantial advantages.

Moreover, in the case of guardbands, macro base stations require a wider guardband size and necessitate both analog and digital filtering with high linearity on the transmitter and receiver sides. In contrast, micro base stations do not have such stringent requirements, and a narrower guardband can be used. Therefore, the guardband size will vary depending on the type of base station. As a result, the SBFD RBs for all classes of base stations need to be defined separately.

Observation 1: the 30 kHz SCS is used in FR1 to balance coverage and throughput, and SBFD is supported in TDD operation, allowing it to be employed based on UE requirements.

Observation 2: The guardband size for SBFD will vary depending on the type of base station

Proposal 3: Define the SBFD RBs requirement for all classes of base stations.

# Conclusion

In this contribution, we discuss about SBFD general aspects with the following proposals and observations.

Proposal 1: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested, and no restrictions should apply to other FR1 BS classes or FR2 BS.

Proposal 2: The UL subband size relative to the channel bandwidth can range from 20% to 40%.

Observation 1: the 30 kHz SCS is used in FR1 to balance coverage and throughput, and SBFD is supported in TDD operation, allowing it to be employed based on UE requirements.

Observation 2: The guardband size for SBFD will vary depending on the type of base station

Proposal 3: Define the SBFD RBs requirement for all classes of base stations.

# Reference

- R4-2416584: Way forward for [112bis][306] NR_duplex_evo_General, Samsung.

- R4-2416134: SBFD general aspects, Ericsson