# 3GPP TSG-RAN WG4 Meeting #113		R4-2417796

Orlando, US, 18th – 22nd November, 2024

Source:	China Telecom

Title:	Views on 6 layer support evaluation for 6Rx UE

Agenda Item:	7.1.1.3.2

Document for:	discussion

Introduction

In RAN4#112bis meeting, issues on 6 layer support evaluation was discussed and as an outcome, the WF is approved in [1].

In this contribution, our views on the open issues are given.

Discussion

6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following. - Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx.

Among some companies’ simulation results provided in the last meeting [2-5], performance gain of 5/6 layer over 4 layer PDSCH transmission can be observed especially under high SNR conditions. As operator, we would be supportive of introducing maximum 6 layer PDSCH transmission as an optional new feature for Rel-19.

We also observe some companies’ results show negligible performance gain, and we are fine to continue discussion on the link level simulation evaluation assumptions.

As per the WF, we notice that there are different options for the BS correlation matrix and new matrices are requested from BS vendor:

Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113

Since this feasibility study is focused on PDSCH performance, we do not see the reason to introduce additional BS correlation other than 3GPP defined ones, which seems unfair for 6Rx UEs considering we have already introduced 8Rx UE with max 8 layers.

Proposal 1: Introduce 6 MIMO layers support as an optional feature.

Proposal 2: For the feasibility study, do not introduce other BS correlation matrix other than 3GPP specified ones.

Conclusion

Proposal 1: Introduce 6 MIMO layers support as an optional feature.

Proposal 2: For the feasibility study, do not introduce other BS correlation matrix other than 3GPP specified ones.

Reference

R4-2417095 WF on 6Rx UE requirements, AT&T, TSG-RAN WG4 Meeting #112bis

R4-2414985 R19 Simulation results of 6Layer performance, OPPO, TSG-RAN WG4 Meeting #112bis

R4-2415647 Discussion on the demodulation performance requirements for 6Rx devices, Qualcomm. TSG-RAN WG4 Meeting #112bis

R4- 2415991 Discussion on the MIMO layers for 6Rx UE, Ericsson, TSG-RAN WG4 Meeting #112bis

R4-2416342 Discussion on MIMO layer evaluation for 6Rx UE ZTE Corporation, Sanechips TSG-RAN WG4 Meeting #112bis