3GPP TSG-RAN WG4 Meeting #113	R4-2417805

Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	7.17.3.1

Source:	MediaTek Inc.

Title:	Discussion on CSI prediction testability and interoperability

Document for:	Discussion

# Introduction

In RAN4#112 and RAN4#112-bis, RAN4 agreed to continue discussion on CSI prediction testability and interoperability issues [1][2]. In this contribution, we provide our views on CSI prediction testability and interoperability issues.

# Discussion

In this Chapter, we provide our views on CSI prediction testability and interoperability issues.

Issue 4-5: CSI prediction accuracy metric

Agreement:

Use relative throughput as requirement metric for CSI prediction.

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

CSI prediction accuracy should be tested with PMI reporting test. Test metric of these tests has always been relative throughput compared to random PMI of Type I codebook. We see that using the same approach to test CSI prediction accuracy is sufficient.

Observation #1: Existing PMI reporting tests use random PMI of Type I codebook as reference throughput.

Proposal #1: We propose to use random PMI of Type I codebook as reference throughput in test metric for CSI prediction.

Issue 1-4: AI/ML Performance

Agreement:

- For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

  - FFS which legacy requirement/feature are relevant

  - FFS whether AI/ML based performance should be better than the legacy

In Release 18 we introduced new PMI requirements for eType II doppler codebook MIMO evolution work item that is CSI prediction without AI/ML. During this work RAN4 observed great difficulties finding suitable test configuration that would enable significantly measurable PMI prediction gains compared to legacy non-predicting eType II reporting. Therefore, we propose to use legacy non-predicting eType II codebook as reference when designing any new PMI reporting tests to confirm test configuration enables any measurable prediction gains.

Observation #2: When defining Release 18 PMI prediction test requirements, it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

For eType II doppler codebook in TS38.214 Clause 5.2.2.2.10 (Enhanced Type II codebook for predicted PMI) [3] there are four Rel-18 requirements defined in TS38.101-4 in Clauses 6.3.2.1.8, 6.3.3.1.8, 6.3.2.2.9, and 6.3.3.2.8 [4]. The configurations of these tests can be considered as starting point. However, Rel-18 WI run out of time to fully optimize test configurations and there could be room to improve configuration further to enable more PMI prediction gains. We suggest to further check test configuration in Rel-19 work.

Observation #3: There are four existing PMI requirements for PMI prediction that can be used as starting point for Rel-19 work.

Observation #4: There is still room to improve existing test configuration to enable even better prediction gain.

Proposal #3: We propose to use existing eType II doppler codebook test configurations as a starting point for Rel-19 work.

Proposal #4: We propose to further study if existing eType II doppler codebook test configurations can be enhanced for better prediction opportunities.

# Conclusion

In this contribution we provide the view on the CSI prediction testability and interoperability issues. The following observations and proposals are made:

Observation #1: Existing PMI reporting tests use random PMI of Type I codebook as reference throughput.

Proposal #1: Use random PMI of Type I codebook as reference throughput in test metric for CSI prediction.

Observation #2: When defining Release 18 PMI prediction test requirements it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

Observation #3: There are four existing PMI requirements for PMI prediction that can be used as starting point for Rel-19 work.

Observation #4: There is still room to improve existing test configuration to enable even better prediction gain.

Proposal #3: We propose to use existing eType II doppler codebook test configurations as a starting point for Rel-19 work.

Proposal #4: We propose to further study if existing eType II doppler codebook test configurations can be enhanced for better prediction opportunities.

# Reference

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm inc.

- R4-2414323, WF on AI/ML, Qualcomm inc.

- 3GPP TS 38.214 V18.4.0 (2024-09), Physical layer procedures for data (Release 18)

- 3GPP TS 38.101-4 V18.5.0 (2024-09), User Equipment (UE) radio transmission and reception;
Part 4: Performance requirements (Release 18)