3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417827

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on spatial channel modelling methodology for demodulation performance requirements

Agenda item:	7.14.2

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF[1] for [112][327] FS_NR_demod_SCM was agreed and remaining issues on spatial channel modelling methodology needs further discussion.

In this contribution, we provide our views on spatial channel modelling methodology for demodulation.

- Discussion

2.1 TDL Based Methodologies

TDL Model Extensions

As per the WF [2], the issue concerning TDL model extensions is shown as below:

TDL Model Extensions Way Forward: Open issue needs further discussion, 2 contributions provided during RAN4 #112, interested companies to review and provide views at next meeting.

Referring to sub-clause 7.7.4 of TR 38.901, the TDL models can be generated from CDL models by spatial filtering with assuming antenna pattern at both Tx and Rx. The assuming antenna pattern can be ideal isotropic antenna, non-isotropic antennas or array antennas. In sub-clause 7.7.4.1 of TR 38.901, the isotropic antenna pattern, rectangular mask pattern, and simplified antenna pattern give in ITU-R M.2135 are defined. In sub-clause 7.7.4.2 of TR 38.901, the generation procedure for TDL generated from CDL are defined, From equation 7.7-4 of TR 38.901 as following, the TDL cluster powerfor cluster n can be calculated with CDL cluster power for cluster n, assuming rx antenna pattern ,and assuming tx antenna pattern .

From our understanding, the assuming rx antenna pattern ,and assuming tx antenna pattern are single antenna element pattern.

The TDL models described in Clause 7.7.2 of TR 38.901 are generated from the CDL models assuming ideal isotropic antennas at both Tx and Rx. From Table 7.3-1 of TR 38.901, the element response pattern is normalized 3D radiation power pattern, which is calculated from vertical cut of the radiation power pattern and horizontal cut of the radiation power pattern.

Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) |  |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

So, for TDL model extensions, it is reasonable to define the radiation power pattern of a single antenna element for spatial filtering.

Observation 1: Define the radiation power pattern of a single antenna element for spatial filtering for TDL model extensions.

For TDL model extensions for MIMO simulations, the correlation matrix is needed, and correlation matrix is dependent on array pattern. The sub-clause 7.7.5.2 of TR 38.901 define how to apply a correlation matrix to TDL model extensions as following:

The TDLs and the spatial-filtered TDLs can be used with the correlation matrices for MIMO link-level simulations. Typical correlation parameters can be derived from 1) delay & angular scaled CDLs with antenna array assumptions, 2) system-level model with antenna array assumptions, or 3) by selecting extreme cases, e.g. uncorrelated, highly correlated etc. For example, these following options can be considered: 1) Zero correlation (IID channel coefficients) can be used for any number of antenna elements 2) The correlation matrix construction method from TS36.101/104 [15][16] can be used for linear and planar (single- or dual-polarized) arrays. - Other correlation parameters α, β, γ than those specified in TS36.101/104 [15][16] and extensions to larger antenna arrays can be considered. For typical scenarios, α and β will be in the range 0-1 - A representative set of values is {0,0.7,0.9,0.99} Note: This approach can be applied to TDLs derived from spatially filtered CDLs to emulate hybrid BF system Note: Other methodologies could also be developed, e.g. - extending the TS36.101/104 [15][16] procedure to planar arrays or more elements - using CDLs in combination with array assumptions to derive per-tap correlation matrices as in [17].

So, it is reasonable to use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

Observation 2: Use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

2.2 CDL Based Methodologies

Radiation pattern

As per the WF [1], the issue concerning radiation pattern is shown as below:

Radiation Pattern Proposals: - Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern - Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1

From our understanding, generally, antenna element pattern includes isotropic pattern, omnidirectional pattern, and directional pattern. Referring to radiation pattern parameters included in TR 38.827 Section 7.2 in Option 1, it is same with that in 38.901 Table 7.3-1 in Option 2. The TDL models described in Clause 7.7.2 of TR 38.901 are generated from the CDL models assuming ideal isotropic antennas at both Tx and Rx. So for the purposes of CDL alignment, the isotropic pattern can be considered for aligning the parameters other than antenna element pattern.

Observation 3: For the purposes of CDL alignment, the isotropic pattern can be considered for aligning the parameters other than antenna element pattern.

Frequency Bands

As per the WF [1], the issue concerning frequency bands is shown as below:

Frequency Bands Proposals: - Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

In TR 38.901, the basic TDL channel model was defined, and it can be used for all carrier frequencies/bands in given frequency range by scaling of delays. The scaling of delays information for different frequency in different scenario is shown in Table 7.7.3-2 of TR 38.901 copied as below:

Table 7.7.3-2. Scenario specific scaling factors - for information only

| Proposed Scaling Factor in [ns] |  | Frequency [GHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 6 | 15 | 28 | 39 | 60 | 70 |
| Indoor office | Short-delay profile | 20 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Normal-delay profile | 39 | 30 | 24 | 20 | 18 | 16 | 16 |
|  | Long-delay profile | 59 | 53 | 47 | 43 | 41 | 38 | 37 |
| UMi Street-canyon | Short-delay profile | 65 | 45 | 37 | 32 | 30 | 27 | 26 |
|  | Normal-delay profile | 129 | 93 | 76 | 66 | 61 | 55 | 53 |
|  | Long-delay profile | 634 | 316 | 307 | 301 | 297 | 293 | 291 |
| UMa | Short-delay profile | 93 | 93 | 85 | 80 | 78 | 75 | 74 |
|  | Normal-delay profile | 363 | 363 | 302 | 266 | 249 | 228 | 221 |
|  | Long-delay profile | 1148 | 1148 | 955 | 841 | 786 | 720 | 698 |
| RMa & RMa O2I | Short-delay profile | 32 | 32 | N/A | N/A | N/A | N/A | N/A |
|  | Normal-delay profile | 37 | 37 | N/A | N/A | N/A | N/A | N/A |
|  | Long-delay profile | 153 | 153 | N/A | N/A | N/A | N/A | N/A |
| UMi / UMa O2I | Normal-delay profile | 240 |  |  |  |  |  |  |
|  | Long-delay profile | 616 |  |  |  |  |  |  |

From our understanding, like TDL channel model, the basic CDL channel model can be defined and  used for all carrier frequencies/bands in given frequency range by scaling delays  and  angles.

Observation 4: Basic CDL channel model can be defined and used for all carrier frequencies/bands in given frequency range by scaling delays and angles.

Angles of Departure and Angles of Arrival

As per the WF [1], the issue concerning AoD and AoA is shown as below:

Angles of Departure and Angles of Arrival Proposals: - Option 1: Replace the mean angle in the last position of departure and arrival angles generated equation by the desired angle as .

After checking the equation (7.2-2) of TR 38.827, the  equation (7.2-2) of TR 38.827 is typo. So we prefer Option 1 for updating the equation (7.2-2) of TR 38.827.

Proposal 1: Adopt Option 1 for generation of Angles of Arrival.

- Conclusion

This contribution provides analysis on spatial channel modelling methodology for demodulation performance requirements, and the following proposals and observations are concluded as follows:

Observation 1: Define the radiation power pattern of a single antenna element for spatial filtering for TDL model extensions.

Observation 2: Use method in sub-clause 7.7.5.2 of TR 38.901 to generate correlation matrix for TDL model extensions.

Observation 3: For the purposes of CDL alignment, the isotropic pattern can be considered for aligning the parameters other than antenna element pattern.

Observation 4: Basic CDL channel model can be defined and used for all carrier frequencies/bands in given frequency range by scaling delays and angles.

Proposal 1: Adopt Option 1 for generation of Angles of Arrival.

- References

- R4-2416591, Way Forward Minutes for [112bis][318] NR_SCM, Moderator(Nokia),RAN4#112-bis

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Moderator (Nokia), RAN4#112

- RP-241610, Study on spatial channel model for demodulation performance requirements, Nokia, BT Plc, etc., RAN#104