3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417830

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	7.9.5

Document for:	Approval

- Introduction

In RAN4#112bis meeting, a WF [1] on RF core requirements was agreed and remaining issues on SAN RF core requirement in WF require further discussion.

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

SAN type and RF core requirements

As per the WF [1], the remaining issues concerning SAN RF core requirements are shown as below:

- Sub-topic 2-1: General issues for Ku band SAN RF requirements #### Issue 2-1-1: Supported SAN type for NR NTN Ku band - At least the OTA type of requirements will be specified, FFS on whether conducted type of requirements is considered

SAN type is dependent on FR1-NTN vs. FR2-NTN, and SAN RF core requirement is dependent on frequency range and its numerology. The conducted type of requirement is dependent on FR1-NTN vs. FR2-NTN. For OTA type of requirements, the initial applicability considerations for radiated requirements for SAN type 1-O and SAN type 2-O for NTN Ku-band separately are shown in Table 2-1.

Table 2-1 initial applicability considerations for radiated requirements for Ku-band NTN

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

If we reach agreement on FR1-NTN vs. FR2-NTN for Ku-band, the above initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

Proposal 1: The following initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: The following initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

- References

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104