3GPP TSG-RAN WG4 Meeting #113                                                                                     R4-2417850

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on the remaining issues for 2U/2D CA_n5-n8

Source: 	CATT

Agenda item:	6.3.3

Document for:	Approval

- Background

In last RAN4 meeting, WF [1] for 2UL n5-n8 was agreed as following,

- RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2)

  - 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL

  - Send LS to RAN2 to complete necessary signalling designs

- Stop discussion on Option 3 in Rel-19. If option 3 is discussed in Rel-20 or later, new band numbers are considered.

This contribution provides the views on the remaining issues to define the requirements.

- Discussion

2.1 RF requirements

The technical discussion for RF requirements has been finished in R18, there’s no need to reopen the discussion that the requirements can be defined as concluded in TR 38.872. As only non-concurrent n5 DL and n8 UL is supported in R19, so no new MSD requirements compared with n5UL/(n5+n8)DL is needed. Then the RF requirements for 2UL/2DL n5+n8 in R19 is easy that the note for the non-concurrent n5 DL and n8 UL should be added.

Observation 1: For the RF requirements for 2U/2D n5+n8, no new MSD requirements compared with n5UL/(n5+n8)DL is needed. A note to clarify non-concurrent n5 DL and n8 UL should be added for 2U/2D CA.

A draft CR [6] is provided based on this observation.

2.2 Signaling and RAN2 LS

For the signaling design, FDD-FDD interband CA simultaneous RxTx capability is need to be defined. The capability is similar with the existing simultaneousRxTxInterBandCA capability for TDD-TDD and TDD-FDD inter-band CA.

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

For FDD-FDD interband CA simultaneous RxTx capability, it’s more complicated than the above capability that the necessary information is as following,

- This capability is defined per band combination, per band pair.

- Currently only n5+n8 band pair has this issue, the band combination can be 2 or 3 band inter-band CA.

- In the specific band pair (n5+n8), the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

- For FDD-FDD interband CA, usually a band pair A+B can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. If anyone of the cases can’t be supported, UE should report to network.

- This capability is mandatory for the corresponding band combinations defined in TS 38.101-1.

Proposal 1: The above information is sent to RAN2 to request RAN2 to define FDD-FDD interband CA simultaneous RxTx capability.

LS [7] is drafted based on the above understanding.

2.3 Release independent

For the release independent issue, our understanding is that 2U/2D n5-n8 can still be supported from R-15 as other PC3 2U/2D inter-band CA. So no TS 38.307 CR is needed.

Proposal 2: No TS 38.307 CR is needed for R19 2U/2D n5-n8.

- Summary

This contribution provides our analysis and views for the remaining issues for 2U/2D CA_n5-n8. We have the following observations and proposals.

Observation 1: For the RF requirements for 2U/2D n5+n8, no new MSD requirements compared with n5UL/(n5+n8)DL is needed. A note to clarify non-concurrent n5 DL and n8 UL should be added for 2U/2D.

- This capability is defined per band combination, per band pair. Currently only n5+n8 band pair has this issue, the band combination can be 2 or 3 band inter-band CA.

- In the specific band pair (n5+n8), the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

- For FDD-FDD interband CA, usually a band pair A+B can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. If anyone of the cases can’t be supported, UE should report to network.

- This capability is mandatory for the corresponding band combinations defined in TS 38.101-1.

Proposal 1: The above information is sent to RAN2 to request RAN2 to define FDD-FDD interband CA simultaneous RxTx capability.

Proposal 2: No TS 38.307 CR is needed for R19 2U/2D n5-n8.

Draft CR [6] and LS [7] are provided in this meeting.

Reference

[1] R4-2417125, “WF on UL CA_n5-n8 CATT”, China Telecom, Huawei, HiSilicon, RAN4#112b

[2] TR 38.872

[3] R4-2416039, “Discussion on UL CA_n5-n8”, Huawei, HiSilicon

[4] R4-2415877, “Draft CR to 38.101-1: UE RF requirements for uplink CA_n5-n8”, China Telecom, RAN4#112b

[5] R4-2415103, “Discussion on how to handle the RF requirements for 2U/2D CA_n5-n8”, CATT, RAN4#112b

[6] R4-2417849, “Draft CR to TS 38.101-1: introduction of 2U/2D CA_n5-n8”, CATT, China Telecom, Huawei, RAN4#113

[7] R4-2417851, “LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, CATT, RAN4#113