3GPP TSG-RAN WG4 Meeting # 113                                                           R4-2417856

Orlando, US, 18th – 22nd November, 2024

Title: 	                 TP to TR 38.769 for A-IoT BS TX requirements

Source: 	CATT

Agenda item:	7.21.3.1

Document for:	Approval

- Introduction

This contribution provides a TP to TR 38.769 for A-IoT BS TX requirements. The agreements of TX requirements in the WF [2] are included in the TP.

Reference

[1]TR 38.769, V1.0.0

[2] R4-2417088, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112bis

TP for TR 38.769

<Start of Text Proposal>

### 6.7.2	Ambient IoT BS

      - Ambient IoT BS TX requirements

6.7.2.1.1 Base station output power

Base station output power requirement is potentially needed, RAN4 agrees to use current requirements of Micro NR BS type 1-C as the starting point.

6.7.2.1.2 Output power dynamic

Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement. This requirement depends on the physical layer power control design. Before clear agreement on the power control mechanism, it is FFS in SI stage. Physical layer design doesn't have power boosting mechanism, so power boosting requirement is not needed.

6.7.2.1.3 Transmit ON/OFF power

Regarding the transmit on/off requirement, it usually includes of transmitter off power requirement and transmitter transient period requirement, RAN4 agrees to further study. Detail information on the frame structure is needed to make the final decision.

6.7.2.1.4 Transmit signal quality

Regarding the transmitter signal quality, RAN4 agrees that frequency error and modulation quality (EVM) are potentially needed.

TAE is only applicable for BS transmitting from multiple antennas via MIMO, CA, or combination of them.  A-IoT BS doesn’t support these features, so TAE requirement is not needed.

6.7.2.1.5 Unwanted emissions

Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. RAN4 agrees that OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement are potentially needed.

6.7.2.1.6 Transmitter intermodulation

For transmitter intermodulation requirement for A-IoT BS, RAN4 agreed that further study is needed. More analysis for the co-location scenario is needed.

6.7.2.1.7 Summary

RAN4 agreements on A-IoT BS TX RF requirements are summarized in Table 6.7.2-1.

Table 6.7.2-1: RAN4 agreements on A-IoT BS TX RF Requirement

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | TBD |
|  | Total power dynamic range | TBD |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD |
|  | Transmitter transient period | TBD |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NA |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | TBD |

<End of Text Proposal>