3GPP TSG-RAN WG4 Meeting # 113                                                            R4-2417857

Orlando, US, 18th – 22nd November, 2024

Title: 	                 Discussion on A-IoT intermediate UE requirements

Source: 	CATT

Agenda item:	7.21.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for AIoT intermediate UE Tx and Rx requirements according to the WF [1].

- A-IoT intermediate UE requirements analysis

2.1 Tx requirements

Transmit OFF power requirements apply only to TDD operation, for D2T2, the R2D signal uses the FDD UL spectrum, if the CW node is outside the topology and the CW uses the FDD DL spectrum, then the frequency ranges of the R2D signal and CW are different, and this requirement is not needed. If the CW node is inside the topology, CW uses the FDD UL spectrum, and the frequencies of R2D and CW/D2R are also same, this requirement is needed.

A-IoT intermediate UE doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, so ON/OFF time mask is necessary for A-IoT intermediate UE with TDM operation.

Carrier leakage and in-band emissions are UE requirements, these two requirements are not needed for intermediated UE based on the definitions of them.

UE transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or BS receive band as an unwanted interfering signal. When A-IoT system and NR system coexist and operating bands are adjacent, NR UEs and A-IoT intermediated UEs may be close to each other, and transmit intermodulation requirement is needed for intermediated UE.

Table 1: A-IoT intermediate UE potential Tx applicable requirements

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |  |
| --- | --- | --- | --- | --- |
|  |  |  | RAN4#112bis WF | RAN4#113 CATT |
| Output power dynamics | Transmit OFF power |  | TBD | This requirement is needed if frequency bands of R2D and CW/D2R are same. |
|  | Transmit ON/OFF time mask |  | TBD | YES |
| Transmitted signal quality | Transmit modulation quality | Carrier leakage | TBD | NO |
|  |  | In-band emissions | TBD | NO |
|  | Transmit intermodulation |  | TBD | YES |

2.2 Rx requirements

If A-IoT system coexists with NR system, and their operating frequency spectrum are adjacent, narrow-band blocking may be needed.

Spurious response is UE requirement, is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector, and this requirement is also needed  for A-IoT intermediate UE.

Table 2: A-IoT intermediate UE potential Rx requirements

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |  |
| --- | --- | --- |
|  | RAN4#112bis WF | RAN4#113 CATT |
| Narrow band blocking | TBD | YES |
| Spurious response | TBD | YES |

- Summary

This contribution provides our further analysis for AIoT intermediate UE requirements. We have the following proposals.

Proposal 1: For the A-IoT intermediate UE Tx RF requirements, views from our side are provided in Table 1.

Proposal 2: For the A-IoT intermediate UE Rx RF requirements, views from our side are provided in Table 2.

Reference

[1] R4-2417088, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112bis