3GPP TSG RAN WG4 Meeting #113		R4-2417861

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on system parameters for LP-WUS

Agenda Item:	7.23.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, discussions continued on system parameters for LP-WUS and the following outlines the agreements [1]:

Issue 1-2-1: Channel raster for LP-WUR Agreements: - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not. Issue 1-2-2: considerations on LP-WUS allocation Agreements: - RAN4 to assume PRB grid alignment between LP-WUS and NR. - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier. Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS Agreements: - Follow RAN1 agreement for number of RBs for LP-WUS with 15KHz SCS, i.e., 11RBs.

As stated in Issue 1-2-1, companies are encouraged to directly bring the specs impact if any on the channel raster for LP-WUR, in this contribution we would like to make further clarification on its necessity to facilitate the discussion in addition to the corresponding draft CRs [4][5].

# 2 Discussion

Fig. 1, Homodyne/zero-IF architecture for LP-WUR [2]

For the sake of simplicity, let’s take one LP-WUR architecture, e.g., homodyne/zero-IF, as the example. For this architecture, LO must generate the carrier frequencies of LP-WUS, hence defining channel raster has the following purposes:

- Indicating the possible positions of LP-WUS in the frequency domain

- Indicating the frequencies generated by LO frequency synthesizer at LP-WUR to be able to receive LP-WUS

The first one could be conveyed from network to LP-WUR in some way, however, if LO frequency synthesizer is required to only generate limited number of frequency points, lower costs of LO frequency synthesizer can be expected.

Observation 1: Even if the centre operation frequency of LP-WUS could be conveyed from network to LP-WUR, the demand for generating a limited number of frequency points by an LP-WUR’s LO frequency synthesizer helps to reduce the costs of LO frequency synthesizer.

Since LP-WUS is used for UE in all RRC states, i.e., IDLE, IN_ACTIVE and CONNECTED, if a UE supporting LP-WUS camps on a cell where there is no MIB or SIB information indicating the centre frequency of LP-WUS of the cell, the UE has to perform scanning for LP-WUS, hence channel raster for LP-WUS is required.

Observation 2: LP-WUS channel raster is required for a UE in IDLE mode with the support of the feature.

From this perspective, it is beneficial to introduce channel raster for LP-WUS.

As RAN4 has agreed that PRB grid alignment between LP-WUS and NR is assumed, and the LP-WUS RBs can be flexibly allocated within the wider NR carrier, the channel raster for LP-WUS embedded in NR bands with 100k channel raster should be revised to 10kHz which is derived in [3], and in NR bands with SCS-based channel raster, the existing NR channel raster is reused.

Proposal 1: RAN4 to introduce channel raster for LP-WUS where for NR bands with 100kHz channel raster, a 10kHz channel raster is defined, and for NR bands with SCS based channel raster, the SCS based channel raster is reused for LP-WUS.

The corresponding companion draftCRs can be found in [4][5].

Proposal 2: RAN4 to agree on the draftCRs in [4][5].

# 3 Conclusion

In this contribution, we have the following observation and proposals for system parameters for LP-WUS:

Observation 1: Even if the centre operation frequency of LP-WUS could be conveyed from network to LP-WUR, the demand for generating a limited number of frequency points by an LP-WUR’s LO frequency synthesizer helps to reduce the costs of LO frequency synthesizer.

Observation 2: LP-WUS channel raster is required for a UE in IDLE mode with the support of the feature.

Proposal 1: RAN4 to introduce channel raster for LP-WUS where for NR bands with 100kHz channel raster, a 10kHz channel raster is defined, and for NR bands with SCS based channel raster, the SCS based channel raster is reused for LP-WUS.

Proposal 2: RAN4 to agree on the draftCRs in [4][5].

# Reference

[1] R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo

[2] 3GPP TR 38.869 v18.0.0

[3] R4-2415130, “Further discussion on system parameters for LP-WUS”, CATT

[4] R4-241xxxx, “draftCR to TS 38.101-1 on system parameters for LP-WUS”, CATT

[5] R4-241xxxx, “draftCR to TS 38.104 on system parameters for LP-WUS”, CATT