3GPP TSG RAN WG4 Meeting #113		R4-2417864

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on REFSENS, ASCS and ACS for LP-WUS

Agenda Item:	7.23.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, discussions continued on REFSENS, ASCS and ACS for LP-WUS, however, very little progress has been made [1].

In this contribution, we present our views on specifying these requirements.

# 2 Discussion

Table-1 describes the current status on REFSENS. Since REFSENS is associated with the choices of target SNR, NF and implementation margin, it was agreed to collect input on REFSENS with values for each element (NF, IM, SNR) and justification on each element.

Table – 1 Status on REFSENS

| REFSENS | Target SNR | NF | IM |
| --- | --- | --- | --- |
| Status | To collect (SNR, NF, IM) inputs to define REFSENS |  |  |
|  | FFS: single value for both OOK-based and OFDM-based receiver |  |  |

If sufficient inputs are obtained, there should be a way of consolidating these inputs into one single value as requirements, e.g., averaging all input REFSENS, or introducing some validating rules to exclude some values, and averaging validated values as the final value for REFSENS.

Proposal 1: In addition to collect inputs of (SNR, NF, IM), RAN4 should also discuss and agree on how to consolidate these inputs into one single value as requirements, and the below two options can be considered as a starting point:

- Option 1: averaging all inputs

- Option 2: introducing some validating rules to exclude some values, and averaging validated values

Regarding single value or different values for OOK-based and OFDM-based receiver, obviously these two types of receivers may have different SNRs (4~5 dB difference observed), NF and IM, it is reasonable to consider two different values. However, the possibility of a similar REFSENS value by summing up their own SNR, NF and IM, hence, RAN4 can focus on collecting and consolidating REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Proposal 2: At this stage RAN4 can focus on collecting and consolidating REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Table – 2 summarizes the current status on ASCS and ACS.

Since the need for defining such requirements is confirmed, the next step is to launch simulation campaigns to check guard RBs and requirement values. With the progress of signal design for LP-WUS in RAN1, RAN4 needs to align and update simulation assumptions at first.

Proposal 3: RAN4 to align and update simulation assumptions for defining ASCS and ACS requirements according to RAN1’s progress on signal design for LP-WUS.

Table – 2 Status on ASCS and ACS

|  | ASCS | ACS |
| --- | --- | --- |
| Need for defining the requirement | Yes, confirmed | Yes, confirmed |
| Guard RBs | TBD | TBD |
| Requirement value | TBD | TBD |
| Test cases | Yes, FFS details | Yes, FFS details |
| Phase noise modelling included? | FFS | Yes |
| Simulation assumptions | To be aligned and updated according to RAN1’s progress | To be aligned and updated according to RAN1’s progress |

And for test cases definition, the need is also confirmed, however, details for test cases are depending on signal designs, and requirements definitions, hence the discussion can be hold on.

Proposal 4: RAN4 hold on discussions on details on test cases for LP-WUS.

# 3 Conclusion

In this contribution, we have the following proposals on REFSENS, ASCS and ACS for LP-WUS:

Proposal 1: In addition to collect inputs of (SNR, NF, IM), RAN4 should also discuss and agree on how to consolidate these inputs into one single value as requirements, and the below two options can be considered as a starting point:

- Option 1: averaging all inputs

- Option 2: introducing some validating rules to exclude some values, and averaging validated values

Proposal 2: At this stage RAN4 can focus on collecting and consolidating REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Proposal 3: RAN4 to align and update simulation assumptions for defining ASCS and ACS requirements according to RAN1’s progress on signal design for LP-WUS.

Proposal 4: RAN4 hold on discussions on details on test cases for LP-WUS.

# Reference

[1] R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo