3GPP TSG RAN WG4 Meeting #113		R4-2417868

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	TP to TR 38.774 on Clause 6 (Simulation assumptions and evaluation)

Agenda Item:	7.23.1

Source: 	CATT

Document for:	Endorsement

# 1 Introduction

According to the work split agreed in RAN4#112bis, we are providing the initial version for Clause 6 of TR 38.774 on simulation assumptions and evaluation.

# 2 Text proposal

< start of text proposal >

# 6	Simulation assumptions and evaluation

## 6.1	Performance metric

<Editor’s note: performance metric to specify RF requirements. >

In order to compare performance of different LP-WUR architectures, missed detection rate (MDR) is used as performance metric, and at the same time, false alarm rate (FAR) is also provided.

## 6.2	Simulation assumption

<Editor’s note: aligned simulation assumptions for evaluation. >

### 6.2.1	General

Both OFDM-based and OOK-based receivers are simulated and considered for specifying corresponding LP-WUS requirements. The detailed simulation assumptions can be found in Section 6.2.2.

### 6.2.2	Simulation parameters

### Table 6.2.2-1 Simulation assumptions

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Phase noise | Not modelled |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not |  |  |

## 6.3	Simulation evaluation results

<Editor’s note: detailed simulations from each company. FFS detailed structure for each RF requirement>

<end of text proposal>