3GPP TSG RAN WG4 Meeting #113		R4-2417883

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.1.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, MPR applicability for FR1 intra-band UL CA was discussed, and the following issues are part of open issues identified [1]:

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - WF - FFS PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet #### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication. - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated. - WF - FFS in next meeting #### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell. - Proposal 2: - For UE supporting contiguous ULCA with the dualPA IE: - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively. - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support: - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. - WF - FFS in next meeting

In this contribution, we present our views on these open issues.

# 2 Discussion

For PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet for intra-band contiguous CA with single activated cell, we could consider to create the following general framework to enable PC1.5 for the intra-band contiguous band combination with single activated cell:

- PC2 for the intra-band contiguous CA is specified for the band combination;

- PC1.5 MPR is specified for the single uplink band in single carrier operation;

- UE can report the MSD degradation against these for the PC3 or PC2 of the band combination;

- Based on its own assessment, network can enable the UE PC1.5 single uplink transmission via explicit commands for the intra-band contiguous band combination.

Proposal 1: RAN4 to consider a general framework to enable PC1.5 for the intra-band contiguous band combination with single activated cell.

For spurious emissions, ACLR, and SEM tables, the aggregated channel bandwidth acts as a ”virtual” larger channel bandwidth. It is reasonable to apply spurious emission, ACLR and SEM for single UL CC transmission for CA bandwidth class B and C based on the aggregated bandwidth.

Proposal 2: Apply aggregated bandwidth as the basis for spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# 3 Conclusion

In this contribution, we have the following proposals on MPR applicability for FR1 intra-band UL CA:

Proposal 1: RAN4 to consider a general framework to enable PC1.5 for the intra-band contiguous band combination with single activated cell.

Proposal 2: Apply aggregated bandwidth as the basis for spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# Reference

[1] R4-2417204, “WF on power domain enhancements”, Huawei