3GPP TSG-RAN WG4 Meeting #113                                           R4-2417886

Orlando, US, 18th -22th Nov, 2024

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS

Agenda Item:	7.23.2.2

Document for:	Approval

Introduction

Based on the last meeting discussion for Rel-19 LP-WUS, the issues to be further studied for REFSNES, ACS and ASCS are captured in WF [1].

Notably, the phase noise profile has been further discussed, and it was agreed in the WF that LLS, along with phase noise modelling, can be evaluated further. This has implications not only for the guard RBs but also for the ACS/ASCS requirements regarding interferer levels and the degradation of the REFSENS level.

This contribution presents our further analysis on REFSENS, ACS, and ASCS. Based on the discussion in [2], two different LP-WUR types should be considered. This contribution primarily focuses on LP-WUR type 1, which targets ultra-low power consumption.

Discussion

REFSENS

As discussed in previous meetings, the legacy approach is used to derive the REFSENS requirement, with the most critical parameters being SNR and NF.

Noise Figure

The targeted coverage for LP-WUS is to achieve the comparable value of PUSCH for Msg3. To fulfill the target, some NF are considered by RAN1 in the evaluation, which could be conceived as a reference. It is noticed that 7dB rather than 9dB was adopted by RAN1 as the basis for MR.

Conclusion in RAN1#116bis: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

In the RAN4 specification, the NF adopted for REFSENS varies across different bands, ranging from 9 dB to 13 dB or even higher. As discussed in [2], the distinction between OFDM-based and OOK-based receivers is ambiguous, as the same receiver can support both types of LP-WUS signals. The critical factor is the design target for power consumption in the LP-WUR, which fundamentally determines the applicable requirements and how we specify them. Therefore, we propose considering two types of LP-WUR that aim for distinct power consumption levels in the receiver. It is important to highlight that ZIF is selected as the baseline architecture for both OOK and OFDM-based LP-WUR. We believe that separate NF values in addition to MR could be considered for the two LP-WUR UE types. Given the significant differences in power consumption assumptions, an additional 8 dB and 2 dB NF compared to MR could be considered for LP-WUR type 1 and type 2 UE, respectively.

Regarding the IM, we prefer to reuse the same value as MR.

Proposal 1: It is proposed to adopt +8 dB and +2 dB as NF values on top of MR for the two different LP-WUR UE types, each aiming for distinct power consumption assumptions.

Proposal 2: IM can use the same value as MR.

SNR

The SNR evaluation in RAN1 is based on TDL model, while the usual model adopted by RAN4 in SNR simulation for purpose of RF requirements is AWGN based. It is foreseen that the OFDM based receiver has better SNR compared to OOK envelope detection receiver, and OOK-4 with chip rate M=4 is worse than other cases. According to RAN1 working assumption, OOK-1 is considered as specific case of OOK-4 with M=1.

Table below presents the simulated SNR value for OOK and OFDM based signals. It can be seen that for both 3rd order or 5th order Butterworth filter and 4-bit or 8-bit ADC, -2dB SNR could be achieved for 1% BLER or MDR.

Table 1: SNR@1%BLER with 8-bit ADC under AWGN

| Modulation | Chip rate | Channel structure | LP-WUS | SNR@1% MDR |
| --- | --- | --- | --- | --- |
| OOK-4 | M = 1 | 8bit | Envelope | -6.1 dB |
|  |  |  | OFDM | -10.5 dB |
|  |  | 16bit | Envelope | -5.5 dB |
|  |  |  | OFDM | -10.1 dB |
|  | M = 2 | 8bit | Envelope | -4.1 dB |
|  |  |  | OFDM | -8.0 dB |
|  |  | 16bit | Envelope | -3.7 dB |
|  |  |  | OFDM | -6.8 dB |
|  | M = 4 | 8bit | Envelope | -2.1 dB |
|  |  |  | OFDM | -4.3 dB |
|  |  | 16bit | Envelope | -2.0 dB |
|  |  |  | OFDM | -4.1 dB |

From the simulation results it can be seen that SNR for OFDM based LP-WUS is better than that of envelop detection, but for sake of simplicity, we may just define single SNR for both LP-WUS signals as we already have different NF assumption for the LP-WUR types.

Proposal 3: It is proposed to define single SNR value with -2dB for both LP-WUR types.

ACS

Phase noise assumption

In the approved WF in last meeting, it agrees that phase noise can be considered in LLS simulation analysis, and Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded.

In SI phase, phase noise as an RF impairment factor has been discussion. The reciprocal mixing impact due to phase noise was reproduced from the SI TR [3].

Figure 2: Reciprocal mixing for ACI case

Due to reciprocal mixing, companies recognize that the allocation of guard RBs is related to phase noise assumptions. As indicated by one of the analyses in the Rel-18 SI, the phase noise of LR is only about 10 dB worse compared to MR, with the difference in integrated phase noise (IPN) being 6 dB.

Figure 3: Reciprocal mixing for ACI case [5]

Since LP-WUR is widely used in Wi-Fi, it is reasonable to adopt a similar phase noise assumption, which is based on several hundred microwatts of power consumption for the LO design. According to Wi-Fi LP-WUR assumptions, the phase noise could be around -116 dBc/Hz at 20 MHz [4]. A comparable phase noise level can also be found in the company input in [6].

Table 1: Phase noise profile [6]

|  | Phase noise (dBc/Hz) |  |  |
| --- | --- | --- | --- |
| Offset | NF 9 dB | NF 12 dB | NF 15 dB |
| @5MHz | -99.4 | -99.4 | -99.4 |
| @10MHz | -108.1 | -105 | -102.1 |
| @15MHz | -120 | -117 | -114 |

After reviewing the phase noise profiles considered widely for Wi-Fi industry, if we want to keep the power consumption of LP-WUR in a reasonable level, we have to revisit the PN model used in LLS simulation. A PN model for LP-WUR based on the formula below is provided in Figure 1.

S(f0)=PSD0n=1N(1+(f0fz,n)αz,n)m=1M(1+(f0fp,m)αp,m)

Figure 4: Phase noise profile

The corresponding phase noise profile parameters can be found in Table 2:

Table 2: Phase profile parameters for LP-WUR

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| PSD0 | 1.225e2 |  |  |
| fz,1 | 100 | αz,1 | 2 |
| fz,2 | 2.28e2 | αz,2 | 1 |
| fz,3 | 1.00e7 | αz,3 | 2 |
| fp,1 | 0.5 | αp,1 | 2.63 |
| fp,2 | 4.26e3 | αp,2 | 2.15 |

The following analysis is based on assumption of MR IPN around -40dBc but with LR IPN around 8dBc.

Observation 1: Phase noise of sufficient low power consumption LO is far worse than that used for MR, which could cause huge impact on the guard RB required for ACS and on the requirements to be derived.

Proposal 4: It is proposed to consider phase noise level similar to that of Wi-Fi as assumption to further evaluate the ACS/ASCS guard RB as well as the possible endurable interferer levels at least for LP-WUR type aimed at ultra-low power consumption.

Guard RBs for ACS

According to the study outcome in Rel-18, RAN4 had the following conclusion for guard RBs for ACS:

Conclusions for RAN4 ACS guard RB analysis:

Based on the analysis above, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ACS is in the range of 1RB ~ 6RBs for 30kHz SCS, or 2RBs ~12RBs for 15kHz SCS. For lower power, i.e., lower order filters, the required guard RBs may increase. There is no recommendation on which filter order should be considered to the baseline in SI phase.

With phase noise modeled, we performed the simulation for two cases of envelop detection LP-WUR:

Case 1: without additional digital domain filtering

Case 2: with additional digital domain filtering

Table 3: Interferer level vs guard RB with phase noise (w/o additional filtering)

| Phase Noise | Interferer (dB) | Guard RB and SNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 4 |
| w/o | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | N/A |
|  | 19.5 | N/A | 1.85 | -0.91 |
|  | 13.5 | -0.87 | -2.01 | -2.01 |
|  | 7.5 | -2.03 | -2.05 | -2.01 |
|  | -INF | -2.04 | -2.06 | -2.02 |
| w/ | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | N/A |
|  | 19.5 | N/A | 3.63 | -0.80 |
|  | 13.5 | -0.84 | -1.61 | -1.82 |
|  | 7.5 | -2.02 | -2.04 | -2.01 |
|  | -INF | -2.04 | -2.05 | -2.01 |

Table 4: Interferer level vs guard RB with phase noise (with additional filtering)

| Phase Noise | Interferer (dB) | Guard RB and SNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 4 |
| w/o | 31.5 | -2.08 | -2.11 | -2.09 |
|  | 26.5 | -2.09 | -2.11 | -2.08 |
|  | 19.5 | -2.10 | -2.11 | -2.08 |
|  | 13.5 | -2.09 | -2.11 | -2.09 |
|  | 7.5 | -2.09 | -2.11 | -2.09 |
|  | -INF | -2.09 | -2.11 | -2.09 |
| w/ | 31.5 | 3.63 | 1.09 | -0.49 |
|  | 26.5 | -1.49 | -1.79 | -2.02 |
|  | 19.5 | -2.07 | -2.07 | -2.07 |
|  | 13.5 | -2.09 | -2.08 | -2.08 |
|  | 7.5 | -2.09 | -2.09 | -2.08 |
|  | -INF | -2.09 | -2.09 | -2.08 |

It has been observed that without digital domain filtering, the LP-WUR is unable to handle large ACS interferers, even in the absence of phase noise. When phase noise is modeled, it clearly becomes a key factor, influencing not only the number of guard RBs required but also the tolerable level of interference.

Based on the simulation results, we can see that with phase noise modeled, even with 4 guard RBs, it is difficult to meet the SNR threshold proposed in section 2.1.2 for large adjacent interferers. While additional digital domain filtering was not considered during the SI studies in both RAN1 and RAN4, it could be left to the LP-WUR implementation but should not be factored into the requirements specification.

Proposal 5: It is proposed to utilize less guard RBs together with the existing guard band for NR CBWs to protect LP-WUS from adjacent NR interreference but with a lowered interferer level for the LP-WUR type aimed at ultra-low power consumption.

Proposal 6: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

ACS requirement

Generally, adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s). The said receiver filter usually is the baseband channel filter. While for LP-WUR, the filter attenuation mainly comes from the analog filter, e.g. 3rd and 5th order Butterworth filters implemented in the Rx chain before ADC, but it does not preclude the possible implementation of digital domain channel filter.

The requirement of ACS comes from the co-existence study, it is derived from ACIR with also consideration of ACLR from the BS side. The assumed BS ACLR is 45dBc for the co-existence study, however, the normal BS implementation for ACLR is better than the assumed value, in addition, the performance metric for LP-WUR ACS is not throughput as considered for the MR, it’s possible to relax the ACS requirement in exchange of smaller guard RB size for LP-WUS.

In current specification ACS is defined as 33dBc for NR at least for 5MHz CBW, by which together with REFSENS degradation, it results the Pinterferer, i.e. REFSENS + 45.5dB. The power imbalance of 31.5dBc is the level considering the ACS value as well as the implementation margin. Apparently, 5th order Butterworth filter cannot provide very high rejection to the adjacent interferer. The evaluated 5th order Butterworth filter can only provide less than 20dB rejection for the adjacent channel with 5MHz frequency offset. Furthermore, if component tolerance is considered for the filter implementation, the suppression capability of this kind of filter could be further degraded. In addition, as pointed out in [7] that with process and temperature variations, the metal–oxide–metal capacitor value in 28-nm CMOS can vary by ± 20%, and poly resistors vary by ±15%. All the factors make the filter rejection much less than the theoretical analysis. The real implementation may just provide ~10dB or even worse suppression for the adjacent interferer, which is much lower than the MR.

Figure 5: Frequency response of Butterworth filter

Supposing 10dB filtering rejection is provided, and still 14dB REFSENS degradation is allowed, with rough estimation, the interferer level LR could handle is about LR REFSENS + 22.5dB or less with IM considered. Alternatively, if keep the same interferer level as MR, the power imbalance between the interferer and LR wanted signal should consider the REFSENS degradation due to the larger NF for LR.

If an 8 dB increase in NF is assumed, this results in a decrease in the power imbalance by 8 dB, bringing it to 23.5 dB. This situation can be interpreted in two ways: either a larger filter rejection is required with fewer guard RBs, or, given a specific filter rejection capability (e.g., 10 dB), more guard RBs would be necessary.

Figure 6: Illustration of ACS requirement

In the current specification, two cases are defined for ACS. Case 2 specifies absolute values rather than relative ones, as in Case 1. The interferer in Case 2 is fixed at -25 dBm, while the desired signal is -56.5 dBm for MR, resulting in a dynamic range of 31.5 dBc. Whether LR can meet this dynamic range depends primarily on two factors: the number of ADC bits and the dynamic range of the AGC.

The ADC's dynamic range can be roughly estimated as 6 dB per bit, meaning a 4-bit ADC provides only 24 dB of dynamic range, while an 8-bit ADC offers 48 dB. If the requirements are based on a 4-bit ADC implementation, it would certainly necessitate reducing the interferer level. Furthermore, due to the low-power consumption design, the dynamic range of the AGC is expected to be relatively small. For example, assuming the AGC has a dynamic range of 30 dB (since higher dynamic range increases power consumption), and if LR's REFSENS is 8 dB higher than MR, this would result in a REFSENS of around -92.7 dBm/5 MHz for 15kHz SCS.

For simplicity, let’s assume the AGC's dynamic range includes the Rx gain, and to prevent the receiver chain from saturating, the LNA can be bypassed. The amplified LR signal must be appropriately set within the ADC’s dynamic range, with REFSENS above the ADC’s noise floor.

With these assumptions, we can perform some basic calculations. Typically, a backoff of around 10 dB from 0 dBFS is considered for an ADC due to the PAPR of the modulated signal, whether it is the desired or interfering signal. This further reduces the effective dynamic range of the ADC. While an 8-bit ADC might just meet the dynamic range requirement for Case 2 with a -25 dBm interferer, if the AGC's dynamic range is reduced, even an 8-bit ADC could fail to meet the required dynamic range.

Ultimately, it’s a tradeoff between power consumption and the LP-WUR's ability to handle large interferers.

The ACS value reflects the adjacent channel rejection capability of the filter and is derived from coexistence studies. However, LR operates under different coexistence scenarios compared to a typical UE, and therefore should not be constrained by the 33 dBc value. In other words, there is no need to maintain the same interferer level for both Case 1 and Case 2 when considering the ACS requirements for LP-WUR. If the dynamic range of LR is expected to be lower than that of MR, the interferer level could be relaxed.

As discussed in section 2.2.1, the primary factor to consider when specifying the ACS/ASCS requirement is the assumed phase noise level at least for LP-WUR type 1 UE. With an anticipated 40–60 dB degradation in phase noise for LR compared to MR, it is expected that LR's interference-handling capability will be compromised.

Observation 2: With worse REFSENS for LR, if keep the same REFSENS degradation level, i.e. 14dB, the ACS value range would be decreased even with the same interferer level as MR.

Observation 3: The filter evaluated by RAN4 in SI stage cannot provide sufficient suppression compared to the level defined in current spec for MR.

Observation 4: Phase noise as a main factor would have significant impact on specifying the ACS requirement based on realistic power consumption assumed for LP-WUR.

With above analysis, we propose to consider the requirement as well as parameters for ACS case 1 and case 2 as follows:

Table 5: Adjacent channel selectivity

| Rx Parameter | Units | 5 MHz |
| --- | --- | --- |
| ACS | dB | 10.0 |

Table 6: Test parameters for Adjacent channel selectivity, Case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + [22] dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. |  |  |

Table 7: Test parameters for Adjacent channel selectivity, case 2

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Power in transmission bandwidth configuration | dBm | -56.5 |
| Pinterferer | dBm | [TBD] |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. |  |  |

The purpose of ACS Case 1 and Case 2 is to assess different aspects of the receiver's performance. Case 1 primarily focuses on evaluating the filtering capability of the receiver, which is demonstrated by the degradation of the desired signal in the presence of a stronger adjacent interferer. Case 2, on the other hand, is designed to assess the receiver's dynamic range, reflected by an increased desired signal level along with a corresponding rise in interferer strength.

However, if power consumption is the key consideration for LP-WUR design, the limiting factor may not be the receiver’s filtering or dynamic range capabilities, but rather its phase noise performance. In this context, even if dynamic range poses no significant challenges for receiver design, verifying this capability may be unnecessary. In other words, since dynamic range is not a critical factor for LP-WUR design, the test case may not effectively address the design's true priorities. Therefore, we believe ACS Case 2 is not needed for LP-WUR.

Proposal 7: It is possible to relax ACS requirement for LP-WUR from co-existence and performance perspective. The proposed ACS is 10dB given the poor filtering capability of LP-WUR.

Proposal 8: It is proposed to only consider case 1 ACS requirement for the LP-WUR type aimed at ultra-low power consumption, while for LP-WUR type aimed at moderate power consumption, both ACS cases could be considered.

Proposal 9: If both cases are considered for LP-WUR type 1 (aimed at ultra-low power consumption), the dynamic range would not be the same as LP-WUR type 2 (aimed at moderate power consumption).

ASCS

Guard RBs for ASCS

According to the study outcome in Rel-18, RAN4 had the following conclusion for guard RBs for ASCS:

Conclusion for RAN4 ASCS guard RB analysis:

Based on the following analysis, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0RB ~ 1RBs for 30KHz SCS, or 0RBs ~2RBs for 15KHz SCS. Similar number of guard RBs could be applied also with lower filter orders as performance was observed to remain similar with 3rd order filter.

From the SI study, it was observed that performance loss is not severe in the presence of an adjacent NR signal with the same PSD as LP-WUS. It is important to note that this conclusion was reached without accounting for the impact of LR phase noise. Given the potential degradation in filter suppression capability, along with the degraded phase noise of the LO in real-world reasonable implementations, incorporating a guard RB is preferable for ASCS. According to our simulations, at least one guard RB should be considered to account for the impact of phase noise.

Proposal 10: It is proposed to adopt 1 RB as the size of guard RB for LP-WUS ASCS regardless of the LP-WUR types.

ASCS requirement

In last RAN4 meeting, we already have the following agreement.

Issue 2-3-1: ASCS requirements value Agreements: - Confirm ASCS RF requirements is needed for LP-WUS/WUR.

Compared to ACS requirement, ASCS primarily reflects the performance demand for LP-WUS in presence with adjacent interferer. In SI evaluation of ASCS Guard RB, the interferer is the NR signal in the channel BW for in-band operation with same PSD. Unlike ACS, the requirement with interferer also could verify the Rx chain linearity, dynamic range as well as phase noise impact, while ASCS may may primarily reflect the filter and phase noise ability, which could be verified by ACS as well. In that sense, if guard RB is specified for ASCS, verifying ASCS requirement may not be necessary. Even if the test cases are specified in RAN4 spec, it may also be further considered whether the tests could be simplified in RAN5.

Conclusion

This contribution provides our further analysis of REFSENS, ACS and ASCS requirements for LP-WUR. With the analysis, we have the following observations and proposals:

REFSNES, NF and SNR:

Proposal 1: It is proposed to adopt +8 dB and +2 dB as NF values on top of MR for the two different LP-WUR UE types, each aiming for distinct power consumption assumptions.

Proposal 2: IM can use the same value as MR.

Proposal 3: It is proposed to define single SNR value with -2dB for both LP-WUR types.

ACS and ASCS guard RBs:

Observation 1: Phase noise of sufficient low power consumption LO is far worse than that used for MR, which could cause huge impact on the guard RB required for ACS and on the requirements to be derived.

Proposal 4: It is proposed to consider phase noise level similar to that of Wi-Fi as assumption to further evaluate the ACS/ASCS guard RB as well as the possible endurable interferer levels at least for LP-WUR type aimed at ultra-low power consumption.

Proposal 5: It is proposed to utilize less guard RBs together with the existing guard band for NR CBWs to protect LP-WUS from adjacent NR interreference but with a lowered interferer level for the LP-WUR type aimed at ultra-low power consumption.

Proposal 6: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Proposal 7: It is proposed to adopt 1 RB as the size of guard RB for LP-WUS ASCS regardless of the LP-WUR types.

ACS requirement:

Observation 2: With worse REFSENS for LR, if keep the same REFSENS degradation level, i.e. 14dB, the ACS value range would be decreased even with the same interferer level as MR.

Observation 3: The filter evaluated by RAN4 in SI stage cannot provide sufficient suppression compared to the level defined in current spec for MR.

Observation 4: Phase noise as a main factor would have significant impact on specifying the ACS requirement based on realistic power consumption assumed for LP-WUR.

Proposal 8: It is possible to relax ACS requirement for LP-WUR from co-existence and performance perspective. The proposed ACS is 10dB given the poor filtering capability of LP-WUR.

Proposal 9: It is proposed to only consider case 1 ACS requirement for the LP-WUR type aimed at ultra-low power consumption, while for LP-WUR type aimed at moderate power consumption, both ACS cases could be considered.

Proposal 10: If both cases are considered for LP-WUR type 1 (aimed at ultra-low power consumption), the dynamic range would not be the same as LP-WUR type 2 (aimed at moderate power consumption).

To facilitate the discussion, draft requirements for REFSENS, maximum input level, ACS and ASCS for LP-WUR are presented in Annex for reference. The requirements are separated for two LP-WUR types and the definitions and discussion of LP-WUR types can be found in [2]. The suffix 'M' is tentatively used here for LP-WUR and may be revised if conflicts arise with the introduction of other features.

- References

[1] R4-2417112, WF on UE RF requirements for LP-WUS, vivo

[2] R4-2417885, Further consideration on general aspects of LP-WUS, Huawei, HiSilicon

[3] TR 38.869, Study on low-power wake-up signal and receiver for NR

[4] Renzhi Liu, et al., An 802.11ba-Based Wake-Up Radio Receiver with Wi-Fi Transceiver Integration, IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 55, NO. 5, pp. 1151-1164, MAY 2020

[5] R4-2320817, LP WUR ACS with phase noise, Murata

[6] R4-2309204, Evaluation of Low power wake-up receiver architectures, Ericsson

[7] A Digital Filtering ADC With Programmable Blocker Cancellation for Wireless Receivers, Qiwei Wang, etc., IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 53, NO. 3, MARCH 2018

- Annex (draft requirements for REFSENS/ACS/ASCS)

Editor’s note: the draft requirements are for information, which are subjected to further revisions according to the standard progress.

- 4.3	Specification suffix information

Unless stated otherwise, the suffixes shown in Table 4.3-1 are used for indicating at 2nd level clause. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| [M] | LP-WUR |

- 7.3M	Reference sensitivity for LP-WUR

- 7.3M.1	General

The reference sensitivity power level REFSENS for LP-WUR is the minimum mean power applied to single UE antenna port, at which the Miss Detection Rate (MDR) shall meet the requirements for the specified reference measurement channel.

Different LP-WUR types are defined based on different power consumption assumptions, each with distinct noise figure (NF) values compared to the MR. The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 7.3M.1-1 below.

Table 7.3M.1-1: The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure (NF) [8] dB higher than the MR. |
| 2 | The LP-WUR is designed for moderate power consumption, assuming a noise figure (NF) [2] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

- 7.3M.2	Reference sensitivity power level for LP-WUR

For a LP-WUR UE equipped with 1 Rx antenna port, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-1 and Table 7.3M.2-2 for LP-WUR type 1 and type 2 UE respectively for the applicable operating bands. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x.

Table 7.3M.2-1: Single antenna port reference sensitivity allowance ΔR1R, LP-WUR type 1

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5 | 7.3 |
| FDD band | 5 | 7.3 |

Editor’s note: TheΔR1R (dB) values in the table account for the 2.5 dB performance difference between 2Rx and 1Rx implementations, the variance of 3.2dB between 5 MHz REFSENS measurements for NR (25 RB) and LP-WUS (11 RB), and the increased noise figure (NF) for LR compared to MR. An additional 8 dB NF increment is assumed for LP-WUR Type 1 UE.

Table 7.3M.2-1: Single antenna port reference sensitivity allowance ΔR1R, LP-WUR type 2

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5 | 1.3 |
| FDD band | 5 | 1.3 |

Editor’s note: TheΔR1R (dB) values in the table account for the 2.5 dB performance difference between 2Rx and 1Rx implementations, the variance of 3.2dB between 5 MHz REFSENS measurements for NR (25 RB) and LP-WUS (11 RB), and the increased noise figure (NF) for LR compared to MR. An additional 2 dB NF increment is assumed for LP-WUR Type 2 UE.

The REFSENS for LP-WUR is based on 11 RBs of configured LP-WUS for 5MHz CBW.

- 7.4M	Maximum input level for LP-WUR

Maximum input level is defined as the maximum mean power received at the UE antenna port. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x., with parameters as specified in Table 7.4-1.

Table 7.4M-1: Maximum input level

| Rx Parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | [-25] |

- 7.5M	Adjacent channel and subcarrier selectivity for LP-WUR

- 7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

- 7.5M.2	Adjacent channel selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 for LP-WUR type 1 UE and the minimum requirements specified in Table 7.5M.2-2 for LP-WUR type 2 UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5M.2-3 and Table 7.5M.2-4 for verification of the requirements specified in Table 7.5M.2-1 for LP-WUR type 1 UE, and as in Table 7.5M.2-5 and Table 7.5M.2-6 for verification of the requirements specified in Table 7.5M.2-2 for LP-WUR type 2 UE. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x.

Table 7.5M.2-1: ACS for LP-WUR type 1 UE

| Rx Parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| ACS | dB | 10.0 |
| NOTE1: The ACS value is defined based on 11 PRBs with xx guard RBs and the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.5M.2-2: ACS for LP-WUR type 2 UE

| Rx Parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| ACS | dB | [23] |
| NOTE1: The ACS value is defined based on 11 PRBs with xx guard RBs and the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + [22] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-4: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | [-33] |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.5M.2-5: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 35.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-6: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with xx guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

- 7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 for LP-WUR. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.x.x.

Table 7.5M.3-1: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS |
| Pinterferer,NR2 | dBm | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is configured at the center of NR carrier with [1] guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |