3GPP TSG-RAN WG4 Meeting # 113	                                                R4-2417892

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:	7.24.2.3

Source:	MediaTek Inc.

Title:	Discussion on conditional LTM

Document for:	Discussion

# Introduction

In this contribution, we provide our views on conditional LTM.

# Discussion

Before discussing conditional LTM, we would like to recall the procedure and requirements of conditional handover at first. As shown in Fig. 1, CHO starts from NW configures the RRC configuration of target cell and the conditions for conditional HO. After the condition is met, UE will start to handover to the target cell.

As far as we know, there are two drawbacks of CHO. One is that target cells should start to reserve the prepared conditional HO resources and context for the UE after the configuration, as target cell does not know when the condition will be met. Another is, source cell does not know when UE will start to handover to the target cell too, so during the HO procedure, source cell may keep scheduling the UE. These two all lead to some waste of NW resources or efforts.

Fig.1

In our view, when discussing conditional LTM, we should try to solve the drawbacks of CHO. For the first one, we don’t think it is RAN4’s work. For the latter, we believe RAN4 can do some optimization. Now the interruption time includes Tprocessing, time for T/F tracking and UL sync. In LTM, early T/F tracking is introduced to reduce both the cell switch delay and interruption. In our understanding, if UE supports early T/F tracking, it is also possible to perform T/F tracking before Tprocessing. As we all know, the time introduced by T/F tracking is mainly spent on waiting for the SSB occasion. The waiting time also contributes to the interruption. If UE performs T/F tracking before Tprocessing, the delay is not reduced, but the interruption will be reduced.

Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.

# Conclusion

In this contribution, we provide our views for conditional LTM. A proposal is proposed as follows.

Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.