3GPP TSG-RAN WG4 Meeting # 113	                                                R4-2417893

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:	7.17.4.1

Source:	MediaTek Inc.

Title:	Discussion on core requirements of AI/ML BM use case

Document for:	Discussion

# Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

# Discussion

## 2.1 Input and output of AI/ML BM models

Last meeting, RAN4 discussed whether Rx beam information is known for predicted beams. As this is the basis to further discuss other issues, we would like to provide our view here.

Issue 2-1: UE Rx beam knowledge Proposals - Option 1: UE Rx beam is known - the UE Rx beam gain has to be taken into account when making the prediction - Option 2: UE Rx beam is unknown - Option 3: depends on some conditions - which conditions? - Option 4: others

As captured in TR38.843 [1], in R18 SI, both DL Tx beam prediction and beam pair prediction are studied.

TR38.843 For both sub-use cases, the following alternatives are studied for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction (deprioritized) - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Using BM-case1 as an example:

In R19 WI, as pointed out in WID, only DL Tx beam prediction is in scope and beam pair prediction is not in the scope.

RP-242399 - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

In our understanding and also as the input of different companies in R18 SI (R18 AI/ML BM evaluation in RAN1), the model input and output of DL Tx beam prediction and beam pair prediction are shown in Fig.1 and Fig.2.

Fig.1 input and output of DL Tx beam prediction AI/ML model

Fig.2 input and output of beam pair prediction AI/ML model

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

Proposal 1: RAN4 to confirm Rx beam info is not part of the AI/ML model inputs or outputs in R19 AI/ML BM.

      - 2.2 Definition of known TCI state

Last meeting, there were some discussions on known TCI state definition in R19 AI/ML BM.

Issue 2-2: TCI State knowledge - Proposals - Option 1: TCI state is known if the RS is QCL-ed with an RS measured within 1280ms - Option 2: TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation. - Option 3: The TCI state which is QCL to an RS in Set B is known: - if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction. - if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction. - Option 4: The known conditions of the predicted TCI state shall take below conditions into account: - The TCI state switch command is received during inference phase of AI/ML mode. - The predicted beam in Set A is the RS in target TCI state. - Option 5: RAN4 should consider relaxing known conditions for TCI states - Option 6: others

The reason to discuss this issue lies in that there may be some impact on beam management related requirements as AI/ML beam prediction is for the purpose of beam management. For instance, after UE reports the predicted beams or RSRP to NW, NW may activate corresponding reported best-predicted beam. In RAN4, we defined TCI state switching delay requirements. Take the requirements of MAC CE based TCI state activation as an example. The TCI state switch delay is THARQ + 3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length, where TL1-RSRP = 0 for known TCI state and TL1-RSRP equals to L1 measurement delay in FR2 for unknown TCI state. If the TCI state is unknown, the switching delay would be long. One of the conditions of known TCI state is that UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command in 1280ms. The question here is whether the predicted beam is known or unknown if a prediction report is sent recently. Next, we would like to provide our views.

As UE is the one who performs TCI state switching, when talking about known condition, the corresponding TCI state should be actually known from UE side. At least two conditions shall be met in FR2. One is that SINR of the corresponding RS cannot be too low. Another is that UE knows its Rx beam for reception (for DL TCI state activation). To meet the 2nd condition, a TCI state is known only if UE has measured the corresponding RS and reported the measured result in last 1280ms before TCI state switching command.

As shown in Observation 1-3, only DL Tx beam prediction is in scope and the output of DL Tx beam prediction AI/ML BM model does not include Rx beam information. So if the beam to switch to does not belong to set B, even there is a prediction report but UE still does not know which Rx beam to use for DL reception.

Proposal 2: The TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation.

For the TCI state whose QCL source RS belongs to Set B, if it is spatial-domain beam prediction, the TCI state can be viewed as known if the corresponding predicted beam is reported in 1280ms. If it is temporal beam prediction, we should use the last observation occasion (T3) instead of the reported occasion as shown in Fig 3. The reason lies in that the time gap between T3 and T4 may be too long to ensure Rx beam unchanged. The report occasion may be close to T4.

Fig 3. Illustration for AI/ML temporal beam prediction

Proposal 3: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

      - 2.3 Metrics/KPIs for beam prediction

The issue which metric(s) to be used to evaluate the performance of AI/ML BM model had been discussed for several meetings. Now there are several options on table. In this section, we want to share some observations in our simulations and hope this is helpful for further discussions.

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

In the simulation, there are 32 Tx beams with 2 elevation angles and 16 azimuth angles in Set A and 8 Tx beams in Set B. We tried different set B patterns (e.g., in pattern 0, Set B = the first 8 beams in Set A) and got the Top 5/1 prediction accuracy requirements as in Fig. 4. It can be observed that the top 5/1 prediction accuracy of different patterns are varied. The gap is more than 10% between the best and worst.

Fig. 4

Observation 4: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it seems not possible to define a scenario-agnostic performance requirements based on the above observations. In R18 MIMO, we faced similar issue when discussing how to define requirements for TDCP. At that time, due to the requirements vary with different UE speeds, different serving frequency as well as different channels, RAN4 decided to define the test cases for limited cases (i.e., under certain doppler shift and channel model) instead of defining scenario-agnostic accuracy requirements. So we propose to discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 4: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

      - 2.4 Impact of measurement error

Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

In our simulations, Option 2 in R4-241721 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After tens of simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the variance value of BB measurement error under SNR= [-10, 20] dB with step 1dB at first.

During inference, the way to add measurement error is shown in Fig. 5:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-10,20] dB, we use interpolation method to obtain the variance value.

- A random BB error is generated following Gaussian distribution with the variance corresponding to the SINR value.

We simulated the performance with and without RF error. When adding RF error, RF error is modelled as Gaussian distribution with variance 4.5dB. For the same Rx beam, the same RF error is added for different Tx beams in each individual simulation.

We also simulated the case assuming SINR of the best Tx-Rx beam is 3dB. In such simulations, after calculating SINR of each Tx-Rx beam pair, we set the highest SINR of beams in set B to 3dB and get Delta, i.e., the highest SINR of beams in set B minuses 3dB. For the other SINR values, we get the equivalent SINR as “original SINR - Delta”. BB error is added based on the calculated equivalent SINR.

Fig. 5 Method of adding measurement error

The simulation results are provided as in Table 3. It can be observed that even Top-5/1 beam prediction accuracy is very low after adding measurement error and the average RSRP difference is huge. We think the reason lies in that it is very likely RSRP and SINR of some beams in Set B are quite low and the corresponding measurement error is very large as shown in Fig 6. With so large measurement error in the input, the model performance will be degraded a lot.

Table 1

| Input with error | Ground truth with error | BB error assumption | RF error assumption | top-1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) | Average RSRP diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | No | N.A. | N.A. | 75.13 | 95.19 | 97.99 | 0.97 |
| Yes | Yes | No limit on SINR of the best beam | N.A. | 37.94 | 53.6 | 61.06 | 16.14 |
| Yes | Yes | No limit on SINR of the best beam | 4.5dB | 39.37 | 55.61 | 61.38 | 16.04 |
| Yes | Yes | Assume SINR of best beam is 3dB | N.A. | 16.77 | 29.1 | 38.89 | 49.04 |
| Yes | Yes | Assume SINR of best beam is 3dB | 4.5dB | 18.78 | 31.43 | 41.01 | 50.06 |
| Note 1: Measurement error is only added at inference. The model is trained not considering measurement error. Note 2: Average RSRP difference: The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam. |  |  |  |  |  |  |  |

Fig. 6 BB error distribution

Observation 5: Beam prediction accuracy Top-K/1 degrades significantly after considering measurement error in set B, especially when the Tx beams are measured in very low SNR conditions.

Considering that in the real field, UE cannot get the RSRP of the beams when both RSRP and SINR are very low as those beams are not detectable. Some special handling on undetectable beams in set B is necessary in the real field. Similarly, it is not appropriate to just input a RSRP value which is very low in the simulation. Due to limited time, we just tried a simple test, i.e., set the normalized RSRP of the beams whose RSRP is 30dB lower than the best beam in set A as 0 at input and the simulation results are provided in Table 4. When determine which beam to be set as 0 normalized RSRP, we use the genie aided value, i.e., before adding measurement error. In this set of simulations, RF error is not added.

Table 2

|  | Input with error | Ground truth with error | Set normalized RSRP of the weak beams in Set B to 0 or not | Set normalized RSRP of the weak beams in Set A to 0 or not when judge the prediction accuracy | top-1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) | Average RSRP diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | No | No | N.A. | N.A. | 75.13 | 95.19 | 97.99 | 0.97 |
| 2 | Yes | Yes | Yes | No | 33.12 | 43.39 | 48.04 | 18.07 |
| 3 | Yes | Yes | Yes | Yes | 64.6 | 83.07 | 87.83 | 5.03 |
| 4 | Yes | No | Yes | No | 66.51 | 86.72 | 92.12 | 2.66 |
| Note 1: Measurement error is only added at inference. The model is trained not considering measurement error. Note 2: Measurement error is added corresponding to the real SINR of each beam, not assuming SINR of the best beam is 3dB. Note 2: Average RSRP difference: The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam. |  |  |  |  |  |  |  |  |

It can be observed that the performance degradation is greatly relieved by setting normalized RSRP of the weak beams in Set B and Set A to 0. Comparing simulation#2 with simulation#3, it can be observed that it is necessary to weed out the weak beams in Set A (simulation#3) when judging whether the prediction is right or not. The reason lies in that RSRP of some weak beams may be high after adding a large measurement error. In the simulation, it is possible to exclude the week beams in Set A as we can always get the genie aided RSRP, while it is not possible in the real field.  We think it may be acceptable as in general high RSRP is likely with high SINR.

We used to have a question that: After adding measurement error at both Set B and Set A, the prediction accuracy degrades. Is it because the model performance is not good or the “ground true” (measured results with measurement error) is not accurate? So we also checked the performance of the model by comparing to the genie aided RSRPs in set A (simulation#4). It can be observed that the prediction accuracy is higher than simulation#3 and the average RSRP difference is smaller. We think this proves that the reasons that the model performance degrades after considering measurement error lie in two folds: degradation of model itself and “ground truth” not accurate enough.

But in the whole simulations, we used the model trained not considering measurement error. In the real field, it is not possible to collect the data without error. To have a basic understanding of the model performance in the real field, we suggest to simulate the performance of models trained with measurement error too.

Observation 6: It is beneficial to improve AI/ML BM model performance by setting a threshold to exclude the dirty beams when measurement error is considered.

Observation 7: AI/ML BM model performance degrades after considering measurement error due to two aspects: degradation of model itself and “ground truth” not accurate enough.

Proposal 5: To practical AI/ML BM model performance in the real field, further study the performance of models trained with measurement error too.

      - 2.5 LCM of AI/ML BM models

In RAN1#118bis, they had some discussion on the signalling procedure on applicable functionality reporting for beam management UE-sided model based on the RAN2 input in LS R2-2407848.

Now they have three candidates as below. In our understanding, in option 2 and 3, the corresponding AI/ML model used for the configured for CSI report configuration(s) should be activated after receiving the RRCReconfiguration message in Step 5. If we consider legacy procedures, e.g., HO, PSCell addition and etc, CSI report configuration(s) can be configured at HO command or PSCell addition command. Now we are not sure whether CSI report configuration(s) in HO command or PSCell addition command will trigger model activation/switch. This depends on further discussion in RAN1/2. If CSI report configuration(s) in HO command or PSCell addition command will trigger model activation/switch, RAN4 should discuss how to extend HO or PSCell addition delay.

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Proposal 6: If some key parameter changes in HO will trigger BM model activation/switch, RAN4 should discuss whether and how to extend HO delay.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

Proposal 1: RAN4 to confirm Rx beam info is not part of the AI/ML model inputs or outputs in R19 AI/ML BM.

Proposal 2: The TCI state QCL to an RS that is not in Set B is unknown even if this RS was included in the L1-RSRP report by UE within 1280ms before TCI state activation.

Proposal 3: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

Observation 4: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Proposal 4: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Observation 5: Beam prediction accuracy Top-K/1 degrades significantly after considering measurement error in set B, especially when the Tx beams are measured in very low SNR conditions.

Observation 6: It is beneficial to improve AI/ML BM model performance by setting a threshold to exclude the dirty beams when measurement error is considered.

Observation 7: AI/ML BM model performance degrades after considering measurement error due to two aspects: degradation of model itself and “ground truth” not accurate enough.

Proposal 5: To practical AI/ML BM model performance in the real field, further study the performance of models trained with measurement error too.

Proposal 6: If some key parameter changes in HO will trigger BM model activation/switch, RAN4 should discuss whether and how to extend HO delay.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface