3GPP TSG-RAN WG4 Meeting # 113	                                                R4-2417895

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:	7.18.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements of AI mobility

Document for:	Discussion

# Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction use-case. In this contribution, we would like to discuss the related RRM requirements.

# Discussion

Last meeting, RAN4 agreed to start the discussion on scenario 2, 3, 4. In the next, we will focus on the three scenarios.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

## 2.1 Metrics used to define the model performance

For AI/ML BM, there were a lot of discussions on the metrics used to define the model performance. Similar as AI/ML BM, the metrics should be discussed for AI/ML mobility too. RAN2 agreed to use average RSRP difference as prediction accuracy metric for RRM measurement prediction.

RAN2#126 Agreements Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged.

Last meeting, RAN4 also discussed which metric(s) to be used and reached the following agreement:

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

In our understanding, the divergence is whether to use beam-level metric for sub-use case 1. We think using cell-level RSRP is enough. In sub-use case 1, even the output of the model is beam-level result, cell level result will be generated. In our understanding, the three sub-use cases correspond to different models. Which kind of model to use is totally up to implementation. And no matter which model used, it is the accuracy of the final cell level result that matters. That’s why RAN2 agreed to the prediction accuracy metric “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases.

Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric.

For information Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

In the next, we would like to discuss the discuss the requirements for each scenario.

For scenarios 2 and 4, i.e., intra-frequency intra-cell temporal prediction, the final output will be cell-level measurement result of a single cell. So the only requirement to define is the absolute accuracy of predicted/generated L3 cell level RSRP.

Proposal 2: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP.

For scenario 3, i.e., FR1 to FR1 inter-frequency inter-cell prediction (frequency domain), now RAN2 focuses on the prediction on single cell on a single inter-frequency layer. In our understanding, the benefit of scenario 3 comes from allowing to use a much longer MGRP or skipping some gap occasions for inter-frequency measurements. As well known in RAN4, if UE only predicts the measurement results on a single cell, it is not possible to reduce MGRP. UE is supposed to use MG to search and measure all the inter-frequency cells on the same and different frequency layers. We believe the scenario is more valuable to predict the measurement results for multiple cells and cells on multiple frequency layers. So the output will be cell-level RSRP of multiple cells. There will be absolute and relative accuracy requirements for discussion. In our understanding, the requirements to define shall meet the needs. If the predict measurement results are used directly for HO decision, both absolute and relative accuracy of the predict results are important.

Proposal 3: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP.

## 2.2 Impact of measurement error

Similar as AI/ML BM, when defining the prediction accuracy requirements, the impact of measurement error should be considered. The methodology shall be similar too. Last meeting, some simulation assumptions are agreed to as a start. We think it is time to use their simulation assumptions as a base to discuss the simulation assumptions here. In the next, we would like to propose a method of adding measurement error for further discussion:

- At first, get the distribution of BB error under different SNRs, i.e., TDL channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After tens of simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the variance value of BB measurement error under SNR= [-10, 20] dB with step 1dB at first.

- During inference, the way to add measurement error is shown in Fig. 1:

- In SLS simulation, SINR of each measurement occasion is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each measurement occasion, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-10,20] dB, we use interpolation method to obtain the variance value.

- A random BB error is generated following Gaussian distribution with the variance corresponding to the SINR value.

- RF error can be modelled as Gaussian distribution or uniform distribution.

Fig. 1

Last meeting, there were some discussions on which group to do the simulations considering measurement error, RAN2 or RAN4. We think the impact of measurement error should be discussed in RAN4. RAN2 are not familiar with measurement uncertainty. If it is RAN2 to do the simulations, they will need RAN4 inputs. We don’t think it is easy to figure out how to add error in RAN4. We expect there will be long discussions on the simulation assumptions and the way to add measurement error. RAN4 may need to adjust the method based on the simulation results. It will be more efficient to discuss the impact of measurement error in RAN4.

Proposal 4: RAN4 should study the impact of measurement error on the prediction accuracy requirements for AI mobility. A method is proposed above for further discussion.

## 2.3 Other RRM impacts

UE measurement and/or report behavior will be different from legacy for all the scenarios 2,3,4. RAN4 shall study whether there is any impact on measurement delay and report delay requirements. Regarding the impact on report requirements, there will be similar issue for AI/ML BM. We suggest to wait for more progress in AI/ML BM.

Proposal 5: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Next, we will focus on the impact on measurement delay requirements.

Scenario 2, FR1 to FR1 intra-frequency intra-cell temporal domain case B prediction, is to predict a subset of measurement instances in temporal domain of the same cell. This scenario is similar as BM case 2. The impact on measurement delay would take BM case 2 as a basis.

Proposal 6: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Scenario 3, FR1 to FR1 inter-frequency inter-cell (frequency domain), in our understanding, this scenario includes “prediction on an inter-frequency layer based on measurement on an intra-frequency layer” and also “prediction on an inter-frequency layer based on measurement on a different inter-frequency layer”. As discussed in previous part, the scenario is more valuable to predict the measurement on multiple frequency layers. In our understanding, MG still needs to be configured at least for model monitoring. If there are more than one inter-frequency layers involved, RAN4 needs to discuss the gap sharing mechanism among different inter-frequency layers both for measurement and monitoring. There would be some impact on measurement delay requirements.

Proposal 7: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Scenario 4, FR2 to FR2 intra-frequency intra-cell temporal domain case A, is to predict measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window. There is no impact on UE measurement behavior, so there is no impact on the measurement delay.

Proposal 8: Scenario 4 has no impact on measurement delay requirements.

# Conclusion

In this contribution, we provide our views for AI mobility. Proposals are summarized as follows.

Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric.

Proposal 2: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP.

Proposal 3: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP.

Proposal 4: RAN4 should study the impact of measurement error on the prediction accuracy requirements for AI mobility. A method is proposed above for further discussion.

Proposal 5: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Proposal 6: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Proposal 7: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Proposal 8: Scenario 4 has no impact on measurement delay requirements.

# Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.