3GPP TSG-RAN WG4 Meeting # 113	                                                R4-2417896

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:	7.18.3

Source:	MediaTek Inc.

Title:	Discussion on testability and interoperability of AI mobility

Document for:	Discussion

# Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction use-case. In this contribution, we would like to discuss the related testability issues.

# Discussion

Last meeting, RAN4 agreed to consider “Conductive testing” as a baseline option for testing in FR1. There were some proposals to study the testability regarding with more cells and more than one frequency layers in the test. In our understanding, there is no testability issue with conductive testing in FR1.

Take scenario#3 FR1 to FR1 inter-frequency prediction as an example. There may be several cells on different frequency layers. There will be no issue to simulate multiple frequency layers which is widely used in some non-AI test cases, e.g., inter-frequency measurement/HO test cases. In our understanding, as UE is connected to the channel simulator through cable, the test would be similar as computer simulations. If the model is to predict measurement results of Cell 2 and Cell 3 on different frequencies from serving cell (Cell 1), a test setup as shown in Fig.1 can be used. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. SSB is transmitted on cell 1 at all the SMTC occasions.  For cell 2 and cell 3, SSB will not be transmitted on the SMTC/MG occasions to be predicted. It is not necessary to change TE Tx power every symbol or slot. We think it is sufficient to change TE Tx power every SMTC. Till now, we didn’t find any testability issue in FR1 test yet.

To avoid missing something, we are open to take a scenario as an example to further discuss the test set up details.

Fig. 1

Proposal 1: Use conductive test in FR1 for AI/ML mobility, there is no testability issues found.

Proposal 2: Open to take a scenario as an example to further discuss the test set up details to check whether there is potential testability issues.

Another thing we would like to discuss is about the “ground truth”. In AI/ML BM, there are lots of discussions on how to get the ground truth as beamforming gain is unavailable. In FR1, using conductive test, ground truth (e.g., RSRP of each cell) is available at TE as legacy.

Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: Use conductive test in FR1 for AI/ML mobility, there is no testability issues found.

Proposal 2: Open to take a scenario as an example to further discuss the test set up details to check whether there is potential testability issues.

Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility.

# Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.