3GPP TSG-RAN WG4 Meeting #113								    R4-2417903

Orlando, US, 18th – 22nd November, 2024

Agenda Item: 7.2.4.1

Source: MediaTek Inc.

Title: Coexistence study for 14800 to 15350 MHz

Document for: Discussion

# 1	Introduction

In WFs [1][2] from RAN4#112-bis meeting, RAN4 agreed on giving priority to a set of simulation parameters, allowing companies to streamline simulation efforts and bring the corresponding results to this meeting. By doing so, companies can compare the results on a common basis, facilitating a consensus on the final ACIR value. Additionally, considering this is the last meeting before sending out the LS reply to ITU, it is imperative the ACS/ACLR values are also finalized in this meeting.

Some prioritized simulation parameters agreed in WFs [1][2] are captured below:

On BS side: Issue 3-2: Scope of scenarios Agreement: - Limit simulations to urban macro Issue 3-9: ISD for simulation Agreement: - 450m Issue 4-17: BS antenna parameters For Urban Macro: Agreement: - Make assumption that reply can say that array size is [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible. - Double check the coverage and [3072] before making final decision on reply - If coverage issues is found for [3072], minimum array size [4096] to achieve coverage could be discussed On UE side: UE-type discussion - LS - Only one UE antenna model to be captured. - 1Tx omnidirectional antenna UE, understood as the worst case (highest ACIR values) assumptions for sharing studies and not on deployment, coverage, implementation, or requirement aspects. UE output power - 23dBm, mention other power classes possible in TR Noise figure - For LS to ITU, noise figure is [13] dB - The common understanding is that the current co-existence simulation work (based on 11 dB UE NF) is not changed due to above noise figure value.

In this paper, we provide our coexistence study outcomes based on the prioritized simulation settings, along with the proposed ACIR values.

# 2	Simulation assumption

For the coexistence study, the simulation methodology is described in Sec. 5.3 of TR38.803.

In the following table, we provide the some of the parameters assumed in our simulations.

| Paramater | Configuration |
| --- | --- |
| Scenario | UMa outdoor with an ISD of 450m |
| Indoor UE ratio | 20% |
| Multi-operator network deployment | Uncoordinated operation |
| Channel BW | 100MHz/200MHz |
| BS max output power | 46dBm for dual-pol |
| Number of BS antenna | 3072/4096 for dual-pol |
| UE antenna | 1TX Omni-directional antenna |
| UE max output power | 23dBm |
| UE noise figure | 11dB |
| Others | See TR38.922 [3] |

# 3	Coexistence simulation results

In this section, we provide our evaluation results for different channel bandwidths and BS antenna sizes.

## 3.1	Coverage issue

In this subsection, we provide our views on the coverage issue in 15GHz bands based on our evaluations.

In addition to ACIR performance, the coverage performance is one key aspect we need to account for, as it plays a significant role on whether BS antenna size needs to go beyond 3072 to provide service for cell-edge users. In our understanding, typically, RAN4 identifies the coverage issues by examining the UL SINR distribution (CDF) to check whether the outage rate is larger than 5% (i.e., Prob.(UL SINR<=-10dB)>0.05) in a given deployment scenario. In this context, it is challenging for UL to achieve target coverage based on the prioritized assumption for coexistence simulation in 14800MHz to 15350 MHz frequency range, even with 4096 BS antennas. The main difficulty comes from

- the single omni-directional TX antenna assumption for UE, and

- the maximum transmission power of 23dBm distributed across the full channel bandwidth of up to 200MHz.

The following figures present the CDF of UL SINR for channel bandwidths of 100MHz and 200MHz with different BS antenna sizes.

Figure 3.1.1 shows the UL SINR distribution for a 100MHz bandwidth across various BS antenna sizes, not considering ACI. Despite the smaller bandwidth, the outage rate is around 4%, which is nearly at the 5% outage threshold, regardless of antenna size. As the channel bandwidth is doubled to 200MHz, as shown in Figure 3.1.2, the outage rate escalates to 7% assuming the maximum UE output power remains unchanged with the increase in bandwidth. This indicates that 5%-tile UE throughput cannot be achieved even without considering ACI.

Therefore, in the results provided in later sections for coexistence study, we calculated the 5%-tile UE throughput loss excluding the outage UEs. That is, if the UL SINR without considering ACI falls below -10dB, it is removed from the throughput analysis. This is because if throughput is already zero before ACI is considered, no additional throughput loss is expected after ACI is considered.

Figure 3.1.1 UL SINR distribution with 3072 and 4096 BS antennas under 100MHz channel bandwidth

Figure 3.1.2 UL SINR distribution with 3072 and 4096 BS antennas under 200MHz channel bandwidth

Observation 1: Outage rate is approximately 4~6%, regardless of channel bandwidth and BS antenna size. This is attributed to the use of single omni-directional antenna for the UE and full-BW PUSCH transmission with a fixed maximum UE output power of 23dB.

## 3.2	100MHz channel bandwidth

The throughput loss performance for ACIR values ranging from 10dB to 30dB are provided in the following figures, including 3072 and 4096 BS antenna elements.

Figure 3.2.1 DL cell average and 5%-tile UE throughput loss with 3072 and 4096 BS antennas

Figure 3.2.2 UL cell average and 5%-tile UE throughput loss with 3072 and 4096 BS antennas

## 3.3	200MHz channel bandwidth

The throughput loss performance for ACIR values ranging from 10dB to 30dB are provided in the following figures, including 3072 and 4096 BS antenna elements.

Figure 3.3.1 DL cell average and 5%-tile UE throughput loss with 3072 and 4096 BS antennas

Figure 3.3.2 UL cell average and 5%-tile UE throughput loss with 3072 and 4096 BS antennas

## 3.4	Summary

Based on our evaluation results, we have observed that the ACIR performance is nearly identical across different BS antenna sizes, with only slight variations when the channel bandwidth is altered. In summary, to meet the 5% loss criterion for a 100MHz channel bandwidth, a DL ACIR of 24dB and an UL ACIR of 17dB are required. For a 200MHz channel bandwidth, a DL ACIR of 23dB and a UL ACIR of 15dB are adequate to maintain the loss below 5%.

Observation 2: For a channel bandwidth of 100MHz, the maximum required DL ACIR value does not exceed 24dB, and the maximum required UL ACIR value does not exceed 17dB.

Observation 3: For a channel bandwidth of 200MHz, the maximum required DL ACIR value does not exceed 23dB, and the maximum required UL ACIR value does not exceed 15dB.

# 4	Conclusion

In this paper, we present our evaluation results for ACIR performance in 14800MHz to 15350MHz frequency range, following the prioritized assumptions agreed in the last meeting. Below, we summarize our observations and proposals.

Observation 1: Outage rate is approximately 4~6%, regardless of channel bandwidth and BS antenna size. This is attributed to the use of single omni-directional antenna for the UE and full-BW PUSCH transmission with a fixed maximum UE output power of 23dB.

Observation 2: For a channel bandwidth of 100MHz, the maximum required DL ACIR value does not exceed 24dB, and the maximum required UL ACIR value does not exceed 17dB.

Observation 3: For a channel bandwidth of 200MHz, the maximum required DL ACIR value does not exceed 23dB, and the maximum required UL ACIR value does not exceed 15dB.

# 5	References

R4-2417108, WF on 15GHz UE parameters, Qualcomm

R4-2417107, WF on 15GHz BS parameters, ZTE

TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 – 4800MHz, 7125 – 8400 MHz and 14800 – 15350 MHz”, 3GPP V0.3.0.