# 3GPP TSG-RAN WG4 Meeting #113	R4- 2417934

Orlando, United States, 18th – 22th November, 2024

Agenda item:	7.8.3.1

Source: 	Xiaomi

Title: 	Discussion on system parameters for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 had some discussion on system parameters for NTN less than 5MHz, and agreement and open issues were captured in WF [1]. The remaining open issues are copied as follows:

System parameters Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Way forward: - Option 1: Mandatory - Option 2: Optional Issue 1-3: Signalling of SS raster - Way forward: - Option 1: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations. - Option 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear. - Option 3: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

This contribution discusses above open issues on system parameter for NTN less than 5MHz and provides our proposals.

Discussion

In RAN#112bis meeting, RAN4 had some discussion on whether the 3MHz channel bandwidth is mandatory or optional. From our perspective, we support 3MHz CBW is introduced as an optional UE capability, since UE may reuse the existing modem and RF components from TN to support NTN operations, and the 3MHz CBW introduced for NR TN is optional in Rel-18. In addition, RAN2 has already define UE capability as optional feature to support 3MHz CBW, and no impact on RAN2.

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

If RAN4 agrees 3MHz CBW is an optional UE capability for NTN bands, the exiting signalling can be reused. In Rel-18, UE capabilities of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 were introduced to support 3MHz CBW in symmetrical and asymmetrical configurations.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

Conclusion

In this contribution, we discussed the remaining open issues on system parameter for NTN less than 5MHz and provides our proposals:

Proposal 1: RAN4 to define 3MHz channel bandwidth is optional for FR1 NR-NTN bands.

Proposal 2: If 3MHz CBW is optional for FR1 NTN bands, the exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

- Reference

[1]	R4-2416587	Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF		Xiaomi