# 3GPP TSG-RAN WG4 Meeting #113	R4-2417936

Orlando, United States, 18th – 22th November, 2024

Agenda item:	7.21.3.2

Source: 	Xiaomi

Title: 	TP for TR 38.769 on RF requirements for device 2a

Document for:	Approval

Introduction

This TP to TR 38.769 is to capture RF requirements for device 2a based on the agreements and conclusions in previous RAN4 meetings.

Text proposal

<Start of text proposal>

## 6.7	RF requirements study

### 6.7.4	Ambient IoT Device

#### 6.7.4.1	Device 1

#### 6.7.4.2	Device 2a

- 6.7.4.2.1	Transmitter characteristics

- Tx maximum output power

For device 2a, the transmitter architecture is based on backscattering technology, and the backscattering power or power backscattering loss needs to be defined to ensure the coverage target. The maximum output power is not needed.

- Output power dynamic

For device 2a, power control is not designed for backscattering transmission, and RAN4 concluded that all the output power dynamic requirements are not relevant for backscattering type devices.

- Transmit OFF power: when there is no incoming CW signal, the device 2a does not emit any high-power level. The requirement of transmit OFF power is not needed.

- Transmit ON/OFF time mask: The time mask requirement is to test the ON/OFF time period for RF chain. For device 2a, this time period is not crucial and it is not needed.

- Minimum output power: For device 2a, the output power level is based on the incoming CW signal and no power control is applied, thus, the minimum output power requirement is not needed.

- Power control: Power control is not designed for backscattering transmission, the power control requirement is not needed for device 2a.

- Transmit signal quality

- Frequency error: TBD

- Modulation quality requirements: TBD

- In band emissions: TBD

- Carrier leakage: TBD

- Output RF spectrum emissions

- Occupied bandwidth: The occupied bandwidth is the width of a frequency band containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Occupied bandwidth is needed for Device 2a. The specific value is dependent on the physical layer design of D2R, which has not yet been finalized at this stage.

- SEM: TBD

- ACLR: TBD

- Spurious emissions: TBD

- Transmit intermodulation:

The transmit intermodulation is the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. However, device 2a is low complexity. SEM, ACLR and spurious emissions may cover this function and requirement. Transmit intermodulation requirement is unnecessary.

- Unwanted emissions: TBD

The summary of Tx requirements for device 2a is captured in Table X.

| A-IoT device TX (D2R) RF Requirement |  | Needed or not |
| --- | --- | --- |
| Transmit output power | Maximum output power /output power | No for maximum output power YES for backscattering power or power backscattering loss |
| Output power dynamic | Transmit OFF power | No |
|  | Transmit ON/OFF time mask | No, as RAN4#112 agreed |
|  | Minimum output power | N/A |
|  | Power control | No, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | TBD |
|  | Modulation quality requirements | TBD |
|  | In band emissions | TBD |
|  | Carrier leakage | TBD |
| Output RF spectrum emissions | Occupied bandwidth | YES |
|  | SEM | TBD |
|  | ACLR | TBD |
|  | Spurious emissions | TBD |
| Unwanted emissions |  | TBD |
| Transmit intermodulation |  | No |

- 6.7.4.2.2	Receiver characteristics

- Reference sensitivity

The reference sensitivity is the minimum mean power applied to device antenna port to guarantee the receive performance. The requirement of reference sensitivity for device 2a is needed.

- Maximum input level

TBD

- Adjacent channel selectivity (ACS)

For device 2a, the RF BPF is not assumed in RF architecture for the reason of cost and size consideration, So the ACS requirement which relies on digital filter is not needed.

- Adjacent Subcarrier selectivity (ASCS)

Same as ACS, no need to define ASCS.

- In-band blocking

Same as ACS, no need to define in-band blocking requirement.

- Out-of-band blocking

Same as ACS, no need to define out-of-band blocking requirement.

- Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The received power comes from R2D signal. For device 2a, LNA is not assumed in RF architecure. Hence, there is no need to define spurious emissions.

- Spurious response

Same as ACS, there is no need to define spurious response.

- Intermodulation Characteristic

Same as ACS, there is no need to define intermodulation characteristic.

- Interference rejection

The interference could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronised reader/CW node) or D2R signal from other devices . Whether interference rejection to be defined needs to be discussed in WI.

The summary of Rx requirements for device 2a is captured in Table Y.

| A-IoT device- RX Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | TBD |
| ACS | NO |
| ACSC | NO |
| In-band blocking | NO |
| Out-of-band blocking | NO |
| Receiver intermodulation | NO |
| Rx spurious emission | NO |
| Spurious response | NO, as RAN4#112 agreed |
| Interference rejection | TBD |

<End of text proposal>