3GPP TSG-RAN WG4 Meeting #113			                                                   R4-2417937

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.1.1.1.2

Source:	Xiaomi

Title:	Discussion on PC1.5 TDD intra-band CA

Document for:	Discussion

- Introduction

Regarding the discussion on introduction of PC1.5 TDD intra-band CA, the agreements and open issues of last meeting were captured in the WF [1], this contribution continues to provide our views based on the WF.

- Discussion

In the last meeting, there is a discussion on MPR evaluation methodology and assumption for PC1.5 intra-band CA. The agreements and open issues listed in the WF are summarized as follow.

1.1 MPR evaluation methodology and assumption Agreement: - Keep the following last meeting’s agreement for intra-band non-contiguous ULCA - Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA - It does not preclude companies to bring evaluation for unequal PSD - MPRc = MPR, which means no change of spec

According to the above WF, it was already agreed that MPR/A-MPR requirements are defined based on equal PSD assumption for intra-band contiguous ULCA. However, whether the PSD imbalance shall be considered for intra-band non-contiguous CA is still FFS. From our perspective, there are at least two issues needs to be clarified before introducing PSD imbalance to MPR requirements.

One is about what the maximum PSD imbalance is. If the extreme configuration where one carrier could be at 100 MHz with full allocation (273RB at 30kHz SCS) while the other carrier is only allocated with 1RB is considered, the PSD imbalance could be as high as 10*log10(273/1) dB = 24.4 dB. However, in above WF, it seems 6dB is considered as the maximum PSD imbalance. We think it is necessary to clarify the reason why above assumption is considered.

Observation 1: the maximum PSD imbalance can be 24.4dB if the extreme configuration is considered.

Observation 2: the reason of the assumption of 6dB PSD imbalance needs to be clarified.

The other is about the impact on MPR/A-MPR due to PSD imbalance. In previous discussion, to be safe, there is an option that defining MPR requirements based on worst case. However, as pointed out in our contribution [2], the impact of PSD imbalance on MPR is very is very complicated. e.g., in some RB allocation areas, the larger the PSD offset, the larger the MPR, but in other RB allocation areas, the situation may be opposite. it is hard to get a quantitative relationship between PSD imbalance and MPR value. Consequently, it is hard to get the worst case if also different bandwidth combination configurations and different PSD imbalance are considered.

Observation 3: The impact of PSD imbalance on MPR is very complicated. In some RB allocation areas, the larger the PSD offset, the larger the MPR, but in other RB allocation areas, the situation may be opposite.

Observation 4: it is hard to get the worst case if also different bandwidth combination configurations and different PSD imbalance are considered.

Therefore, we think defining MPR requirements should be from the perspective of typical scenario in the field other than from the perspective of worst case. For intra-band CA, the typical scenario is that PSD offset is not that much. Therefore, it is reasonable to still assume equal PSD when deriving MPR requirements for PC1.5 intra-band CA. Moreover, this assumption is more convenient for conformance testing.

In view of above, we give the following proposal.

Proposal 1: define MPR requirements for PC1.5 intra-band NC CA also based on equal PSD.

The other open issue can be found in the below:

1.2 PCMAX,C Agreement: - Agreement: - For a contiguous intra-band combination with PC1.5 capability supporting by txDiversity-r16 - The PCMAX,f,c and the total configured output power PCMAX for the band combination are given by clause 6.2A.4.1.1 for intra-band contiguous CA in the current version of 38.101-1, no change for PC1.5 - For a non-contiguous intra-band combination with PC1.5 capability supporting dualPA-Architecture - For PCMAX,f,c, keep the following last meeting’s agreement, and the detailed wording can be discussed based on the CR in future - For PCMAX, FFS

From the agreement, the upper bound of Pcmax,c should be limited to 26dBm per CC for intra-band UL NC CA with dualPA-architecture. From our perspective, current spec doesn’t reflect this agreement. Thus, spec needs to be updated accordingly. The following changes could be as a reference.

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4. For power class 1.5 intra-band non-contiguous carrier aggregation, the PPowerClass per CC shall be PC2 when the signalling is indicated for dualPA-Architecture IE.

Proposal 2: for intra-band UL NC CA with dualPA-architecture, spec needs to be updated to reflect the agreement that the upper bound of Pcmax,c is 26dBm per CC.

- Conclusion

In this paper, we give the further analysis based on the open issues listed in the WF and make the following observations and proposal:

Observation 1: the maximum PSD imbalance can be 24.4dB if the extreme configuration is considered.

Observation 2: the reason of the assumption of 6dB PSD imbalance needs to be clarified.

Observation 3: The impact of PSD imbalance on MPR is very complicated. In some RB allocation areas, the larger the PSD offset, the larger the MPR, but in other RB allocation areas, the situation may be opposite.

Observation 4: it is hard to get the worst case if also different bandwidth combination configurations and different PSD imbalance are considered.

Proposal 1: define MPR requirements for PC1.5 intra-band NC CA based on equal PSD.

Proposal 2: for intra-band UL NC CA with dualPA-architecture, spec needs to be updated to reflect the agreement that the upper bound of Pcmax,c is 26dBm per CC.

Reference

- R4-2417098, WF on HPUE for CA in TN, Samsung

- R4-2415053, Discussion on PC1.5 TDD intra-band CA, Xiaomi

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)