3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417939

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.1.1.1.4

Source:	Xiaomi

Title:	Discussion on increasing high power limit for inter-band CA/DC with 2Tx and or 3Tx

Document for:	Discussion

- Introduction

In the last RAN4#112bis meeting, a draft CR [1] was endorsed based on the following agreements for enabling Rel 19 increasing high power limit for inter-band CA/DC

1. The methdology - Use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability - No new power class is introduced 2. The scenarios - For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination 3. MSD and SAR - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation 4. The following one is not specific for increasing higher power but have impact on the scenarios for this feature - Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario

However, as mentioned in the contribution [2], there is an issue on PCMAX tolerance if the power class configurations that total Pcmax exceeds 29dBm are supported. This contribution further discusses this issue.

- Discussion

From the agreements in the previous meetings, it can be seen that in Rel.19, the scenarios for increasing high-power limit will extend to all the possible power class configurations, other than only limit to PC2 (TDD band) +PC3 (FDD/TDD) or PC3+PC5 case in Rel.17 and Rel.18. Therefore, theoretically, the following power class configurations that total Pcmax exceeds 29dBm also can be supported.

PC3 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx     total power = 30.0 dBm (already considered in the spec)

PC2 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx    total power = 30.8 dBm

However, in current spec, the upper limit for Pcmax in Pcmax tolerance requirement is 29dBm for inter-band CA with 2Tx or 3Tx which is shown as below. Therefore, if above power class configurations that total Pcmax exceeds 29dBm can be enabled by the capability of increasing high power limit, the upper limit 29dBm in Pcmax tolerance requirement needs to updated, otherwise, the requirement is not incomplete as there is no tolerance requirements for Pcmax more than 29dBm

Table 6.2A.4.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands) pasted from Ts38.101-1

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

Observation 1: the total configured maximum output power PCMAX in the following power class configurations can exceeds 29dBm.

PC3 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx     total power = 30.0 dBm (already considered in the spec)

PC2 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx    total power = 30.8 dBm

Observation 2: in current spec, the upper limit of Pcmax in Pcmax tolerance requirement is 29dBm for inter-band CA with 2Tx or 3Tx

Proposal 1: it shall be decided whether the Power class configurations that total Pcmax exceeds 29dBm shall be considered in Rel 19. If Yes, the Pcmax tolerance requirement needs to updated.

The following change could be as a reference.

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 31 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

- Conclusion

In this paper, we make the following observations and proposals:

Observation 1: the total configured maximum output power PCMAX in the following power class configurations can exceeds 29dBm.

PC3 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx     total power = 30.0 dBm (already considered in the spec)

PC2 (TDD/FDD) +PC1.5 indicating PC1.5 with 3Tx    total power = 30.8 dBm

Observation 2: in current spec, the upper limit of Pcmax in Pcmax tolerance requirement is 29dBm for inter-band CA with 2Tx or 3Tx

Proposal 1: it shall be decided whether the Power class configurations that total Pcmax exceeds 29dBm shall be considered or not in Rel 19. If yes, the Pcmax tolerance requirement needs to updated.

The following change could be as a reference.

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 31 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

Reference

- R4-2417096	Draft Rel-19 CR for 38.101-1 on increasing higher power limit with 2Tx or 3Tx, Samsung

- R4-2415055, Discussion on increasing high power limit for inter-band CA/DC with 2Tx and or 3Tx, Xiaomi

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)