3GPP TSG-RAN WG4 Meeting #113			                                          R4-2417940

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.12.2.2

Source:	Xiaomi

Title:			Discussion on OTA test for FR1 NTN devices

Document for:	Discussion

- Introduction

The test methodology and radiated performance metric for FR1 NTN devices has been extensively discussed in previous meetings. The latest progress was captured in the WF [1] as follow. This contribution continues to provide our views on these issues.

### Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN) Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE Agreements: - NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. Issue 3-1-2: Check whether the UE orientation is arbitrary or directional during NR-NTN communication Agreements: - For NR-NTN, the target usage of NR-NTN UE is close to normal NR UE. ### Sub-topic 3-2 UE performance metric Issue 3-2-1: Performance metric for NTN device Agreements: - For NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline. Other performance metric can be considered if issue is identified. - For IoT-NTN UEs, FFS performance metric. Issue 3-2-2: Whether performance metric should be different for GEO and NGEO for handheld UEs Agreements: - FFS whether different metrics for GEO and NGEO would be needed.

This contribution continues to provide our views on these issues.

- Discussion

Performance metrics

According to the above WF, it was already agreed that for NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline in the last meeting. The agreement mainly targets for handheld devices to ensure that spherical coverage is independent on device orientation. In RAN4#112 meeting, we have the following agreements for UE types and Usage Scenarios for IoT-NTN UE in the WF [2]. From the agreements, it can be concluded that only handheld UE type is considered as 1st priority. For handheld UE, it is expected that there is no difference in antenna design between NTN UE and IoT-UE, and therefore it is suggested to adopt the same performance metric.

Issue 2: UE type for IoT-NTN Agreements - For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority. Issue 3: Usage scenarios for IoT-NTN handheld UE Agreements - Same prioritized usage scenarios as NR-NTN handheld UE

Proposal 1: For IoT-NTN handheld UEs, take the same performance metrics as NR NTN UE, i.e., take a whole sphere measurement with TRP and TRS as the performance metrics.

The other issue is whether performance metric should be different for GEO and NGEO (including LEO and MEO). As pointed out in our contribution [3], developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE. The reason is that the elevation angle of the fixed UE is generally fixed, so the spherical coverage can be smaller when it only supports the GSO scenario. However, As the orientation of the handheld UE is arbitrary, so even if the handheld UE only supports the GSO scenario, its spherical coverage is still required to be larger. In addition, from UE implementation point of view, if the supported satellite band is the same, the unified antenna design will be adopted. In view of the above, it is proposed for handheld UE, there is no need to adopt different metrics for GSO and NGSO

Proposal 2: For handheld UE, there is no need to adopt different metrics for GSO and NGSO

3	Conclusions

In this paper, we provide our views on developing test methodology and radiated performance metric for FR1 NTN devices, and provide the proposal as below:

Proposal 1: For IoT-NTN handheld UEs, take the same performance metrics as NR NTN UE, i.e., take a whole sphere measurement with TRP and TRS as the performance metrics.

Proposal 2: For handheld UE, there is no need to adopt different metrics for GSO and NGSO

4 References

[1] R4-2416590, Way Forward for [112bis][323] TRP_TRS_Ph3, Vivo

[2] R4-2413603, Way Forward for [111] TRP_TRS_MIMO_OTA, Vivo, CAICT

[2] R4-2415056,	Discussion on OTA test for FR1 NTN devices, Xiaomi

[3] RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, Moderator (RAN4 VC, Qualcomm Incorporated)