3GPP TSG-RAN WG4 Meeting # 113	R4-2417948

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for beam management

Agenda Item:	7.17.4.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability issues for beam management.

- Channel modelling - Measurement error impact - Ground truth

- Discussion

      - 2.1 Channel modelling

- Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

There are two possible ways for channel model, i.e. CDL/simplified CDL or TDL.

### 2.1.1 TDL channel model

For TDL channel, single probe and single AOA will be used.

How to generate TX power

TDL and CDL channel has the same cluster number and cluster power. Difference is that TDL channel didn’t have spatial angle information. Analogue beamforming can’t be applied for TDL model. A simple solution is that the TX power for different TX beams can be calculated by CDL channel multiplied by TX beamforming and RX beamforming.

Since there are multiple TX beams in set A or set B, there will be different L1-RSRPs for different TX beams. However, TDL channel model didn’t have spatial angle information, analogue beamforming can’t be applied for TDL channel. In order to emulate TX beamforming gain for clusters of different AOD, AOA, ZOD, ZOA, RAN4 needs to discuss how to generate different TX power for beams in set B or set A for TDL channel.

Since TDL and CDL channel has the similar cluster number and relative cluster power. A possible solution is that the TX power for different TX beams can be calculated by CDL channel multiplied by TX beamforming and RX beamforming.

Proposal 1: For TDL channel model, RAN4 needs to discuss how to generate different TX power for beams in set B or set A.

TX power range

The TX power for different TX beams for TDL channel can be calculated based on CDL channel. Take CDL-C as example, ideal L1-RSRP difference for 32 TX beams in set A are shown in Fig.1(a) below. In order to compare the L1-RSRP difference between different TX beams in set A, the maximum L1-RSRP in 32 TX beams is normalized to 0dB.

It can be seen that the maximum L1-RSRP and minimum L1-RSRP in set A can be more than 20dB. Since in test, different AOA, AOD, ZOA, ZOD may be needed, and a CDL-C only represents a single channel realization. we further analyse the L1-RSRP delta for 32 TX beams based on SLS L1-RSRP in Fig.1(b).

    - Ideal L1-RSRP from CDL-C                     (b) Ideal L1-RSRP from SLS

Fig.1 Ideal RSRP difference for beams in set A

It can be seen that L1-RSRP delta between the best 1st beam and worst beam can be more than 25dB for 10%, larger than 23dB for 80%, larger than 20dB for 50% cases. According to different beamforming codebook, the TX power difference may further change.

Observation 1: There will be big TX power difference for TX beams in set A.

### 2.1.2 Simplified CDL model

For CDL channel, both single probe and multiple probes are possible.

We understand the intention of discussing the issue is that there is probe limitation in chamber. Number of angles of arrival (AOA) will have impact on probe number in chamber.

For CDL model, there are several clusters and for each cluster, there are 20 rays. In order to reduce the total number of AOA, for each cluster, only one ray is considered.

Additionally, the number of clusters can be further reduced using the following options:

- Angle Offset Threshold:

  - If the AOA offset between two clusters is smaller than a threshold, these clusters can be merged into one. The AOA offset threshold may depend on the angle resolution of the RX antenna and test setup limitations, which can be further discussed in RAN4.

- Power Threshold:

  - Remove clusters with low power. The power threshold can be further discussed.

- Combination of Angle and Power Thresholds:

  - Apply both angle and power thresholds to reduce the number of clusters.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

How to choose cluster for simplified CDL channel needs to be discussed. Here, we compare two simplified CDL-C channels and the possible impact.

Case 1: Select 8 clusters with 3 probes for simplified CDL-C

- 1st probe: cluster {#2,#3,#4}with the same angle

- 2nd probe: cluster{#6,7,8}with the same angle

- 3rd probe: cluster{#1,#5} with small angle offset

Here, 8 cluster are selected due to larger power and same or small angle offset.

| Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

Case 2: Further simplify channel. Select 3 clusters with 1 probe.

  - Here, cluster{#6,7,8} are selected due to large power and the same angle

| Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

For AI model, it tries to learn the spatial relation between L1-RSRP for different TX beams. Therefore, we would like to compare the distribution of L1-RSRP for different TX beams for CDL-C channel and simplified CDL-C channel. However, CDL-C defined in 38.901 only represents a single channel realization. It can’t represent the statistic spatial distribution for many different AOA, AOD, ZOD, ZOA etc.

We generate multiple BS-UE link by CDL-C rotation method defined in 7.7-5 in 38.901. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling.

(7.7-5)

By translation, mean angle can be changed to  and angular spread can be changed by scaling. Here, we didn’t change the angular spread. We only generate multiple random angles with uniform distribution. The 32 TX beam is aligned with simulation assumption agreed in [2].

Then we compare the CDF curve L1-RSRP difference between CDL-C and simplified CDL-C in Fig.2.

(a) Simplified CDL-C with 8 cluster, 3 AOA         (b) Simplified CDL-C with 3 cluster, 1 AOA

Fig.2 CDF curve L1-RSRP difference between CDL-C and simplified CDL-C

Fig.2(a) compares L1-RSRP difference between CDL-C and simplified CDL-C for case 1. Here, there are totally 8 clusters with three AOAs.

From the curve, it shows, with simplified CDL-C channel model, for the best Top-K TX beams,e.g. K=4, the L1-RSRP delta between CDL-C and simplied CDL-C model is small. For example, the L1-RSRP delta between 1st and 2th beam is 1.6dB and 1.74dB for CDL-C and simplified CDL-C for 50% cases. The L1-RSRP delta between 1st and 4th beam is 4.8dB and 5dB for CDL-C and simplified CDL-C for 50% cases. While L1-RSRP delta for some weaker beams will increase. The L1-RSRP delta between 1st and 12th beam is 12dB and 15dB for CDL-C and simplified CDL-C for 50% cases. It means that weaker beams power will decrease more than strong beam. The impact on AI prediction needs further study.

Fig.2(b) compares L1-RSRP difference between CDL-C and simplified CDL-C for case 2. Here, channel model is further simplified to 3 clusters with single AOA. It can be seen that L1-RSRP delta between TX beams increased a lot. For example, the L1-RSRP between 1st and 4th beam increase to 11dB for 50% cases.

Comparing case 1 and case 2, when cluster number is further reduced, L1-RSRP delta between different beams will increase. It’s more easiler for AI to distingwish the best Top-K TX beam. RAN4 may need to discuss how to select AOA number in order to balance test complexity and prediction performance requirement.

Observation 2: For simplified CDL-C model, L1-RSRP of weaker TX beams will decrease more than strong TX beams when compared with CDL-C model. The impact on AI prediction needs further study.

Observation 3: For simplified CDL-C model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 3: If simplified CDL model will be chosen, RAN4 to discuss how to select cluster and AOA number to balance test complexity and prediction performance requirement.

      - 2.2 Measurement error impact

Prediction accuracy degradation due to measurement error

Here, we apply the same measurement error to all TX beams. Where measurement error is modeled as Gaussian distribution.

| Measurement error (dB) | Prediction accuracy % |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| 0 | 76.35 | 93.85 | 98.17 |
| 2 | 53.89 | 74.92 | 92.46 |
| 4 | 48.17 | 66.39 | 86.75 |
| 6 | 42.94 | 63.02 | 82.34 |

Observation 4: There will be performance degradation due to measurement error.

L1-RSRP difference for beams in set B

For 8 TX beams in set B, there will be 8 L1-RSRPs. In order to have rough idea what’s the L1-RSRP difference between TX beams in set B, we calculate the ideal L1-RSRP delta for set B based on SLS channel model. L1-RSRP delta between best 1st beam and 8th beam can be larger than 20dB for 10% and larger than 15dB for 50% cases. There is a great SNR difference for different beams.

Observation 5: For set B, L1-RSRP difference between best 1st beam and 8th beam is large.

SNR difference Vs L1-RSRP difference

The L1-RSRP delta between the best 1st beam and other beam can be calculated in dB. Since the noise power for all TX beams is relatively similar, for simplicity, the L1-RSRP difference can be equal to SNR difference.

For SLS L1-RSRP, without considering impact of inference, the L1-RSRP can be compared with noise power to generate corresponding SNR. The impact of interference can be further studied. It can also

Proposal 4: For simplicity, L1-RSRP delta for different TX beams in set B can be equal to SNR difference for beams in set B.

Measurement error for different SNR

The measurement error for different SNR is simulated as below. Here, we only simulate for AWGN channel. For fading channel, the measurement error may be larger. Besides, measurement error is dependent on baseband algorithm.

Table 2:  Measurement error for AWGN channel

| SNR | -20dB | -15dB | -10dB | -5dB | -3dB | 0dB | 5dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1-RSRP measurement error | 13.5dB | 8.5dB | 4.8dB | 2.2dB | 1.7dB | 1dB | 0.5dB |

Observation 6: Under low SNR conditions, measurement error can be large.

Since different measurement error can be added for different TX beams, in the next step, we will continue to analyze the performance degradation by adding different measurement error.

      - 2.3 Ground truth

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

In last meeting, it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.3 test procedure

As shown in Fig.3, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.3.1 Ground truth for best beam index

If the test metric is beam index prediction accuracy, UE will report the best beam index in set A after measurement. However, as we analysed in performance metric part in [3], UE may not distinguish for the best 1st beam index and 2nd best beam index due to small L1-RSRP delta between the best 1st beam index and 2nd best beam index.

Fig.4 Ideal L1-RSRP difference for 32 TX beams

In Fig.4, we take the CDF curve “RSRP delta between 1st and 2nd best beam” as an example. 10 percentile of CDF curve shows the L1-RSRP delta between 1st and 2nd best beam is smaller than 0.16dB for 10% cases. If UE can successfully distinguish between the 1st and 2nd beams in 90% case, the measurement accuracy has to be within 0.16dB.

Therefore, UE may need to report for N best beam index. RAN4 can further discuss how to decide N.

Proposal 5: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A. RAN4 to discuss how to decide number N.

### 2.3.2 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 7: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 6: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.3.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 7: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 8: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.3.4 Multiple-AoA test setup impact

For multiple-AOA test setup, as pointed early by TE vendor, multiple directions might be addressed by the DUT with different antennas, so that RF baseline system assumptions are not valid and FF-QZ-MU configuration needs further investigation. it’s more challenging to obtain accurate RSRP measurement as ground truth. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Proposal 9: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

### 2.3.5 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 8: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.5(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.5(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.5 (a) beam index and RS index mapping relation for set A in T1

Fig.5 (b) beam index and RS index mapping relation for set A for T2

Proposal 10: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For TDL channel model, RAN4 needs to discuss how to generate different TX power for beams in set B or set A.

Observation 1: There will be big TX power difference for TX beams in set A.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Observation 2: For simplified CDL-C model, L1-RSRP of weaker TX beams will decrease more than strong TX beams when compared with CDL-C model. The impact on AI prediction needs further study.

Observation 3: For simplified CDL-C model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 3: If simplified CDL model will be chosen, RAN4 to discuss how to select cluster and AOA number to balance test complexity and prediction performance requirement.

Observation 4: There will be performance degradation due to measurement error.

Observation 5: For set B, L1-RSRP difference between best 1st beam and 8th beam is large.

Proposal 4: For simplicity, L1-RSRP delta for different TX beams in set B can be equal to SNR difference for beams in set B.

Observation 6: Under low SNR conditions, measurement error can be large.

Proposal 5: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A. RAN4 to discuss how to decide number N.

Observation 7: For deriving ground truth of set A, SNR level should satisfy that for at least best M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 6: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 7: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 8: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Proposal 9: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Observation 8: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 10: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2417211, Simulation assumptions for beam prediction

- R4-2417949, Discussion on core part requirement for AI beam management, Xiaomi