3GPP TSG-RAN WG4 Meeting #113																	 R4-2417957 TS38.133 [4]

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.3.4

Title: 	Discussion on RRM requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for enhanced requirements and test methodology for NR and IoT NTN was agreed and one of objectives for less than 5MHz for NTN is self-contained as below.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

And in the last RAN4 meeting, the agreements for less than 5MHz for NTN was achieved in [2, R4-2416851]. Therefore, in this discussion we provided further considerations on the open issues for less than 5MHz NTN.

- Discussion

According to WF agreed in the last RAN4 meeting [2], the RRM requirements impacted for less than 5MHz NTN was summarized below.

- Handover

- RLM

- Measurement

In order to effectively coordinate the further draftCR works, we also suggest the CR splitting works in the table below.

Table 1. RRM impacts summary due to spectrum less than 5MHz

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Volunteer company |
| --- | --- | --- |
| Handover | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | Company 1 |
|  | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover | Company 2 |
|  | 6.1C.3 NR SAN Satellite switching with re-synchronization | No need CR the updates of T∆ is same as the one defined in section 6.1C.2.2.2.1. |
| RLM | 8.1C Radio Link Monitoring for Satellite Access | Company 3 |
|  | 8.5C Link Recovery Procedures for Satellite Access | Company 4 |
| Measurement | 9.2C NR intra-frequency measurements for SAN | Company 5 |
|  | 9.3C NR inter-frequency measurements for SAN | Company 6 |

The main open issues for these RRM requirements are

- CHO delay

- requirements for SSB time index detection

- Handover

In the last RAN4 meeting, the general principle to update the handover requirements for less than 5MHz NTN was agreed as:

Issue 1-2-1-1 HO delay Agreements - T∆ in HO requirement for NTN in exiting TS38.133 clause 6.1C.1.2. shall be extended to [3 Trs] Issue 1-2-1-2 CHO delay Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

More especially, the RRM requirements for NTN conditional handover was defined in TS38.133 6.1C.2 [4]. In comparison with the normal handover, beside Tinterrupt the other one important factor which could be impacted by the punctured SSBs in case of less than 5MHz NTN is Tmeasure since L3 measurement is also based on SSB.

6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR SAN FR1 cell to NR SAN FR1 cell. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during DCHO, otherwise the measurement time, preparation time and interruption time may be longer than the requirements in clause 6.1C.2.2.2, 6.1C.2.2.3 and 6.1C.2.2.4. 6.1C.2.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. UE shall start RRM measurement before the time or distance condition is met, the time/distance condition is defined in clause 5.5.4 in TS 38.331[2] When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel or PUSCH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: - TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover - Tmeasure is the measurements time stated in clause 6.1C.4.2.2. - TCHO_execution is the UE conditional execution preparation time for conditional handover in clause 6.1C.2.2.3. - Tinterrupt is the interruption time stated in clause 6.1C.2.2.4. The conditional handover delay requirements are applied if condition T1-2 is later than the end of Tmeasure for time based CHO, or both condition D1-1 and condition D1-2 are fulfilled before the end of Tmeasure for location-based CHO, otherwise no CHO requirement is applied.

As in the last RAN4 meeting, how to extend Tinterrupt for the less than 5MHz NTN normal handover was agreed. Regarding the Tinterrupt for NTN conditional HO procedure is same as that of NTN handover defined in TS38.133 6.1C.1.2.2. [4], it is reasonable to reuse the same way to extend Tinterrupt for the conditional less than 5MHz NTN handover.

Proposal 1: Tinterrupt for the less than 5MHz NTN conditional handover can be defined as

- T∆ in CHO requirement for NTN in exiting TS38.133 is extended to [3 Trs]

The other important factors Tmeasure relies on PSS/SSS/PBCH detection performance also which may depend on reference signal bandwidth. In the last meeting some companies prefer to leverage the requirement for time period for time index detection in the less than 5MHz NTN measurement period requirements. In our view, this is feasible and beneficial to avoid some repeated discussion.

Observation 1: The relaxation on Tmeasure requirement within in less than 5MHz NTN conditional handover can be same as that of measurement period requirement defined in TS38.133 [4] 9.2C.5/9.3C.5 .

Proposal 2: The relaxed Tmeasure for less than 5MHz NTN conditional handover requirements can be leveraged from the same one in Tidentify_intra_with_index and Tidentify_inter_with_index defined in TS38.133 [4] 9.2C.5/9.3C.5.

- Measurement reporting period

Issue 1-2-4-2: How to define the new requirements for SSB time index detection Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

Regarding to the same SSB punctured used and same SINR side conditions, for less than 5MHz NTN, the performance degradation due to the less PSS/SSS/PBCH could be same as these in case TN less than 5MHz. Therefore, we can suggest that:

Proposal 3: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- Conclusion

In this contribution, serval issues related to the RRM requirements under less than 5MHz NTN in Rel19 are discussed. The observations and proposals can be summarized as:

Proposal 1: Tinterrupt for the less than 5MHz NTN conditional handover can be defined as

- T∆ in CHO requirement for NTN in exiting TS38.133 is extended to [3 Trs].

Observation 1: The relaxation on Tmeasure requirement within in less than 5MHz NTN conditional handover can be same as that of measurement period requirement defined in TS38.133 [4] 9.2C.5/9.3C.5 .

Proposal 2: The relaxed Tmeasure for less than 5MHz NTN conditional handover requirements can be leveraged from the same one in Tidentify_intra_with_index and Tidentify_inter_with_index defined in TS38.133 [4] 9.2C.5/9.3C.5.

Proposal 3: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- References

- RP-240857

- R4-2416851

- R4-2413528

- TS38.133 v18.7.0

- TS38.211 v18.3.0