3GPP TSG-RAN WG4 Meeting #113																	 R4-2417960

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.15.2.1

Title: 	Discussion on FR2-1 SSB based L3 measurement delay reduction by Rx beam

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NR Radio Resource Management (RRM) Phase 5 was agreed and the objectives are self-contained as below.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

And in the last RAN4 meeting, there were some discussions for the open issues [2, R4-2416853]. In this contribution we provide more considerations on the possible enhancement to reduce the SSB based L3 measurement delay.

- Discussion

In RAN4#112 meeting, RAN4 agreed that the scenarios which are considered to use L3 measurement delay reduction by optimizing Rx BSF.

Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

In the last meeting, how to define the conditions to trigger UE to activate L3 measurement delay reduction by optimizing RX BSF was discussed.

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656.

According to the agreements above, UE can autonomously activate or deactivate FBS based on the specific criteria/conditions.

Observation 1:  UE can autonomously activate or deactivate FBS to reduce L3 measurements based on the specific criteria/conditions

And the possible conditions/criteria are proposed in RAN4 last meeting also. E.g.

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

At same time, another important aspect needed to be clarified is DRX configuration, which is one of most efficient way to reduce UE’s power consumption in current practical deployments. In case of measurement under DRX, the power consumption and the measurement delay itself shall be more carefully traded-off. Thus we suggest to take  DRX as one of conditions for FBS (de)activation.

More especially, the mobility status can be also implied by DRX configuration in the practical deployments. For an example, in the stationary status the longer DRX can be allowed.

Additionally, in our view, UE location information is unnecessary. UE can roughly estimate its location and distance away the gNB based on the recently measurement RSRP/SINR.

Observation 2: The conditions proposed to activate or deactivate FBS to reduce L3 measurements (e.g. UE mobility, UE location) needs more extra NW signaling.

Therefore, we suggested that:

Proposal 1: Activating/Deactivating FBS for L3 measurements (intra/inter-frequency) can upon the conditions without additional NW signalling (e.g. DRX configuration).

Secondly, we provided our solution on the BSF for scenario 1/2/3/4. For the individual part of L3 measurement, beam sweeping factor is assumed to be same. For an example, in current TS38.133 [3] as shown below 8 for FR2-1 power class 1 or 5 in the L3 cell synchronization and measurement.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

However, theoretically in case of UE acquired some rough timing with PSS/SSS synchronization, UE may complete the further measurement or SSB index decoding from the initial beam sweeping status. That is UE can do measurement within the less candidate beams.

Figure 1. Reduced Rx beam sweeping after PSS/SSS detection

Proposal 2: In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection.

- Conclusion

In this contribution, serval issues related toFR2-1 SSB based L3 measurement delay reduction for connected mode in Rel19 are discussed. The observations and proposals can be summarized as:

Observation 1:  UE can autonomously activate or deactivate FBS to reduce L3 measurements based on the specific criteria/conditions

Observation 2: The conditions proposed to activate or deactivate FBS to reduce L3 measurements (e.g. UE mobility, UE location) needs more extra NW signaling.

Proposal 1: Activating/Deactivating FBS for L3 measurements (intra/inter-frequency) can upon the conditions without additional NW signalling (e.g. DRX configuration).

Proposal 2: In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection.

- References

- RP-240830

- R4-2416853

- TS38.133 v18.5.0

- TS38.331 v17.1.0