3GPP TSG-RAN WG4 Meeting #113	R4-2417972

Orlando, United States, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	(NR_newRAT-Perf) Discussion on FRC corrections to maximum throughput

Agenda item:	4.6

Document for:	Discussion

Introduction

In RAN4#112 Nokia introduced a CR for changes to existing FRCs (see [2]). During the discussions in RAN4#112 it was brought up if normal “round()” or “floor()” should be used when defining the values for “Max. Throughput averaged over x frames”. It was finally agreed in the CR [2] to use “floor()” of the values to ensure the values would not show an impossible to reach maximum in case they were rounded up (i.e. the value of 91,007,600bps was in the CR written as 91.007Mbps using “floor()” instead of 91.008Mbps using “round()”)

At the same time Nokia indicated that since the CR was agreed and there was consensus to use “floor”, we would check the existing FRCs and provide corrections so “floor()” would be used for all FRCs where “round()” were previously used.

Discussion

As there are a great many FRCs defined in 38.101-4 [1], we have started by first looking at all the FRCs related to FDD test cases. We have verified the values in the specification [1] and located the FRCs where “round()” was used and the value was rounded up, which effectively gives an indication of a higher possible max throughput than is practically possible.

We see two ways to potentially solve the problem:

- Introduce values for “Max. Throughput averaged over x frames” using the full “non-rounded/floored” value. We have observed until now, that Max. throughput values will always end with “00”, hence result in values with 4 decimals (Max throughput is given in Mbps).

- Introduce “floor()” to 3 decimals (Max throughput is given in Mbps).

One example we have found, which is also included in our CRs [3], [4], [5] and [6]:

R.PDSCH.1-1.1 FDD:

- Calculated Max. throughput is: 3,708,800 (bps) -> 3.7088 Mbps

- Current value in 38.101-4 is: 3.709Mbps (i.e., rounded up).

The change for this specific case which is represented in the CRs looks like this when “floor()” is used:

“Round()” has been used in several FRCs value “Max. Throughput averaged over x frames”, which in some cases result in maximum throughput values that are higher than practically possible. For some FRCs no rounding was done (i.e. full value is kept).
We see two ways to solve the problem. Either keep the exact maximum throughput values or use “floor” to 3 decimals (Mbps) instead of “round” to 3 decimals.

Correct to use “floor()” of the “Max. Throughput averaged over x frames” parameters in the FRCs, where rounding has resulted in a higher throughput that is practically possible.

For future FRCs always use ”floor()” with 3 decimals (Mbps) when defining the “Max. Throughput averaged over x frames” parameters.

We have provided CRs (see [3], [4], [5] and [6]) for updates to FRCs for FDD cases in this meeting and will provide CRs for further updates on TDD and other cases in the following meetings. Our CRs are correcting the values where “round()” have been used and the value is rounded up to using “floor()” instead for securing 3 decimals only.
If not agreed in RAN4#113 to use “floor()” and instead to not use any reduction, we can change the CR to use the non-reduced values instead for the tables which has been identified having issues (i.e., we will not change all tables)

We have provided CRs for updates to FRCs for FDD cases in this meeting correcting the values where “round()” have been used and the value is rounded up to using “floor()” instead. We plan to provide additional CRs for further updates on TDD and other cases in the following meetings.

Conclusion

In this paper we have provided our view on the topic of requirement definition for 8Rx inter-cell and inter-cell inter-user requirements.

The following Observations and Proposals were made:

- “Round()” has been used in several FRCs value “Max. Throughput averaged over x frames”, which in some cases result in maximum throughput values that are higher than practically possible. For some FRCs no rounding was done (i.e. full value is kept).
We see two ways to solve the problem. Either keep the exact maximum throughput values or use “floor” to 3 decimals (Mbps) instead of “round” to 3 decimals.

- Correct to use “floor()” of the “Max. Throughput averaged over x frames” parameters in the FRCs, where rounding has resulted in a higher throughput that is practically possible.

For future FRCs always use ”floor()” with 3 decimals (Mbps) when defining the “Max. Throughput averaged over x frames” parameters.

We have provided CRs for updates to FRCs for FDD cases in this meeting correcting the values where “round()” have been used and the value is rounded up to using “floor()” instead. We plan to provide additional CRs for further updates on TDD and other cases in the following meetings.

- References

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- R4-2411665 - CR for 38.101-4 on RMC corrections for MultiRx requirements

- R4-2417968 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417969 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417970 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417971 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections