3GPP TSG-RAN4 Meeting #113	 R4-2417980

# Orlando, USA, 18th – 22th November, 2024

Source: 	Huawei, Hisilicon

Title: 	TP to TR38.769 Co-existence simulation

Agenda Item:	7.21.2.2

Document for:	Approval

Introduction

In the last two meetings, the formal co-existence simulation assumptions for Rel-19 Ambient IoT were approved in [1][2]. Companies provided the co-existence evaluation results this meeting.

In this contribution, we provide the draft TP to TR 38.769 for Co-existence evaluation based on [3][4].

Reference

[1] R4-2414304, WF on Co-existence study for ambient IoT, CMCC, RAN4#112, August 2024

[2] R4-2417196, WF on co-existence study for A-IOT, CMCC, RAN4#112bis, October 2024

[3] R4-2414306, TP to TR38.769 for Co-existence evaluation assumptions, Huawei, Hisilicon, RAN4#112, August 2024

[4] R4-2416356, TP to TR38.769 Co-existence simulation, Huawei, Hisilicon, RAN4#112bis, October 2024

Text Proposal to TR 38.769

------------Start of the Change-------------

### 6.6.2	Co-existence scenarios and cases

The coexistence evaluation is conducted considering the different scenarios listed in Table 6.6.2-1

Table 6.6.2-1: Co-existence scenarios

| Deployment scenario No. (Case No.) | Topology | Spectrum |
| --- | --- | --- |
| 1-1(a/b/c/d) | D1T1-A2- NR UE only outdoor | R2D: DL CW2D and D2R: DL |
| 1-2(a/b/c/d) | D1T1-A2- NR UE indoor | R2D: DL CW2D and D2R: DL |
| 2-1(e/f/c/d) | D1T1-B- NR UE only outdoor | R2D: DL CW2D and D2R: UL |
| 2-2(e/f/c/d) | D1T1-B- NR UE indoor | R2D: DL CW2D and D2R: UL |
| 3 (Optional)(a/b/g/h) | D1T1-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 4 (Optional)(e/f/g/h) | D1T1-B- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 5-1(e/f/g/h) | D2T2-A2- NR UE only outdoor | R2D: UL CW2D and D2R: UL |
| 5-2(e/f/g/h) | D2T2-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 6-1(Optional)(a/b/g/h) | D2T2-B- NR UE only outdoor | R2D: UL CW2D and D2R: DL |
| 6-2(Optional)(a/b/g/h) | D2T2-B- NR UE indoor | R2D: UL CW2D and D2R: DL |

The main co-existence scenario considered for ambient IoT is D1T1 and D2T2. The deployment parameters are described in Table 6.6.3.1-1.

The co-existence evaluation captures cases where NR and A-IoT (Reader/device) are both victim and aggressor networks. It is to evaluate impact on legacy NR networks if A-IoT is introduced in indoor scenario, also to understand impact of the legacy NR network on A-IoT system. The co-existence cases are listed in Table 6.6.2-2.

Table 6.6.2-2: Co-existence cases

| Case No. | Aggressor | Victim | note |
| --- | --- | --- | --- |
| a | Device | NR DL | D2R |
| b | NR DL | reader | D2R |
| c | Reader | NR DL | R2D |
| d | NR DL | device | R2D |
| e | Device | NR UL | D2R |
| f | NR UL | reader | D2R |
| g | Reader | NR UL | R2D |
| h | NR UL | device | R2D |

------------Next Change-------------

### 6.6.4	Co-existence simulation methodology

#### 6.6.4.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling.

3)	Once association is done, round robin scheduling is used.

4) For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

-	If SINR degradation is larger than 1 dB, consider the criteria: Outage percentage consider SINR level with [10%] BLER

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

5) For intra-system interference (between A-IoT and A-IoT): Outage percentage consider SINR level with [10%] BLER

6) How to derive the outage percentage value/range:

Select 3 SINR values for D2R and R2D respectively according to following steps:

-	Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary)

-	Set the SNR/CNR range by removing the highest and lowest value from the set in step 1

-	Pick 3 values from the range in step 2, e.g. highest, lowest, middle

Note: R2D CNR needs to be transformed to SINR

# 6.5.4.2. SINR definition

For D1T1/D2T2-A1and -B

SINR definition for D2R:

- SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference+residual CW interference after cancellation

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intra_system interference + inter_system interference+residual CW interference after cancellationThe noise and intra-system interference are within total receiver bandwidth, and the residual CW interference after cancellation is in linear scale.

SINR definition for R2D:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider [180KHz] noise and interference bandwidth after BB LPF as optional

------------Next Change-------------

### 6.6.6	Summary of co-existence evaluation

The sub-clause captures the co-existence simulation results for ambient IoT categorized by scenarios and cases. The conclusions below are derived from the coexistence results.

# 6.6.6.1 Scenario 1-1: D1T1-A2- NR UE only outdoors (R2D/ CW2D/ D2R: DL)

Scenario 1-1 considers D1T1 topology, all the NR UE outdoors and Readers/CW collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.1-1.

Table 6.5.6.1-1: Scenario 1-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader |  |
| c | Reader | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

# 6.6.6.2 Scenario 1-2: D1T1-A2- NR UE indoor (R2D/ CW2D/ D2R: DL)

Scenario 1-2 considers D1T1 topology, 10% of the NR UE indoors and Reader/CW collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.2-1.

Table 6.5.6.2-1: Scenario 1-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader |  |
| c | Reader | NR DL |  |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

# 6.6.6.3 Scenario 2-1: D1T1-B- NR UE only outdoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-1 considers D1T1 topology, all the NR UE outdoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.3-1.

Table 6.5.6.3-1: Scenario 2-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader |  |
| c | Reader | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

# 6.6.6.4 Scenario 2-2: D1T1-B- NR UE indoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-2 considers D1T1 topology, 10% of the NR UE indoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.4-1.

Table 6.5.6.4-1: Scenario 2-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader |  |
| c | Reader | NR DL |  |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

# 6.6.6.5 Scenario 5-1: D2T2-A2- NR UE only outdoor (R2D/ CW2D/ D2R: UL)

Scenario 5-1 considers D2T2 topology, all the NR UE outdoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.5-1.

Table 6.5.6.5-1: Scenario 5-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
| f | NR UL | reader | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| g | Reader | NR UL |  |
| h | NR UL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

# 6.6.6.6 Scenario 5-2: D2T2-A2- NR UE indoor (R2D/ CW2D/ D2R: UL)

Scenario 5-2 considers D2T2 topology, 10% of the NR UE indoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.6-1.

Table 6.5.6.6-1: Scenario 5-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
| f | NR UL | reader |  |
| g | Reader | NR UL |  |
| h | NR UL | device |  |

# 6.6.6.7 Scenario 6-1: D2T2-B- NR UE only outdoor (R2D: UL, CW2D/ D2R: DL)

Scenario 6-1 considers D2T2 topology, all the NR UE outdoors and Readers/CW non-collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.7-1.

Table 6.5.6.7-1: Scenario 6-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL |  |
| b | NR DL | reader |  |
| g | Reader | NR UL |  |
| h | NR UL | device |  |

# 6.6.6.8 Scenario 6-2: D2T2-B- NR UE indoor (R2D: UL, CW2D/ D2R: DL)

Scenario 6-2 considers D2T2 topology, 10% of the NR UE indoors and Readers/CW non-collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.8-1.

Table 6.5.6.8-1: Scenario 6-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL |  |
| b | NR DL | reader |  |
| g | Reader | NR UL |  |
| h | NR UL | device |  |

# 6.6.6.9 Scenario 3 (Optional): D1T1-A2- NR UE indoor (R2D: UL, CW2D/ D2R: DL)

Scenario 3 optionally considers D1T1 topology, 10% of the NR UE indoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.9-1.

Table 6.5.6.9-1: Scenario 3 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL |  |
| b | NR DL | reader |  |
| g | Reader | NR UL |  |
| h | NR UL | device |  |

# 6.6.6.10 Scenario 4 (Optional): D1T1-B- NR UE indoor (R2D/ CW2D/ D2R: UL)

Scenario 4 optionally considers D1T1 topology, 10% of the NR UE indoors and Readers/CW non-collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.5.6.10-1.

Table 6.5.6.10-1: Scenario 4 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL |  |
| f | NR UL | reader |  |
| g | Reader | NR UL |  |
| h | NR UL | device |  |

------------End of the Change-------------