3GPP TSG-RAN WG4 Meeting #113	R4-2418000

Orlando, USA, 18-22 Nov 2024

Title: 	On Power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In previous RAN4 meeting power domain enhancenemts topic approved WF[1], this contribution discussed open issues of  Power domain enhancements for single carrier.

# 2	Discussion

## 2.1	Boundary to apply ACLR and SEM

From WF[1]

#### Issue 1-2-4: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

  - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

We have been so far advocation the proposal 2 i.e. ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. However, we have not been against Proposal 1 either as proposal 2 is one and also the most important scenario of proposal 1 [2]. Let’s us further explain with a figure 1.

In figure 1 we present three cases. Case 1 is in our view the typical scenario where BS CBW and Extended UE CBW are the same. Case 2 represents a scenario where for some reason UE emissions outside BS CBW are reduced symmetrically on both lower and higher side of BS CBW. Case 3 is a scenario where there is a protected frequency range on only one side of the BS CBW and in this case that frequency range is on higher side of BS CBW. In case 3 UE extended CBW is placed on lower side of BS CBW and the width of extended UE CBW is adjusted to be such that emission towards protected frequency range are suppressed to satisfactory level.

To be able to adjust the placement and width of UE extended CBW within a BS CBW new signaling is required.

Proposal 1: RAN4 shall use term extended UE channel bandwith when discussing power domain enhancement for single carrier. Width of Extended UE channel bandwith can be up to basestation channel bandwidth but no more or it can be narrower than BS CBW. Placement of Extended UE CBW within BS CBW can be chosen freely. Extended UE channel bandwidth sets the boundary to apply ACLR and SEM.

Figure 1: Comparison of Extended UE CBW and BS CBW

## 2.2	Boundary to apply SE  and SM 329 from ITU with 250% rule

From WF[1]

#### Issue 1-2-5: Boundary to apply SE

- Proposals

  - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW.

  - Proposal 2: SE is applied at BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

In our view neither proposal 1 nor 2 is fully correct. In our view spurious emissions boundary should be based on Extended UE CBW and follows current RAN4 rule as in table 6.5.3.1-1 from 38.101-1.

Observation 1: Spurious emissions boundary should be based on Extended UE CBW and follows current RAN4 rule as in table 6.5.3.1-1 from 38.101-1.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 |

In other words boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context should be defined as Extended UE CBW + 5 MHz.

There was an issue raised on previous RAN4 meeting that analysis is needed for ITU regulation of 250% necessary bandwidth. This requiremet comes from SM 329 [3].

Quote from SM 329 [3]

2.3 According to the principles stated in Appendix 3 to the RR, the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission. However, this frequency separation may be dependent on the type of modulation used, the maximum bit rate in the case of digital modulation, the type of transmitter, and frequency coordination factors. For example, in the case of some digital, broadband or pulse-modulated systems, the frequency separation may need to differ from the ±250% factor. As the RR forbid any radio service to cause harmful interference outside its allocated band, transmitter frequencies should be determined so that out-of-band emissions do not cause harmful interference outside the allocated band in accordance with RR No. 4.5. Alternatively, the ±250% may apply to channel separation instead of the necessary bandwidth. As an example, for frequency coordination of the digital fixed service, Recommendation ITU-R F.1191 recommends the use of ±250% of the channel separation of the relevant radio-frequency channel arrangement as frequency boundaries between the out-of-band and spurious domains.

If we follow the ITU guidance of the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission and assume that necessary bandwidth is same as UE channel bandwidth or in this context Extended UE CBW we can see that 3GPP FOOB is always more stringent that ITU recommendation, see table 2.

Observation 2: 3GPP definition for spurious emissions domain will be more stringent than ITU recommendation if FOOB is defined as Extended UE CBW + 5 MHz

Table 2: Comparison of 3GPP FOOB  and ITU 250% rule

| Extended UE CBW (MHz) | 3GPP FOOB (MHz) | ITU 250% (MHz) |
| --- | --- | --- |
| 100 | 105 | 200 |
| 90 | 95 | 180 |
| 80 | 85 | 160 |
| 70 | 75 | 140 |
| 60 | 65 | 120 |
| 50 | 55 | 100 |
| 40 | 45 | 80 |
| 30 | 35 | 60 |
| 20 | 25 | 40 |
| 10 | 15 | 20 |

Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz.

# 3	Conclusion

In this contribution we have made following proposals

Proposal 1: RAN4 shall use term extended UE channel bandwith when discussing power domain enhancement for single carrier. Width of Extended UE channel bandwith can be up to basestation channel bandwidth but no more or it can be narrower than BS CBW. Placement of Extended UE CBW within BS CBW can be chosen freely. Extended UE channel bandwidth sets the boundary to apply ACLR and SEM.

Observation 1: Spurious emissions boundary should be based on Extended UE CBW and follows current RAN4 rule as in table 6.5.3.1-1 from 38.101-1.

Observation 2: 3GPP definition for spurious emissions domain will be more stringent than ITU recommendation if FOOB is defined as Extended UE CBW + 5 MHz

Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz.

# 4	References

[1] R4-2417204, WF on power domain enhancements, RAN4#112bis

[2] R4-2415369, Power domain enhancements for single carrier, Nokia, RAN4#112bis

[3] RECOMMENDATION ITU-R SM.329-10