3GPP TSG RAN WG4 #113	R4-2418022

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.20.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the existing BS RF TX requirements for SBFD operation.

# Discussion

## ACLR and OBUE

The following was agreed in RAN4#112bis:

WF: - FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

We do not see the necessity of additional ACLR and/or OBUE would be needed from 3GPP to define. The described scenario where two operators SBFD capable gNB’s would coexist, is a new scenario and a rare one. It has not been discussed in the study item phase and should not be included here. Thus, we believe that there is no need to define additional 3GPP requirements for this special case. Customer specific proprietary requirement can be used in such case where two SBFD systems are co-existing and reasonable co-location is needed. If we take a look on the proposal on how to define the additional co-existence requirements, there are many things that would make it almost impossible to agree on. Typical CL values according to different companies are ranging excessively and there is no consensus what a typical SBFD-capable gNB implementation would be. Therefore, it would be practically impossible to define the needed improvements to related requirements.

No need to specify additional ACLR and/or OBUE requirements.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our proposal is listed below.

- No need to specify additional ACLR and/or OBUE requirements.

# References

- R4-2416585 “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, RAN4#113, Hefei, China, 14th-18th October, 2024