3GPP TSG-RAN WG4 Meeting #113		 					R4-2418031

Orlando, US, 18th – 22nd November, 2024
 
Agenda Item:  		7.9.4
Source: 			Intelsat, Fraunhofer. BMW 
Title: 			Characterisation of Ku Band Mobile VSAT Antennas for

GSO and NGSO Applications

Document for: 		Discussion and Decision

# Introduction

In this contribution we provide definition of Earth Stations in Motion (ESIM) also known as Mobile Very Small Aperture Terminals (VSAT) and characteristics of three main type of Mobile VSATs that are used in the Satellites communication. Furthermore, we introduce three types of array antenna models for the Ku Band Standardisation work.

Mobile VSATs, which include terminals on stations buildings, on aircraft, ships, and land vehicles, facilitate high-bandwidth, reliable connections to moving or stationary satellite systems. These terminals have evolved from the traditional large parabolic dish antennas to electronically steered advance devices capable of tracking multiple satellites.

Mobile VSATs constitute to a large category of terminals which are the workhorse of the satellite communication industry and provide connectivity to GEO and to NGSO Satellite constellations.

In general, the Mobile VSATs can be categorised into three main types:

- Land Mobile VSATs

- Maritime Mobile VSATs and

- Aeronautical Mobile VSATs

The above mentions Mobile VSATs have different antenna sizes with differing radiation patterns, and therefore it will be a lengthy and complicated exercise to use their antenna characteristics for standardisation process.

To streamline the standardisation process and to provide a baseline model of their antenna characteristics, we provide three antennas as baseline models for the VSATs.

These proposed antennas are in addition to the five UE terminal types as specified in TS 38.101-5 for Ka-band, which are also considered in the Ku-band WID [RP-241690].  Please also see section  7.3.

Observation 1: Mobile VSAT terminals for NGSO connectivity are not yet specified by 3GPP, neither in the Ka-band nor in the Ku-band.

The two large and medium models introduced in this contribution are based on empirical data gathered from a variety of Mobile VSAT terminals currently in use. Moreover, the small model is a new conceptual antenna.

The intention is to use these three models for the Ku Band standardisation work for connectivity to GSO and NGSO satellites.

# Definition of Mobile VSAT

Mobile VSATs and ESIMs are essentially the same. Section 7 provides CEPT and ETSI references on the earth station in motion (ESIM) use in the Ku-band and ITU Definitions.

# Use Cases of Mobile VSATs

All modern Mobile VSATs are equipped with Electronically Steered Array (ESA) antennas which are compatible with both Geostationary (GEO) and Low-Earth Orbit (LEO) constellations, ESA delivers reliability and consistency along with low latency so end users can text, browse or stream, wherever they are. Redundant GEO capacity makes it ideal for routes over hub cities, while the LEO network offers low latency and polar coverage.

# Types of Mobile VSATs

All modern Mobile VSATs are equipped with Electronically Steered Array (ESA) antennas which allows connectivity with both GEO and multi-orbit NGSO satellite constitution. Modern VSATs are equipped with ESA Antennas which much are lighter than traditional parabolic antennas and offer no moving parts. Furthermore, the low profile of the ESA offers best aerodynamic design with lower air drag.

In the proceeding section we offer a brief introduction of various modern VSATs.

## Land VSAT

For sparsely populate areas, and for regions where terrestrial coverage is difficult, land based VSATs, offer a complementary connectivity solution. These terminals which are static have electronically tuned antennas which point and track GSO and NGSO satellites, respectively.

This land based Mobile VSATs, can also be housed on trucks and other moving vehicles and can provide additional internet connectivity when the vehicle is out of terrestrial coverage.

Furthermore, these devices provide local connectivity to various devices through Wi-Fi or any other type of connection.

Some use cases are as follows:

- Data connectivity to remote regions and communities e.g., remote farmlands, national parks, offshore oil platforms, observation posts etc.

- Supplementary coverage to terrestrial coverage where additional capacity and or extended coverage is required.

- Supplementary data coverage for vehicles and Emergency First Responder e.g., Police, Ambulance, Fire qn Disaster Recovery

- Data coverage for TV and Radio field Broadcast Vehicles

- IoT connectivity to static and moving assets e.g., Farming vehicles,

- Connectivity to long-haul coaches

- Complementary coverage for cars and vehicles normally connected via a terrestrial network.

See following Figure 1:

Figure 1: Land VSAT.

## Maritime Mobile VSAT

A Maritime Mobile VSAT is a two-way satellite internet terminal that receives and transmits real-time data via satellites. Maritime Mobile VSAT refers to gyroscopically stabilized maritime antenna mounted on a vessel. The gyroscopic mount enables the antenna to track and maintain a precise aim to the desired satellite. The antenna are protected by a fibreglass radome, which prevents satellite interference from wind and other harsh environmental conditions.

Furthermore, with electronical steering, they provide connectivity to GSO and NGSO Satellite constellations.

Some of the use cases of the Maritime Mobile VSAT is as follows:

- Data connectivity to cruise liners and yachts

- Data connectivity to cargo and container ships

- Data connectivity to Marine vessels, fishing boat /trawlers and rescue boats

- Fleet monitoring and tracking

- IoT Connectivity to containers on cargo/container ships

Following illustrates a modern electronically steered Maritime Mobile VSAT

Figure 2: Electronically Steered Maritime Mobile VSAT.

## Aeronautical Mobile VSAT

Aeronautical Mobile VSATs are low profile flat panel antennas which are mounted on the top of aircrafts. These flat panel antennas, with their electronically steering, provide connectivity with GSO and NGSO satellite constellations. These Mobile VSATs enabling pilots, crew, and ground personnel to access critical information instantaneously as well as providing data services to onboard passengers. This connectivity extends beyond mere communication; it underpins essential functions like weather updates, navigation, traffic management, maintenance diagnostics, and operational logistics.

A typical Aeronautical Mobile VSAT deployment is illustrated in the following Figure 3

Figure 3: A typical Aeronautical Mobile VSAT on an Aircraft.

## Vehicular VSAT

With the growing need for Vehicular connectivity as identified by 5GAA in a recently published technical report [2] , either in the shape of vehicle to vehicle or vehicle to the network, the Non-Terrestrial Network offers the most practical choice for supplementary coverage solution for these types of services.

To date several satellite connectivity have been proposed over L-, S-, and the Ka-Bands with corresponding antenna characteristics. However, the rich services and wide satellite coverage currently provided by the Ku Band, offers a more an accessible solution.

See following Figure 4

Figure 4: A typical Vehicular Mobile VSAT mounted on top of a car.

The implementation of Vehicular VSAT and connectivity with satellite systems, offers seamless mobility between the Terrestrial and Non-Terrestrial Networks.

Proposal 1: With the increasing interest in the Land, Maritime, Aeronautical and Vehicular sectors of the industry, Mobile VSAT terminals for NGSO should be taken in the Rel-19 work plan for the Ku-band

# Mobile VSAT Antenna Models

To date, Mobile VSATs have not been properly introduced in the 3GPP due to large assortment of performances, antenna form-factors, and radiation patterns./

The large variations of Mobile VSATs builds makes the standardisation process difficult due to large number of antenna characteristics.

To provide a baseline model of the Mobile VSAT terminals, and to streamline the standardisation activities, we propose three antennas models that reflect the performance and characteristics of combined Land, Maritime, Aeronautical and Vehicular Mobile VSATs.

The first two i.e. 60cm x 60cm and 40cm x 40cm antenna models, presented in this document are based on empirical values gathered from a variety of Mobile VSAT terminals already deployed in the Ku-Band. Furthermore, the antenna models presented for the 20cm x 20cm reflect a new conceptual approach.

For convenience, for these three antenna models, the essential characteristics which are generally used in the standardisation work are defined in one place.

## Three Antenna Types

From the empirical data gathered, the most widely used mobile VSAT antennas can be categorised into two categories, large and medium terminals with the following physical dimensions:

Typical Physical sizes

- Large Array 60cm x 60cm

- Medium Array 40cm x 40cm

And for practical Vehicular applications, we introduce a small form factor antenna:

- Small Array 20cm x 20cm

See following Figure 5. The number of patches provided below for the small, medium, and large array are notional – the exact number of patches per array are provided in the next section for Ku-band.

Figure 5: Large, Medium, and Small Arrays form factors used in this study.

## Electrical Characteristics of the Antenna Arrays

Following several iterations and optimisation process, three generic, practical antenna models were developed. The radiation patterns of the three antennas were also double checked with another internal software tool based on Method of Moments and we have high confidence in the radiation pattern values presented.

The simulation operating frequency for the downlink was selected to be at 14.25 GHz.

In all the proceeding simulations we have assumed realistic and practical values which are used in the industry.

### 20cm x 20 cm Antenna

The conceptual 20cm x 20cm Array antenna was simulated and following several iterations was optimised to an 9x9 element array. See following Figure 6.

Figure 6: 20cm x 20cm Array antenna Model.

The radiation plot (or the antenna gain plot) of the 20cm x 20cm on the left indicate one dominant beam with diminishing sidelobes as expected. Additionally, the FCC 25.218 mask [1] was superimposed on the right plot which represents the co-polarized EIRP spectral density (ESD) measured in 4kHz bandwidth. The power density was chosen such that the 20cm x 20cm antenna gain combined with the power was just below the FCC mask. See following Figure 7.

Figure 7: Radiation Pattern and ESD of a 20cm x 20 cm Array Antenna Model.

The simulation assumes the size of the square patch is half a wavelength, both in length and width.  The spacing between the two patches edge-to-edge is half a wavelength.  With this constraint, 9x9 patch elements can fit in 20cm x 20cm area.  The antenna is assumed pointing at boresight. The operating frequency is 14.25 GHz, and the element gain is 5.5 dBi.  The peak antenna gain is 23.9 dBi.  The peak ESD (EIRP spectral density) is -1.53 dBW/4kHz.  For example, in 50 MHz bandwidth, this equates to BUC power of 36 watts, or EIRP of 39.5 dBW.  The salient message here is that for typical terminal with ~ 5W BUC the EIRP spectral density is well below the FCC mask.

Observation 2: The 20x20 cm Antenna model provides a maximum gain of 23.9 dBi at boresight.

### 40cm x 40 cm Antenna

The 40cm x 40cm antenna optimised to a 18x18 element array with elements separated by half wavelength. See following Figure 8.for

Figure 8: 40cm x 40cm Array Antenna Model.

The radiation plot (or the antenna gain plot) of the 40cm x 40cm on the left indicate, as with the smaller antenna, one dominant beam with diminishing sidelobes as expected. Additionally, the FCC 25.218 mask [1] was superimposed on the right plot which represents the co-polarized EIRP spectral density (ESD) measured in 4kHz bandwidth. The power density was chosen such that the 40cm x 40cm antenna gain combined with the power was just below the FCC mask. See following Figure 9.

Figure 9: Radiation Pattern and ESD of a 40cm x 40 cm Array Antenna Model.

The simulation assumes the size of the square patch is half a wavelength, both in length and width.  The spacing between the two patches edge-to-edge is half a wavelength.  With this constraint, 18x18 patch elements can fit in 40cm x 40cm area.  The antenna is assumed pointing at boresight. The operating frequency is 14.25 GHz, and the element gain is 5.5 dBi.  The peak antenna gain is 29.9 dBi.  The peak ESD (EIRP spectral density) is 5.46 dBW/4kHz.  For example, in 50 MHz bandwidth, this equates to BUC power of 45 watts, or EIRP of 46.4 dBW.  The salient message here is that for typical terminal with ~ 5W BUC the EIRP spectral density is well below the FCC mask.

Observation 3: The 40x40 cm Antenna model provides a maximum gain of 29.9 dBi at boresight.

### 60cm x 60 cm Antenna

The 60cm x 60cm antenna optimised to a 28x28 element array with elements separated by half wavelength. See following Figure 10.

Figure 10: 60cm x 60cm Array Antenna Model.

The radiation plot (or the gain plot) of the 60cm x 60cm indicate on the left, as with the smaller antenna, one dominant beam with diminishing sidelobes as expected. Additionally, the FCC 25.218 mask [1] was superimposed on the right plot which represents the co-polarized EIRP spectral density (ESD) measured in 4kHz bandwidth.  The power density was chosen such that the 60cm x 60cm antenna gain combined with the power was just below the FCC mask. See following Figure 11.

Figure 11: Radiation Pattern and ESD of a 60cm x 60 cm Array Antenna Model

The simulation assumes the size of the square patch is half a wavelength, both in length and width.  The spacing between the two patches edge-to-edge is half a wavelength.  With this constraint, 28x28 patch elements can fit in 60cm x 60cm area.  The antenna is assumed pointing at boresight. The operating frequency is 14.25 GHz, and the element gain is 5.5 dBi.  The peak antenna gain is 33.7 dBi.  The peak ESD (EIRP spectral density) is 8.57 dBW/4kHz.  For example, in 50 MHz bandwidth, this equates to BUC power of 38 watts, or EIRP of 49.5 dBW. The salient message here is that for typical terminal with ~ 5W BUC the ESD is well below the FCC mask.

Observation 4: The 60x60 cm Antenna model provides a maximum gain of 33.7 dBi at boresight.

### Receiver Characteristics

The following Table 1 illustrates the essential satellite receiver characteristics of the 20cm x 20cm, 40cm x 40cm, and 60cm x 60cm antennas, assuming a 3 dB Noise figure. The 3 dB Noise Figure is a common industry benchmark value.

Table 1 Essential receiver characterisitcs of 20cm x 20cm, 40cm x 40cm, and 60cm x 60cm antennas at boresight operating at 11.8 GHz.

|  | 20cm x 20cm | 40cm x 40cm | 60cm x 60cm |
| --- | --- | --- | --- |
| Antenna Gain (dBi) | 22.3 | 28.3 | 32.1 |
| Antenna Temp (K) | 25 | 25 | 25 |
| Noise Figure (dB) | 3 | 3 | 3 |
| Noise Figure Temp (K) | 288.6 | 288.6 | 288.6 |
| System Temp (k) | 313.6 | 313.6 | 313.6 |
| G/T (dB/K) | -2.7 | 3.3 | 7.1 |

The antenna gain values shown in the Table are at boresight, but it is well known that when the beam is scanned away from boresight the antenna gain decreases and the beam widens. The antenna temperature 25K is assumed.  The noise figure is 3 dB.  The G/T is about -2.7 dB/K, 3.3 dB/K and 7.1 dB/K for 20cm x 20cm, 40cm x 40cm and 60cm x 60cm antennas, respectively.

For satellite systems, the G/T is a figure of merit for receive antennas.  The objective for satellites systems is to maximize G/T by increasing the antenna gain or minimize the system noise temperature.  A smaller value for G/T indicates lower receiver performance, while a larger value indicates better performance.  For terrestrial systems, on the other hand, the receiver sensitivity is a figure of merit which is a measure of how well a receiver can detect a signal. The receiver sensitivity is the minimum signal power required to achieve a certain bit error rate (BER) or block error rate (BLER). Receiver sensitivity is usually measured in dBm and can be found on interface data specification sheets. A smaller value for receiver sensitivity indicates better receiver performance, while a larger value indicates lower performance.

Proposal 2: For 20cmx20cm, 40cmx40cm and for 60cmx60m VSAT antenna models, we propose to use the values illustrated in Table 1 for the Ku Band normative work.

The mobile VSAT antennas described in this document are beam steered antennas that can track LEO, MEO and GEO satellites.  They can electronically steer radio frequency (RF) beams by adjusting the phase difference between the elements without physical movement of the antennas.  The antennas can also be designed generate and receive multiple beams to track multiple satellites simultaneously.

Proposal 3: Select these three antenna models for Ku Band Normative work.

# Conclusion and Proposal

From the detailed analysis provided in this contribution we can observe that:

Observation 1: Mobile VSAT terminals for NGSO connectivity are not yet specified by 3GPP, neither in Ka-band nor in Ku-band.

Observation 2: The 20x20 cm antenna model provides a maximum gain of 23.9 dBi at boresight.

Observation 3: The 40x40 cm antenna model provides a maximum gain of 29.9 dBi at boresight.

Observation 4: The 60x60 cm antenna model provides a maximum gain of 33.7 dBi at boresight.

In this contribution three array antenna models have been provided for Mobile VSAT applications.

The 40cm x 40 cm and 60cm x 60cm models were developed from empirical data gathered from various Ku Band terminals currently in use in the field and the 20cm x 20cm phase array antenna was developed as a new conceptual model.

The optimised array antenna patterns and the simulated plots were used to derive the peak EIRP spectral density (ESD) for 20cm x 20cm, 40cm x 40cm and 60cm x 60cm array antennas to comply with the FCC mask when the terminal is pointing at GEO.  It was shown that for typical terminal with ~ 5W BUC the EIRP spectral density is well below the FCC mask.

Proposal 1: With the increasing interest in the Land, Maritime, Aeronautical and Vehicular sectors of the industry, Mobile VSAT terminals for NGSO should be taken in the Rel-19 work plan for Ku-band.

Proposal 2: For 20cmx20cm, 40cmx40cm and for 60cmx60m VSAT Antenna Models, we propose to use the values illustrated in Table 1 for the Ku Band Normative work.

Proposal 3: Select these three antenna models for Ku Band Normative work.

# Technical Annex

## Regulatory aspects of the ESIMs

The following lists the CEPT and ETSI references on the ESIMs in the Ku band

| ECC Decision | Satellite | Frequency band | Type of ESIM | ETSI Standard |
| --- | --- | --- | --- | --- |
| ECC/DEC/(05)10 | GSO | 14-14.5 GHz (E2s) 10.7-11.7 GHz (s2E) 12.5-12.75 GHz (s2E) | Maritime | EN 302 340 |
| ECC/DEC/(05)11 | GSO | 14-14.5 GHz (E2s) 10.7-11.7 GHz (s2E) 12.5-12.75 GHz (s2E) | Aeronautical | EN 302 186 |
| ECC/DEC/(18)04 | GSO | 14-14.5 GHz (E2s) 10.7-12.75 GHz (s2E) | Land | EN 302 448 (Trains) EN 302 977 (Véhicule-Mounet) |
| ECC/DEC/(18)05 | non-GSO | 14-14.5 GHz (E2s) 10.7-12.75 GHz (s2E) | Maritime, aeronautical, land | EN 303 980 EN 303 981(Wide Band) |
| ECC/DEC/(19)04 | GSO & non-GSO | 12.75-13.25 GHz (E2s) 10.7-12.75 GHz (s2E) | Aeronautical | EN 302 186 (GSO only) |

Legend: E2s: Earth to Space; s2E: Space to Earth

## ITU Definition of VSAT and ESIM

Ubiquitous satellite user terminals consist of both VSAT and ESIM, operating in both GSO and NGSO satellite systems.

| VSAT | - Generally used for connectivity at fixed locations - Preferred means to easily establish reliable communications in remote areas (also for backhaul purposes) and during emergencies (e.g. humanitarian aid and relief efforts in disaster zones) |
| --- | --- |
| ESIM | - Installed on aeronautical (A-ESIM), maritime (M-ESIM) or land (L-ESIM) mobile platforms to provide seamless connectivity worldwide for passengers, crew members and operators. - Other acronyms also exist at ITU, e.g. - Aeronautical Earth Stations (AES) equivalent to A-ESIM according to Article 1.82 of the RR: “An earth station in the fixed-satellite service, or, insome cases, in the aeronautical mobile-satellite service, located at a specified fixed point on land to provide a feeder link for the aeronautical mobile-satellite service” - Earth station on board vessels (ESVs) equivalent to M-ESIM, see for example ITU-R Resolution 902 |

## NTN VSAT Types

Table 9.2.1.0-1 which is from TS 38.101-5:

Table 9.2.1.0-1: The definitions of NTN VSAT Types (TS 38.1010-5)

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

# Reference

[1] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.218

[2] https://5gaa.org/maximising-the-benefit-of-future-satellite-communications-for-automotive/

The FCC 25.218 mask [1] is applicable only when the mobile VSAT is pointing at a GEO.  When the mobile VSAT is pointing at NGSO, the mask is much more complicated and is based on Equivalent Power Flux Density (EPFD) metric to meet interference thresholds as defined in the Radio Regulations.

BUC is a Block Up Converter which converts lower frequency to a higher frequency to transmit to the satellite.