# 3GPP TSG-RAN WG4 Meeting #113	R4-2418033

# Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# Background

In RAN4#112bis, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed which mainly focuses on the approach that converts full RB allocation in UE CBW to “inner RB allocation” with a new emission bandwidth.

In this contribution, we provide our further views on the MPR reduction via relaxed ACLR/SEM/spurious emission modification.

# The mechanism for MPR reduction via a new emission BW

In RAN4#112, the approach of defining a “new emission BW” to scale the emission limit was discussed intensively. In principle, the applicable frequencies of emission limits (both in-band and out-of-band) in 3GPP that depend on the UE-specific CBW shall be scaled according to this “new emission BW,” including IBE, SEM, ACLR, and SE as shown in Fig.1

Figure. 1. Illustration of Rel-19 single CC MPR reduction approach based on a new emission bandwidth

Proposal 1: To reduce the MPR, a new emission BW can be indicated to the UE and UE can scale the applicable frequencies of general emission limits, including IBE, SEM, ACLR, and SE, according to this new emission BW.

One potential issue here is that the measurement BW of ACLR will be increased once it is scaled with the new emission BW. Generally speaking, the emission level that is further away from the UE CBW is likely to contribute very little to the mean power level on the adjacent channel, so such an issue may not cause any problem for UE MPR reduction. In any case, RAN4 may need to confirm if an enlarged ACLR MBW would not limit the MPR that UE applied.

Observation 1: Although the ACLR BW is enlarged since the ACLR boundary is further away from the UE CBW, the overall ACLR requirement is likely still relaxed compared to the original ACLR level.

Proposal 2: RAN4 confirms if the enlarged ACLR BW would not affect the MPR reduction.

# Applicable scenario

In RAN4#112bis, the following agreement has been made:

#### Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios)

- WF

  - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation.

    - It does not preclude separate solutions for each applicable scenario

No relaxation of ACLR/SEM/SE values

Scenario 1 implies that the relaxed emission limit can be applied in the region that is outside the operator’s holding spectrum. Based on the feedback from operators, such a scenario may appear in multiple regions. However, such a scenario is more band-specific than general. Therefore, it is important to determine whether a general solution should be specified or if another band-specific approach should be adopted. Therefore, it is suggested that we first focus on scenario 2 to create a baseline approach and then check if any general solution can be defined for scenarios 1-1.

Observation 2: it is more feasible to consider a specific solution to reduce the MPR on the UE for scenarios 1-1 and 1-2.

Proposal 3: It is proposed to create a complete solution based on scenario 2 first and then check if the same approach can be applied to scenarios 1-1 and 1-2.

# The new emission BW in scenario 2

With only focusing on scenario 2 in mind, if the new emission BW edge exceeds the BS CBW edge, it may cause potential issues where out-of-band protection can not be guaranteed. Therefore, it is proposed that the new emission BW should not go beyond the BS CBW.

Observation 3: Allowing extended UE CBW to go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection in scenario 2.

Proposal 4: To ensure the protection out of the frequency band, the new emission CBW should not exceed the BS CBW in scenario 2.

Meanwhile, emission limits that currently do not depend on UE CBW, for example, the spurious emission for co-existence or emission limits due to regulatory requirements, shall not be changed to ensure the 3GPP requirements can provide sufficient protection between different frequency bands and be compatible with regional regulatory requirements.

Proposal 5: No changes to spurious emission for co-existence or emission limits due to regulatory requirements with the new emission BW scenario 2.

Then, the next question is how small the new emission limit bandwidth needs to be so that the UE can achieve sufficient MPR reduction. By looking at the MPR currently specified for PC3 and PC2, it can be seen that about 1 to 1.5 dB gain can be obtained by transfer to the edge/outer RB to inner RB while there is no gain or very limited gain (~ 0.5 dB) by transfer the edge RB to outer RB allocations. Therefore, it is reasonable to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated. In other words, the “new emission BW” shall be at least 1.5* (legacy UE CBW).

Proposal 6: Focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

Another aspect regarding the applicable BW of the “new emission BW” is that the “new emission BW” can be transparent to the UE as long as “new emission BW” is one of the CBW that the UE can support. The network can always configure the UE with a UE channel BW but with limited RB allocations, and it will be equivalent to the case illustrated above as a “new emission BW.” On the other hand, new UE behavior, e.g., MPR reduction, needs to be specified if the new emission BW is wider than the maximum UE CBW it can support.

Observation 4:  No specification impact of the “new emission BW” if it is one of the CBWs that the UE can support.

Proposal 7:  RAN4 focuses on the case that “new emission BW” is wider than the maximum UE CBW that it can support.

One issue raised in [3] is that if we take the inner RB allocation based on the new emission BW, then some RB allocation that originally belonged to the inner region will become outer (as illustrated in Fig. 2). RAN4 may need to further study if such an issue would happen and ensure that no higher MPR would appear once a large BW is specified.

Figure. 2. Illustration of some RB is transferred from inner to outer due to the new emission BW

Observation 5: The UE RB allocation may be transferred from inner to outer in some RB allocations once a new emission BW is applied.

Proposal 8: RAN4 to study and confirm if it could happen for some inner RB to become outer RB with a wider emission BW and ensure that no increased MPR would be applied.

# Signalling of the new emission BW

Having defined the operation mechanism and applicable bandwidth of the new emission BW above,  how to signal this “new emission BW” to UE is still an open issue. In RAN4#112, two approaches have been proposed: one is to define a new “extended UE CBW” while the other is to adopt the BS CBW, which has already been included in SIB1 as carrierBandwidth. Both approaches have pros and cons, and we summarize our analysis in Table I.

Table I. The pros and cons of a different definition of the “new emission BW.”

| The “new emission BW” definition | Pros | Cons |
| --- | --- | --- |
| extended UE CBW | It can be used more flexibly, which can support the scenario that the operator spectrum is wider than a single BS CBW | Additional signaling overhead and specification work, such as the concept of extended UE CBW, does not exist as of today. |
| BS CBW | Lower signaling overhead and minimal spec impact as BS CBW has already been indicated in SIB1 as carrierBandwidth (multi-CC case can be further supported via DL CA configuration). Meanwhile, the UE also aware the offset of the RB allocation to the reference point. | Less flexibility comparing to extend UE CBW method. |

Observation 6: From a signalling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

On the other hand, one limiting factor of using BS CBW in SIB1 (carrierBandwidth) is that the scenario with multiple CC would not be able to be supported. One potential solution here is for the network to configure the UE with DL CA. Per our understanding, with such a configuration, the UE can learn the overall BW across multiple CCs and also its relive RB allocation within them.

Observation 7: To support multi-CC scenarios without introducing any new signalling for the new emission BW, the network can configure the UE with DL CA, so that UE can know the overall BW over multiple CCs and its relative RB allocation inside.

# Performance gain

In addition to the methodology study, the actual performance gain of supporting this reduced MPR also needs to be evaluated. Focus on the scenario that a narrower UE channel BW within wider BS bandwidth, allocating a device with narrow bandwidth, e.g., Redcap and eRedcap, towards the inner location within an operator’s spectrum block, may not be a common scenario in real life to avoid spectrum fragmentation from the network aspect. In addition, since such an operation scenario requires the operator spectrum block to be wide enough, it may not be feasible for most FDD bands, especially those at sub-1GHz.

Observation 8: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 9: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

Proposal 9: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Observation 10: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 10: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Observation 1: Although the ACLR BW is enlarged since the ACLR boundary is further away from the UE CBW, the overall ACLR requirement is likely still relaxed compared to the original ACLR level.

Observation 2: it is more feasible to consider a specific solution to reduce the MPR on the UE for scenarios 1-1 and 1-2.

Observation 3: Allowing extended UE CBW to go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection in scenario 2.

Observation 4:  No specification impact of the “new emission BW” if it is one of the CBWs that the UE can support.

Observation 5: The UE RB allocation may be transferred from inner to outer in some RB allocations once a new emission BW is applied.

Observation 6: From a signaling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

Observation 7: To support multi-CC scenarios without introducing any new signalling for the new emission BW, the network can configure the UE with DL CA, so that UE can know the overall BW over multiple CCs and its relative RB allocation inside.

Observation 8: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 9: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

Observation 10: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 1: To reduce the MPR, a new emission BW can be indicated to the UE and UE can scale the applicable frequencies of general emission limits, including IBE, SEM, ACLR, and SE, according to this new emission BW.

Proposal 2: RAN4 confirms if the enlarged ACLR BW would not affect the MPR reduction.

Proposal 3: It is proposed to create a complete solution based on scenario 2 first and then check if the same approach can be applied to scenarios 1-1 and 1-2.

Proposal 4: To ensure the protection out of the frequency band, the new emission CBW should not exceed the BS CBW in scenario 2.

Proposal 5: No changes to spurious emission for co-existence or emission limits due to regulatory requirements with the new emission BW scenario 2.

Proposal 6: Focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

Proposal 7:  RAN4 focuses on the case that “new emission BW” is wider than the maximum UE CBW that it can support.

Proposal 8: RAN4 to study and confirm if it could happen for some inner RB to become outer RB with a wider emission BW and ensure that no increased MPR would be applied.

Proposal 9: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

Proposal 10: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# References

- R4-2415218	Topic summary for [112bis][114] NR_ENDC_RF_Ph4_part1	Moderator(Huawei)

- R4-2417111	WF on power enhancement for UE	Huawei

- R4-2416177	On power domain enhancements for single carrier	Huawei, HiSilicon, China Unicom, Orange