3GPP TSG-RAN WG4 Meeting #113		               R4-2418034

Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	Coexistence evaluation on D1T1 deployment scenario

Agenda item:	7.21.2.2

Document for:	Approval

# Background

In RAN4#112bis, the coexistence simulation results were further discussed [1] and agreed as captured in [2]. In this contribution, we further analyse selected simulation parameters and results for the D1T1 deployment scenario.

# Discussion on the simulation results

## Deployment Scenario 1-1

The coexistence performance of deployment scenario 1-1 has been analyzed in RAN4#112bis, and the observation below has been made. It can be seen that there is no conclusion for case b, where NR DL is the aggressor and the reader is the victim.

| a |  | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| --- | --- | --- | --- |
| b |  | NR DL->reader | TBA |
| c |  | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| d |  | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage |

In our simulation, we observed less than 0.05 dB SINR degradation on both 50% CDF and 5% CDF in case b. Therefore, we think RAN4 can conclude that inter-system interference is negligible in this case.

Observation 1: the SINR degradation on both 50% and 5% in case b of deployment scenario 1-1 is negligible.

## Deployment scenario 1-2

For deployment scenarios 1-2, the following conclusion has been drawn during RAN4#112bis based on the 10% indoor UE percentage:

| a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| --- | --- | --- |
| b | NR DL->reader | TBA |
| c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |
| d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |

With 10 % UE indoor percentage, for NR DL as the aggressor and the reader as the victim, negligible interference can be observed in our simulation. In our understanding, this is the same situation as in deployment scenario 1-1, and the penetration loss ensures the isolation between the AIoT and NR systems.

Observation 2: The SINR loss for NR DL as aggressor and reader as victim is less than 1 dB.

On the other hand, there was no explicit conclusion drawn on a higher indoor UE percentage in RAN4#112bis. When the NR UE is 100% indoor, the SINR degradation on the AIoT system due to NR UL, and the SINR degradation on NR DL due to the AIoT system are significant, as shown in Fig. 1.

Figure. 1. The performance with 100% indoor NR UE of (a) NR DL with AIoT reader as the aggressor, (b) NR DL with AIoT device as the aggressor, (c) AIoT D2R with NR UL as the aggressor, and (d) AIoT R2D with NR UL as the aggressor.

Observation 3: The SINR degradation on the AIoT system due to NR UL and the SINR degradation on NR DL due to the AIoT system are significant when the NR UE percentage is 100% indoor.

In addition, to study the effect of indoor UE percentage, we looked into a particular case of the AIoT reader as the aggressor and NR DL as the victim, e.g., scenario 1-2 c. The simulation results are shown in Fig. 2, and it can be observed that SINR degraded linearly with an increased indoor UE percentage. Therefore, a reasonable indoor UE percentage shall be assumed for the simulations to ensure coexistence. In addition, further studies can be conducted on the mechanism that can support a higher percentage of indoor UE.

Figure. 2 The impact of indoor UE percentage on the SINR degradation of NR DL

Proposal 1: RAN4 can conclude that some mechanisms to support a higher percentage of indoor UE need to be studied.

## Active rate of reader

Currently, RAN4 assumes that in total, 18 readers would be uniformly deployed in a factory with 120m *60 m area, but only 2 readers with a minimum separation of 60m as baseline are activated simultaneously at each simulation drop. The reason for limiting the active rate of readers is to ensure an acceptable intra-system interference (baseline SINR) for the AIoT system. To show the effect, we randomly choose x of 18 readers, where x = 1 to 18 in the simulation results plotted in Fig. 3. It can be observed that when multiple readers (more than 1) are activated, the baseline SINR will be significantly reduced.

Figure. 3 the impact of AIoT reader activation rate on the intra-system SINR of AIoT R2D link.

On the other hand, deploying a denser number of readers and activating multiple of them simultaneously can improve the response speed in use cases like inventory management. Therefore, it may still be a preferred deployment case in real life. From this perspective, RAN4 may suggest further considering the mechanism to coordinate multiple readers as the outcome of the coexistence study. Since there is no frequency selectivity of the RF-ED receiver, FDD between different readers may not be helpful, thus multiplexing over the spatial domain might be a possible solution when multiple readers would need to operate simultaneously.

Observation 4: Since there is no frequency selectivity of the RF-ED receiver, FDD between different readers may not help increase the simultaneous activation rate of readers.

Proposal 2: RAN4 can conclude that mechanisms to coordinate multiple readers to be active simultaneously need to be studied and specified.

## SINR in R2D receiver for RF-ED.

To correctly model the SINR, we need to look into the operation mechanism of the RF envelope detector (ED) receiver. A simplistic RF-ED usually comprises a rectifier (a diode) and an LPF. The diode can be modeled as a component that squares the signal, which can be seen as a self-mixer that down-converts signals at any frequency point to DC, while some other inter-modulation components will also appear. Then, the LPF can (ideally) remove all the high-frequency components. Such a procedure is illustrated in Fig. 3.

Figure. 3 The illustration of operation of RF-ED receiver.

Based on this principle, it can be seen that an interference signal on a neighboring channel and the noise will be converted to DC after the diode; thus, the LPF will not be able to remove such interference. Therefore, the correct way to compute the SINR after the RF-ED receiver is in (1):

SINRo=SINRi28*SINRi+4*BWRFBWBB                                                                (1)

Where SINRi is Input SINR at the antenna connector with noise/interference, and SINRo is Output SINR after BB LPF assuming downconverted noise/interference. (1) follows the principle of the RF-ED explained above (passing signal, noise, and interference via the self-mixing procedure) and derives the SINR equation accordingly. However, one assumption here is that a perfect square filter is assumed, and there is no consideration of the roll-off factor, which will underestimate the SINR degradation caused by the interference in neighboring channels.

Proposal 3: to obtain the SINR in R2D link correctly, the RF-ED receiver needs to be correctly modelled with consideration of the self-mixing effect.

# Summary:

In this contribution, we provide our analysis of the coexistence performance of AIoT, where the following observations and proposals are given:

Observation 1: the SINR degradation on both 50% and 5% in case b of deployment scenario 1-1 is negligible.

Observation 2: The SINR loss for NR DL as aggressor and reader as victim is less than 1 dB.

Observation 3: The SINR degradation on the AIoT system due to NR UL and the SINR degradation on NR DL due to the AIoT system are significant when the NR UE percentage is 100% indoor.

Observation 4: Since there is no frequency selectivity of the RF-ED receiver, FDD between different readers may not help increase the simultaneous activation rate of readers.

Proposal 1: RAN4 can conclude that mechanisms to support a higher percentage of indoor UE need to be studied and specified.

Proposal 2: RAN4 can conclude that mechanisms to coordinate multiple readers to be active simultaneously need to be studied and specified.

Proposal 3: to obtain the SINR in R2D link correctly, the RF-ED receiver needs to be correctly modelled with consideration of the self-mixing effect.

# References

- R4-2415233   Topic summary for [112bis][129] FS_Ambient_IoT_solutions_part1   Moderator(CMCC)

- R4-2414304   WF on Coexistence study for ambient IoT	CMCC