# 3GPP TSG-RAN WG4 Meeting #113	R4-2418035

# Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	Further considerations on the ambient IoT device implementation and RF aspect

Agenda item:	7.21.3.2

Document for:	Approval

# Background

During RAN4#112bis, the possible RF implications on AIoT device implementation and RF requirement have been further discussed [1] [2]. However, RAN4 has not discussed so much on the baseline assumption on device implementation. In our view, it is essential to create a baseline architecture so that RAN4 can further progress on the work. In this contribution, we provide our view on how the device architecture should be defined.

# Reception part (R2D)

## RF BPF

One particular issue observed during the LP-WUS SI in Rel-18 is that the RF-ED receiver lacks frequency selectivity [3]. Therefore, to suppress the interference from signals on adjacent spectrums, a high-Q RF BPF is usually required, where the bandwidth of BPF needs to be equal to the AIoT BW.

However, if a multi-band operation needs to be supported in the R2D link, either no RF BPF at all or multiple RF BPFs that can cover all the frequency bands with all the possible RB allocations shall be used. Considering the complexity and power consumption limits for AIoT devices, the former choice (no RF BPF) may be more feasible, at least for device type 1.

In addition, even if there is only a single FDD band that the AIoT device needs to support, if the R2D link may happen both in the DL and UL spectrum due to the reader being either BS or intermediate UE, it will also be preferred not to implement any RF BPFs.

Proposal 1: RAN4 conclude that no RF BPF filter is assumed for at least device type 1.

## BB LPF

For the BB LPF, a similar issue has also been studied for LP-WUS [3], where 3rd-order and 5th-order Butterworth filters were taken as baseline assumptions. The same assumption was adopted in the RAN1 LLS assumption for AIoT in [4]. Considering the extremely constrained power consumption and complexity, a possible starting point of BB LPF filters could be the 3rd-order Butterworth filter.

Proposal 2: RAN4 can conclude 3rd order Butterworth filter as the baseline assumption for the BB LPF to discuss the RF performance of AIoT devices further.

## ADC

Another critical aspect that affects R2D performance is the ADC. As we know, many ADC bits provide a higher dynamic range and SNR but also increase power consumption. Therefore, considering the minimal power budget for device type 1, a 1-bit ADC seems the only viable option. Meanwhile, more power consumption may be allocated to ADC for devices 2a and 2b, which allows 4-bit ADC to be considered. To our knowledge, this also aligns with RAN1 LLS assumptions that were agreed in RAN1#117.

Proposal 3: RAN4 can conclude that 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b as baseline assumption for future work in RAN4.

# Transmission part (D2R)

## Output power

In RAN4#112bis, it has been agreed that the maximum output power  might be needed in the context of testing the device for backscattering power or power backscattering loss for device type 1/2a. For backscattering type of devices, e.g., devices 1 and 2a, the power is transmitted to the reader via the reflection of a carrier wave (CW) from external nodes. Therefore, its transmission performance depends on both the design of AIoT devices and the property of the external CW and the emitter node.

Generally speaking, we can say that the maximum output power of device types 1 and 2a will be the sum of the device's reflection gain and the maximum input power level it can tolerate from the CW in dB domain. Please note that this maximum input power level is different from the one in R2D, as this power level would be determined by the backscattering modulator and the reflection amplifier if applicable. For passive RFID tags, the type of maximum input power level that does not saturate the tag ranges from about -20 to -10 dBm. When it comes to the reflection amplifier, based on the literature review, a 10-20 dB amplification gain with a couple of hundred of uW power consumption can be considered as a starting point for the reflection performance. Therefore, defining the maximum output power and power class of device type 1 in the range of -20 to -10 dBm while about -10 dBm to 0 dBm for device 2a might be feasible. RAN4 shall further study what would be a reasonable assumption here.

Observation 1: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a with assuming a maximum input power level about -20 to -10 dBm.

## Frequency shift

When it comes to the frequency accuracy requirement for device type 2a, it is critical if a frequency shift function would be needed for the backscattering type of devices. Generally speaking, there are three levels of frequency shift: 1) no explicit frequency shift where the backscattering signal is on the same frequency as the CW signal (note that any frequency shift caused by the coding scheme may still be included). 2) a small but explicit frequency shift where the backscattering signal is slightly shifted away from the CW signal (e.g., one AIoT channel away) so that the interference from the CW on the backscattering signal can be reduced. 3) a large explicit frequency shift where the backscattering signal can be shifted to another band than the CW signal, e.g., the CW can be transmitted in the DL bands whereas the backscattering signal is transmitted in the UL frequency bands. The three different frequency shift schemes are illustrated in Fig. 1, which are represented by backscattering signals 1, 2, and 3, respectively, in the plot.

Generally speaking, an oscillator consumes more power at a higher frequency. For example, assuming a ring oscillator has a few uW when it oscillates at a few MHz but can go up to hundreds of uW or even mW if it needs to oscillate at hundred(s) of MHz (depending on the oscillator design, for example, the number of stages in a ring oscillator). Therefore, with the given power budget (≤ a few hundred µW peak power consumption), the feasibility of a larger frequency shift needs to be further studied. Based on the state-of-the-art works, at least generating a frequency shift for backscattering communication in the range of tens of MHz has been realized.

Figure. 1. The possible frequency shift scheme for backscattering AIoT devices.

Another aspect related to the feasibility of frequency shift is the accuracy. Based on the latest discussion in RAN1#118 [4] and RAN1#118bis, it has been observed that large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz. Therefore, maintaining good frequency accuracy when the frequency shift is larger will be more challenging, and the feasibility of using frequency shift and the corresponding requirements may also differ depending on the type of frequency shift that needs to be applied.

Proposal 4: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

Regarding the necessity of frequency shift, it has also been observed in RAN1 that a large frequency shift may allow the reader to avoid implementing in-band full duplex capability on the reader in the case that CW is within the topology. In our understanding, a large frequency shift can enable a conventional FDD implementation of the reader by shifting the CW signal from DL spectrum to the UL spectrum. Meanwhile, a small frequency shift may enable the sub-band full duplex (SBFD) implementation on the reader side. Eventually, a full duplex mode may need to be supported by the reader if no frequency shift can be supported at all on the device side, assuming the CW node is integrated with the reader. Therefore, it is also necessary to consider from the reader implementation aspect on whether the frequency shift function would be necessary for AIoT devices to be implemented.

Proposal 5: The reader implementation aspect (e.g., which duplex mode reader can support) shall also be discussed regarding whether the frequency shift function would be necessary for implementing AIoT devices.

# Testability

In RAN4#112, it has been agreed that the OTA test is taken as the baseline for backscattering type of AIoT devices (devices 1 and 2a) [2]. In addition, it has been agreed that the side conditions in the OTA test setup including CW signal and RF energy source, etc. are FFS.

## Impact of RF energy source and CW

To be fully functional, AIoT devices require a dedicated energy source for harvesting to be provided during the test. Different types of resources can be used for different devices, and it is still a question of how the energy source can be provided within the OTA test chamber. Taking an RF energy source as example, an external antenna may be needed to provide the CW and also provide RF energy, while a separate testing antenna is also needed, which are illustrated in Fig. 2. There are a few issues that need to be studied if such a test system is feasible: 1) the cross talk between the CW/energy source antenna and the testing (receiving) antenna. 2) the power density of the CW and the power of the energy source that exist within the imagined quiet zone (QZ). The former issue would affect the background noise/interference signal level and thus it must be suppressed to be low enough so that the test system has sufficient dynamic range. The latter one is critical as the power density must be above a certain level to trigger the AIoT device under test.

Figure. 2. OTA test chamber setup

Proposal 6: RAN4 shall investigate 1) the cross talk between the CW/energy source antenna and the testing (receiving) antenna. 2) the power density of the CW and the power of the energy source within the imagined quite zone (QZ) in OTA test of backscattering devices.

In addition, in [5], it is assumed that the duration of one device's unavailability due to charging by energy harvesting can be assumed to be up to several tens of seconds, which was endorsed in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability. Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of certain testing.

Observation 2: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds based on the endorsed observation in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability.

Observation 3: Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of some RF tests.

It is worth highlighting that this is an entirely different situation than for normal UEs in a test. Regular IoT devices or normal smartphones can operate over hours in conformance tests when intensive transmission and reception are performed. In addition, it is known that the conformance test can be very time-consuming, especially for OTA tests, as each test may last tens of minutes or even over an hour since the probe needs to scan a full space over the device. If an AIoT device were tested similarly, the test time may explode, considering that the device would be unavailable from time to time.

Observation 4: the transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 7: RAN4 needs to study the device behaviour (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been made:

Observation 1: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a with assuming a maximum input power level about -20 to -10 dBm.

Observation 2: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds based on the endorsed observation in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability.

Observation 3: Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of some RF tests.

Observation 4: the transmission and reception behaviour of AIoT devices are different from legacy UEs in 3GPP.

Proposal 1: RAN4 conclude that no RF BPF filter is assumed for at least device type 1.

Proposal 2: RAN4 can conclude 3rd order Butterworth filter as the baseline assumption for the BB LPF to discuss the RF performance of AIoT devices further.

Proposal 3: RAN4 can conclude that 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b as baseline assumption for future work in RAN4.

Proposal 4: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

Proposal 5: The reader implementation aspect (e.g., which duplex mode reader can support) shall also be discussed regarding whether the frequency shift function would be necessary for implementing AIoT devices.

Proposal 6: RAN4 shall investigate 1) the cross talk between the CW/energy source antenna and the testing (receiving) antenna. 2) the power density of the CW and the power of the energy source within the imagined quite zone (QZ) in OTA test of backscattering devices.

Proposal 7: RAN4 needs to study the device behaviour (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

# References

- R4-2415234   Topic summary for [112bis][130] FS_Ambient_IoT_solutions_part2  Moderator(Huawei)

- R4-2417088   WF on RF requirement impact of A-IoT	Huawei

- TR 38.869	     Study on low-power Wake-up Signal and Receiver for NR

- R1-2407274    RAN1#118 FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture     Moderator (Qualcomm)

- RP-240116 "View on Ambient IoT scope". Qualcomm.