3GPP TSG-RAN WG4 Meeting #113	                                   R4-2418037

Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	UE RF requirement for NTN Redcap and eRedcap

Agenda item:	7.26.2.1

Document for:	Approval

# Background

In RAN4#112bis, the RF requirement for NTN Redcap in Rel-19 has been further discussed [1] but without much further progress. In this contribution, we provide our view on the remaining missing RF requirement for NTN Redcap and eRedcap in Rel-19.

# REFSENS for HD-FDD

In RAN4#112, it has been agreed that a 0.5 dB tightening factor is adopted for n256 for HD-FDD operation, which results in the REFSENS for 5MHz becoming -100 dBm for HD-FDD, and it aligns with the principle of how we derived the TN Redcap REFSENS.

In addition, In RAN4#111, it has also been agreed:

- [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

Observation 1: Tentative agreement has been made in RAN4#111 on the REFSENS of n254 with 0.5 dB tightening.

The 0.5 dB tightening on n254 and n256 followed the same principle as in the TN case, which was originally proposed in [2], and the principle is summarized in the table below:

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

In our view, the same approach should be taken as in the TN case so that the RF front end of TN Redcap and NTN Redcap can be largely reused to facilitate the cost and time-to-market of Redcap/eRedcap NTN system.

Observation 2: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

From the coverage perspective, it is our understanding that the uplink coverage is the bottleneck for both legacy TN Redcap and legacy NTN networks. Therefore, further stringent REFSENS requirements may not significantly benefit network coverage.

Observation 3: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Therefore, it is proposed that the same approach can also be used to derive the REFSENS for NTN Redcap HD-FDD, where RAN4 shall confirm 0.5 dB tightening for n254 and adopt 0 dB tightening for n255.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

# GNSS operation

In RAN4#111, it is identified that the GNSS operating in the frequency range 1559~1610MHz and may have difficulty operating simultaneously with frequency band n255 due to the NTN UL/DL interference and n254 due to the UL interference [4]. In addition, a few bands (e.g., [n251]) are currently under discussion in 3GPP RAN4, which may also impact the GNSS receiver due to the very close spectrum allocation. An illustration of NTN spectrum 3GPP vs. GPS L1 receiver frequency is presented in Fig. 1.

Fig. 1. An illustration of NTN spectrum 3GPP and GPS L1 receiver frequency.

Observation 4: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

On the other hand, it is explicitly stated in the WI that the simultaneous GNSS and NR-NTN operation are supported in RedCap/eRedCap UE [3]. Such a restriction was not required explicitly for normal NR NTN UEs, and it was explicitly not needed at all for IoT NTN UEs. Considering the complexity of Redcap UE is somewhere between normal NR UE and IoT UEs, it is a bit unclear what the implication of having this requirement for Redcap NTN devices is.

Considering the Redcap NTN is a lower device than normal NR NTN, it is also a question of whether the Redcap NTN can fully support the Doppler and TA pre-compensation in the same way as normal NR UEs. Therefore, RAN4 shall investigate how to handle the scenario in which the GNSS can’t operate simultaneously with NR communication.

Proposal 2: RAN4 shall further study the impact of GNSS not operating simultaneously with NR NTN.

Per our knowledge, no measurement gap has ever been defined for NR NTN operation, only for IoT NTN. Therefore, there is currently no approach from 3GPP that can be used for NR NTN UE to mitigate such interference.

Observation 5: No measurement gap has been defined for NR NTN operation.

To resolve this issue without affecting the core requirements defined in RAN4, a possible tool to be used is the in-device coexistence (IDC) mechanism. IDC is a mechanism to solve any coexistence issues within the UE that can not be solved by UE implementation, and UE requests some action from the network. Actually, GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in Rel-10 or so. Then, in subsequent releases, many other use cases were identified and defined, including those for NR.

Observation 6: GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in Rel-10 or so.

With IDC solution, NR NTN UE can send an indication to the network along with some more information like interferer type, etc, and then the network either applies FDM (handover to another frequency or allocates resources away from interfering source) or TDM (time multiplex resources between two sources) between the GNSS and NR NTN. Therefore, adding the GNSS receiver as an IDC event for the NTN system in 3GPP can ensure the protection of the GNSS receiver with NR NTN operation for Redcap NTN UE.

Proposal 3: It is proposed that GNSS reception be added as an event for IDC to protect the GNSS operation with the NR NTN option for Redcap NTN UE.

With the IDC solution, a universal method can be created for all the existing and future NTN bands that may potentially jeopardize the GNSS operation. Meanwhile, the proposed approach can minimize the impact on the RAN4 RF requirement, where no core requirement changes are needed, but it only needs to ensure that no IDC event is triggered during the conformance test. For this matter, RAN4 can consider send a LS to RAN5 once the IDC solution is agreed, and a accompany LS has been attached in the paper.

Observation 7: No change in the RF requirement is expected by using IDC-based approach but rather ensure that the IDC event shall not be triggered during the conformance test by designing the test configuration properly.

Proposal 4: RAN4 send LS to RAN5 to ensure that no IDC event would be triggered in the conformance test if the IDC-based approach is agreed upon to solve the coexistence between GNSS and NR NTN.

# Speed limitation on frequency error requirement

Another issue related to GNSS is on the speed limitation on the applicability of frequency [4]. The two issues were sort of discuss under the same umbrella in the past a few RAN4 meeting. However, in our understanding, the concern on speed limitation for the applicability of RAN4 requirement for frequency error comes from the UE power consumption where UE needs to perform very frequent pre-compensation when it moves in a high speed. While we acknowledge the concern, but we would like to clarify that this is a separate issue than the limitation to the simultaneously operation between GNSS and NTN. Therefore, it is prosed that RAN4 shall discuss those two issues separately.

Observation 8: The speed limitation on frequency error requirement affect the power consumption of GNSS receiver.

Proposal 5 RAN4 further discuss the speed limitation on frequency error requirement with consideration of UE power consumption but under separate issue with the blocking issue of GNSS receiver.

# Conclusion

In this paper, we presented our view on the RF requirement for NTN Redcap and eRedcap, where the following observations and proposals are made:

Observation 1: Tentative agreement has been made in RAN4#111 on the REFSENS of n254 with 0.5 dB tightening.

Observation 2: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

Observation 3: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Observation 4: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

Observation 5: No measurement gap has been defined for NR NTN operation.

Observation 6: GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in Rel-10 or so.

Observation 7: No change in the RF requirement is expected by using IDC-based approach but rather ensure that the IDC event shall not be triggered during the conformance test by designing the test configuration properly.

Observation 8: The issue of speed limitation n frequency error requirement is mainly due to the power consumption concern rather than the blocking issue of GNSS receiver.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

Proposal 2: RAN4 shall further study the impact of GNSS not operating simultaneously with NR NTN.

Proposal 3: It is proposed that GNSS reception be added as an event for IDC to protect the GNSS operation with the NR NTN option for Redcap NTN UE.

Proposal 4: RAN4 send LS to RAN5 to ensure that no IDC event would be triggered in the conformance test if the IDC-based approach is agreed upon to solve the coexistence between GNSS and NR NTN.

Proposal 5 RAN4 can further discuss the speed limitation on frequency error requirement with consideration of UE power consumption but under separate issue with the blocking issue of GNSS receiver.

# References

- R4-2416561   Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF	Qualcomm

- R4-2200442   RedCap UE HD-FDD REFSENS requirements   Apple

- RP-234078     New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R4-2409336   Discussion on the other potential impacts for Rel-19 NTN RedCap UE    Huawei, HiSilicon

# Appendix LS to RAN5

# 3GPP TSG-RAN WG4 Meeting #113	R4-24xxxxx

# Orlando, US, 18th – 22nd November, 2024

Title:	LS on NR NTN IDC event for GNSS operation in conformance test

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG5

Cc:	-

Contact person:	Kun Zhao

Kun.1.zhao@sony.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has studied the emissions and blocking effects from NR NTN transmissions to GNSS receivers and observed that successful GNSS reception cannot always be guaranteed, the problem being worst in nearby frequency band n254 and n255. Especially, it’s very important for NR NTN UE to acquire the accurate location information during both initial access and connected mode to do the pre-compensation when the UE is moving.

To avoid any field performance issues both for NR NTN RedCap as well as NR NTN non-RedCap UEs, it has been agreed in RAN4 that a reliable in-device co-existence (IDC) solution should be specified and a request to specify such a solution has been sent to RAN2.

Therefore, upon the RAN2 work, RAN4 respectfully asks RAN5 to consider a solution to ensure that NR NTN IDC event on GNSS operation will not be triggered during the conformance test to ensure that GNSS reception of NR NTN Redcap UEs and non-RedCap UEs can guaranteed in the conformance test with vicinity of the emissions and blocking effects from NR NTN transmissions.

# Actions

ACTIONS:

To RAN WG5: RAN4 respectfully asks RAN5 to consider a solution to ensure that NR NTN UEs IDC event on GNSS operation will not be triggered during the conformance test.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #114	Feb 17 - 21, 2025	Athens, GR

TSG-RAN4 Meeting #114bis	April 7-11, 2025	China, CN